| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17 Oct. –24 Oct. 2014 | Document: JCTVC-S0005 |
| --- | --- |

| Title: | JCT-VC AHG report: Verification test preparation (AHG5) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | V. Baroncini, M. Karczewicz, N. Ramzan, C. Rosewarne, K. Sharman, T. K. Tan, J.-M. Thiesse, W. Wan | Email: | vittorio@fub.it martak@qti.qualcomm.com Naeem.Ramzan@uws.ac.uk chris.rosewarne@cisra.canon.com.au Karl.Sharman@eu.sony.com tktan@m-sph.com jm.thiesse@ateme.com wwan@broadcom.com |
| Source: | AHG 5 |  |  |

_____________________________

# Abstract

This report summarises the Verification test preparation Ad-Hoc Group (AHG5) between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg (October 2014).

# Mandate

The mandates of the Verification test preparation Ad-Hoc Group (AHG5) are as follows:

- Make preparations for verification testing of HEVC for interlaced video content.

- Make preparations for verification testing of HEVC range extensions.

# Activity

The e-mail reflector of the AHG5 is the JCT-VC main one (i.e. jct-vc@lists.rwth-aachen.de).  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG5] in the subject line for easy grouping.

Test material

| Class | Test sequence |
| --- | --- |
| YCbCr 4:4:41 | Traffic_2560x1600_30_10bit_444_crop.yuv |
|  | EBULupoCandlelight_1920x1080_50_10bit_444.yuv2 |
|  | EBURainFruits_1920x1080_50_10bit_444.yuv2 |
|  | VenueVu_1920x1080_30_10bit_444.yuv |
|  | BirdsInCage_1920x1080_60_10bit_444.yuv |
|  | CrowdRun_1920x1080_50_10bit_444.yuv |
| YCbCr 4:2:0i | CattleDogs_1920x1080_420_60i_8bit.yuv |
|  | WaveRocks_1920x1080_420_60i_8bit.yuv |
|  | NewMobCal_1920x1080_420_50i_10bits.yuv3 |
|  | ParkRun_1920x1080_420_50i_10bits.yuv3 |
|  | SthlmPan_1920x1080_420_50i_10bits.yuv3 |

Notes:

1. 4K 10-bit YCbCr in BT.709 test material cropped to HD is desired for the 4:4:4 test, to match display device limitations.

2. These sequences believed to be encoded in BT.2020.

3. Sourced from SVT archive and with interlace-aware 4:2:2 -> 4:2:0 downsampling applied (via ChromaConvert utility).

Two sequences ‘Basketball’ and ‘Tennis’ may also be available.

Software

- HM-16.0 software with the following patches to be used:

  - Interlace PSNR patch1 (see ticket 1282).

  - Delta-QP for POC #1 setting via configuration setting2.

  - “SecondFieldBias” patch to increment GOP ‘depth’ based on POC and GOP size2.

- JM-18.6 software with no further patching required:

Notes:

1. Merged into future HM release.

2. Patches obtained from modified HM-13 software as prepared in the MPEG interlace AHG.

Encoding conditions notes:

- A "minimal" testset could be:

  - 4 (bit-rates) * 1 (RA) * 5 (4:2:0 8-bit sequences) = 20 streams

  - 4(bit-rates) * 1 (RA) * 3 ( 4:2:2 10-bit sequences) = 12 streams

  - 4 (bit-rates) * 1 (RA) * 3 (4:4:4 10-bit sequences) = 12 streams

  - Total: 54 streams

- Resolution to test: Only HD (1080i50 or 60)?

  - Not sure if any current consumer display can handle 4K in 10-bits. Cropping 4K material to HD may be easier.

- Suggested bit-rates:

  - RA 4:2:0i 8-bit in HEVC: 1.5Mbps, 3Mbps, 5Mbps, 8Mbps (in AVC, double these bit-rates)

  - RA 4:2:2 10-bit in HEVC: 5Mbps, 8Mbps, 12Mbps, 16Mbps1

  - RA 4:4:4 10-bit in HEVC: TBD

- For RA, 1 second GOP selected.

- Addition of LD for 4:2:0 sequences could be added depending on resources of Vittorio and others contributors.

- SAFF tool risk of artefacts at switchover points and current test material has no scene changes, so less likelihood of switching between field and frame coding.

- For JM, MBAFF and PicAff enabled.

Notes:

1. It was remarked that for higher bit-rates, e.g. as used in contribution, MOS scores for the multiple QP points may all be high and subjective quality may be indistinguishable. In such cases, a comparison using objective metric may be appropriate, i.e. PSNR for the overall comparison and another metric to check for the absence of larger (visually significant) local variations.

Bitstream exchange

- A shared FTP site for bitstream exchange was established.

- Currently this site contains a package of SAFF-related materials used in earlier interlace study.

- QP selection for the 4:2:0i test sequences is underway.

Post-processing notes:

- For 4:2:0 de-interlacing, use ‘off-line’ de-interlacer so that a progressive YUV sequence can be played back for the subjective test. This avoids any temporal interaction resulting from difficulty in synchronising playback with the built-in hardware de-interlacer in the display.

- The ffmpeg software provides several de-interlaces and two candidates are suggested to be ‘w3fdif’ and ‘mcdeint’.

# Recommendations

- Acquire more test sequences for the subjective test.

- Finalise test conditions and prepare bitstreams.

- Conduct the subjective test.