| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0008 |
| --- | --- |

| Title: | JCT-VC AHG report: Screen content extensions software development (AHG 8) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Bin Li Robert Cohen Xiaoyu Xiu Tzu-Der Chuang Meng Xu | Tel: Email: | krapaka@qti.qualcomm.com libin@microsoft.com cohen@merl.com xiaoyu.xiu@interdigital.com peter.chuang@mediatek.com m.xu@huawei.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This report summarizes the activities of Ad Hoc Group 8 on screen content extensions software (SCM) development that have taken place between the 18th and 19th JCT-VC meetings.

# Mandates

The ad hoc group was mandated to:

- Coordinate development of the HM SCM software and its distribution.

- Prepare and deliver HM 15.0-SCM-2.0 software version and the reference configuration encodings according to JCTVC-R1015.

- Plan and work on migration toward RExt-based HM 16 basis

- Prepare and deliver additional "dot" version software releases and software branches as appropriate.

- Perform analysis and reconfirmation checks of the behavior of the draft design, and report the results of such analysis.

- Suggest configuration files for additional testing of tools.

- Coordinate with AHG15 to address any identified issues regarding text and software relationship

# Software revisions

Multiple versions of the HM SCM software were produced and SCM2.0 was announced on the JCT-VC email reflector. The integration details and performance summary of these revisions are provided in the next subsections. The performance results of software revisions were observed to be consistent with the adopted techniques.

## HM15.0_RExt8.0_SCM2.0

HM15.0_RExt8.0_SCM2.0 has been tagged on the SVN HHI repository on August 1st. This release includes following adoptions:

- JCTVC-R0102 - Fix and improvement for hash based inter search

- JCTVC-R0104 - Removal of Hadamard transform in lossless coding.

- JCTVC-R0105 - Motion estimation starting point

- JCTVC-R0147 - Adaptive in-loop color transform

- JCTVC-R0162 - Intra block copy hash search chroma refinement

- JCTVC-R0186 - Block vector coding improvements for Intra block copy

- JCTVC-R0309 - Block vector prediction improvements for Intra block copy +

- JCTVC-R0081 - Encoder bug fixes

- JCTVC-R0348 - Palette coding tool

- Migration to HM-15.0+RExt-8.0

In addition as per the meeting discussion, the support for flexible intra block copy search range has been provided. This enables CE specific Intra BC search configurations that are different from to CTC.

The release was announced on the email reflector. The software can be downloaded at 
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0/

The performance of this version against HM-14.0+RExt-7.3+SCM-1.1 was described according to the common test conditions in JCTVC-R1015. For the lossy configuration, it is reported that this version provides BD-rate reduction of 33.0%, 23.4% and 18.1% for RGB 1080p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 31.3%, 17.6% and 11.9% for YUV 1080p text and graphics category in AI/RA/LB configuration, respectively. For the lossless configuration, it is reported that this version provides BD-rate reduction of 19.6%, 11.1% and 10.4% for RGB 1080p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 22.6%, 10.8% and 10.0% for YUV 1080p text and graphics category in AI/RA/LB configuration, respectively.

Table 1 and Table 2 summarize BD-rate change for lossy and lossless configuration respectively.

Table 1. BD-Rate change in Lossy configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -33.0% | -29.0% | -28.7% |
| RGB, text & graphics with motion,720p | -30.9% | -20.5% | -23.5% |
| RGB, mixed content, 1440p | -26.2% | -15.2% | -14.9% |
| RGB, mixed content, 1080p | -23.5% | -13.6% | -13.8% |
| RGB, Animation, 720p | -25.3% | -18.1% | -15.4% |
| RGB, camera captured, 1080p | -25.3% | -5.1% | -9.9% |
| YUV, text & graphics with motion, 1080p | -31.3% | -33.7% | -32.8% |
| YUV, text & graphics with motion,720p | -17.8% | -22.8% | -26.5% |
| YUV, mixed content, 1440p | -9.3% | -15.8% | -16.0% |
| YUV, mixed content, 1080p | -11.9% | -17.2% | -17.4% |
| YUV, Animation, 720p | 0.0% | -8.5% | -5.5% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 148% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -23.4% | -18.6% | -18.3% |
| RGB, text & graphics with motion,720p | -30.5% | -17.5% | -22.2% |
| RGB, mixed content, 1440p | -31.4% | -16.4% | -15.8% |
| RGB, mixed content, 1080p | -25.3% | -13.1% | -12.9% |
| RGB, Animation, 720p | -25.6% | -16.2% | -11.8% |
| RGB, camera captured, 1080p | -27.9% | -5.4% | -14.1% |
| YUV, text & graphics with motion, 1080p | -17.6% | -21.2% | -20.7% |
| YUV, text & graphics with motion,720p | -14.8% | -19.1% | -24.0% |
| YUV, mixed content, 1440p | -7.1% | -14.9% | -15.0% |
| YUV, mixed content, 1080p | -8.6% | -15.3% | -15.0% |
| YUV, Animation, 720p | 0.0% | -8.3% | -4.7% |
| YUV, camera captured, 1080p | 0.4% | -0.1% | 0.1% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -18.1% | -13.3% | -12.9% |
| RGB, text & graphics with motion,720p | -26.4% | -11.6% | -16.4% |
| RGB, mixed content, 1440p | -30.5% | -12.3% | -11.0% |
| RGB, mixed content, 1080p | -23.0% | -6.8% | -6.6% |
| RGB, Animation, 720p | -24.3% | -11.5% | -5.1% |
| RGB, camera captured, 1080p | -26.2% | -1.7% | -12.1% |
| YUV, text & graphics with motion, 1080p | -11.9% | -15.9% | -15.4% |
| YUV, text & graphics with motion,720p | -9.5% | -14.0% | -17.9% |
| YUV, mixed content, 1440p | -4.2% | -12.5% | -12.7% |
| YUV, mixed content, 1080p | -4.2% | -13.0% | -11.9% |
| YUV, Animation, 720p | -0.1% | -6.4% | -4.7% |
| YUV, camera captured, 1080p | 0.3% | 0.0% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 106% |  |  |

Table 2. BD-Rate change in Lossless configuration

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 19.6% | 19.7% | 13.6% | 28.2% |
| RGB, text & graphics with motion,720p | 13.5% | 11.9% | 2.4% | 26.1% |
| RGB, mixed content, 1440p | 4.4% | 4.4% | 4.3% | 4.5% |
| RGB, mixed content, 1080p | 4.2% | 4.2% | 4.2% | 4.2% |
| RGB, Animation, 720p | 4.3% | 4.3% | 4.3% | 4.3% |
| RGB, camera captured, 1080p | 0.4% | 0.2% | -0.6% | 1.0% |
| YUV, text & graphics with motion, 1080p | 22.6% | 22.6% | 12.5% | 32.9% |
| YUV, text & graphics with motion,720p | 12.4% | 11.9% | 1.1% | 23.2% |
| YUV, mixed content, 1440p | 2.4% | 2.5% | 2.0% | 3.1% |
| YUV, mixed content, 1080p | 2.8% | 2.8% | 2.8% | 2.8% |
| YUV, Animation, 720p | 1.7% | 1.7% | 1.7% | 1.7% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 177% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.1% | 13.5% | 8.5% | 20.9% |
| RGB, text & graphics with motion,720p | 3.4% | 7.1% | 1.7% | 12.7% |
| RGB, mixed content, 1440p | 2.3% | 2.3% | 2.1% | 2.5% |
| RGB, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.9% |
| RGB, Animation, 720p | 1.8% | 1.8% | 1.8% | 1.8% |
| RGB, camera captured, 1080p | -0.2% | -0.2% | -0.4% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.8% | 15.1% | 10.3% | 24.2% |
| YUV, text & graphics with motion,720p | 3.0% | 7.3% | 0.9% | 16.2% |
| YUV, mixed content, 1440p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, mixed content, 1080p | 0.5% | 0.5% | 0.5% | 0.5% |
| YUV, Animation, 720p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 106% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 10.4% | 10.9% | 7.1% | 14.7% |
| RGB, text & graphics with motion,720p | 2.5% | 5.2% | 1.5% | 9.0% |
| RGB, mixed content, 1440p | 1.6% | 1.6% | 1.5% | 1.7% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 1.8% | 1.8% | 1.8% | 1.8% |
| RGB, camera captured, 1080p | -0.2% | -0.2% | -0.4% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.0% | 11.9% | 9.0% | 16.8% |
| YUV, text & graphics with motion,720p | 2.2% | 4.8% | 0.6% | 10.0% |
| YUV, mixed content, 1440p | 0.6% | 0.6% | 0.2% | 1.0% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 106% |  |  |  |

## HM15.0_RExt8.1_SCM2.0rc1, HM16.0_SCM2.1rc1 and HM16.1_SCM2.1rc1

HM15.0_RExt8.1_SCM2.1rc1 has been tagged on the SVN HHI repository on October 15th. This release includes:

- SCM source base updated to HM15.0_RExt8.1.

- Removal of unused macros.

- Fix for Ticket 1308

- Fix related to compilation issues when DEBUG_STRING is enabled.

The software can be downloaded at 
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.1+SCM-2.0rc1/

HM16.0_SCM2.1 and HM16.1_SCM2.1 has been committed on the SVN HHI repository on October 17th that updates SCM source to HM16.0 and HM16.1.

The coding performance of HM15.0_RExt8.1_SCM2.1rc1, HM16.0_SCM2.1rc1 and HM16.1_SCM2.1rc1 was observed to similar compared to HM15.0_RExt8.1_SCM2.1.

It is planned to tag and announce these version on the email reflector during this meeting cycle after removal of SCM2.0 macros.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-3.0, version SCC-2.0.

# Recommendations

- Continue to develop reference software based on HM16.1_ SCM2.1 and improve its quality.

- Remove macros introduced in previous versions before starting integration towards SCM-3.0 such as to make the software more readable.

- Continue merging with later HM versions.

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, July. 2014.