| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0010 |
| --- | --- |

| Title: | JCT-VC AHG report: Complexity of IBC, intra line & intra string copy coding (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Joel Sole Shan Liu Jizheng Xu | Email: | joels@qti.qualcomm.com Shan.Liu@mediatek.com jzxu@microsoft.com |
| Source: | AhG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on “Complexity of IBC, intra line & intra string copy coding (AHG10)” between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg, FR (October 2014).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| Complexity of IBC, intra line & intra string copy coding (AHG10) (jct-vc@lists.rwth-aachen.de) - Analyze complexity characteristics of f IBC, intra line & intra string copy methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, pixel processing. - Analyze the complexity impact of the search area size on the design. - Quantify the average and worst-case throughput (e.g., in context-coded bins) for these methods and compare them with the average and worst-case throughput for other coding elements. - Identify criteria to determine the hardware implementability of the key elements. - Identify bottlenecks in the current design with regard to implementation complexity. | J. Sole (chair) S. Liu J. Xu (vice-chairs) | No |

# Status

Analysis related to memory bandwidth and CABAC throughput have been carried out during this meeting cycle.

## Memory Bandwidth

A software patch and excel sheets were released by Chun-Chi Chen for intra BC and intra line copy based on the memory bandwidth tool that had been used in SCCE1 (JCTVC-Q1121) for memory bandwidth calculations. The software includes in the SCM2.0 decoder the functionality to calculate the read/write memory access bandwidth per frame for "motion compensation" and "intra block copy" under various memory configurations.

In the context of CE4, the average results for different configurations and memory access parameters are provided.

The worst-case memory bandwidth has been computed using the formula

where m × n is the memory tile size (memory pattern) and M×N the accessed block size. Results are summarized in the table below:

|  | Configurations |  |  |  |  |  |  | Per-pixel Memory Access Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Block Size |  | Num. of Samples | IF Length | Bi Pred? | Memory Pattern | m= | 4 | 8 | 4 | 8 | 4 |
| Prediction Mode | M | N | MxN | L | Y or N |  | n= | 1 | 1 | 2 | 2 | 4 |
| 8x8, Bi-prediction | 8 | 8 | 64 | 8 | Y |  |  | 9.375 | 11.25 | 10 | 12 | 12.5 |
| 4x4, IntraBC | 4 | 4 | 16 | 1 | N |  |  | 2 | 4 | 3 | 6 | 4 |
| 1x4, IntraLC (R0132) | 1 | 4 | 4 | 1 | N |  |  | 4 | 8 | 6 | 12 | 8 |

## CABAC throughput

The CABAC throughput has also been analyzed for intra BC and intra line copying. The table below provides the worst-case scenario of context coded bins for intra BC and intra LC. The main difference between both methods are highlighted.

| Prediction Mode | 4x4, IntraBC (SCM-2.0) |  | 4x1, IntraLC (R0132) |  |
| --- | --- | --- | --- | --- |
| (Note: CU Size = 8x8) | Syntax Elements | Number | Syntax Elements | Number |
| Mode Flag | cu_transquant_bypass_flag | 1 | cu_transquant_bypass_flag | 1 |
|  | cu_skip_flag | 1 | cu_skip_flag | 1 |
|  | intra_bc_flag | 1 | intra_bc_flag | 1 |
|  |  |  | intra_lc_flag | 1 |
|  | pred_mode_flag | 1 | pred_mode_flag | 1 |
|  | palette_mode_flag | 1 | palette_mode_flag | 1 |
| Partition | part_mode | 3 | part_mode | 3 |
| MV or BV | abs_bvd_greater0_flag | 8 | abs_mvd_greater0_flag | 32 |
|  | mvp_l0_flag | 4 | abs_mvd_greater1_flag | 32 |
| Residual (4 4x4 TBs, 3 Color Components) | rqt_root_cbf | 1 | rqt_root_cbf | 1 |
|  | cu_residual_act_flag | 1 | cu_residual_act_flag | 1 |
|  | cu_qp_delta_abs | 5 | cu_qp_delta_abs | 5 |
|  | cu_chroma_qp_offset_flag | 1 | cu_chroma_qp_offset_flag | 1 |
|  | cu_chroma_qp_offset_idx | 5 | cu_chroma_qp_offset_idx | 5 |
|  | log2_res_scale_abs_plus1 | 32 | log2_res_scale_abs_plus1 | 32 |
|  | res_scale_sign_flag | 8 | res_scale_sign_flag | 8 |
|  | cbf_cb | 5 | cbf_cb | 5 |
|  | cbf_cr | 5 | cbf_cr | 5 |
|  | cbf_luma | 4 | cbf_luma | 4 |
|  | split_transform_flag | 1 | split_transform_flag | 1 |
|  | transform_skip_flag | 12 | transform_skip_flag | 12 |
|  | explicit_rdpcm_flag | 12 | explicit_rdpcm_flag | 12 |
|  | explicit_rdpcm_dir_flag | 12 | explicit_rdpcm_dir_flag | 12 |
|  | last_sig_coeff_x_prefix | 36 | last_sig_coeff_x_prefix | 36 |
|  | last_sig_coeff_y_prefix | 36 | last_sig_coeff_y_prefix | 36 |
|  | sig_coeff_flag | 180 | sig_coeff_flag | 180 |
|  | coeff_abs_level_greater1_flag | 96 | coeff_abs_level_greater1_flag | 96 |
|  | coeff_abs_level_greater2_flag | 12 | coeff_abs_level_greater2_flag | 12 |
| Number of Ctx-coded Bins |  | 484 |  | 537 |
| Bins per Pixel |  | 7.56 |  | 8.39 |

# Recommendations

The recommendation of the AhG are

- Consider the data provided in the context of this AhG

- Review the contributions related to complexity of intra block, line and string copy.

- JCTVC-S0136, Non-CE4: On CABAC Throughput of Intra Line Copy

- JCTVC-S0145, On IntraBC bandwidth