| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0021 |
| --- | --- |

| Title: | Summary report of Core Experiment 1 (CE1): Vector Entropy Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chao Pang, Xiaozhong Xu | Email: | cpang@qti.qualcomm.com xiaozhong.xu@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of CE1 on vector entropy coding is reported. The CE is studying two aspects: block vector difference (BVD) entropy coding for intra block copy (IBC) mode, and motion vector difference (MVD) entropy coding for Inter mode. Performance of the proposed methods as well as combinations was evaluated for lossy and lossless configurations based on the SCC common test conditions.

# List of CE contributions

## Test 1: BVD entropy coding for IBC mode

- JCTVC-S0142, CE1 Test 1.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0166, Crosscheck of CE1 Test 1.1 (JCTVC-S0142) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0057, CE1 Test 1.3: CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0210, Crosscheck of CE1 Test 1.3 (JCTVC-S0057) [C.-H. Hung, C. -C Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)]

- JCTVC-S0162, CE1 Test 1.4: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0058, Crosscheck of CE1 Test 1.4 (JCTVC-S0162) [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

## Test 2: MVD entropy coding for Inter mode

- JCTVC-S0142, CE1 Test 2.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0166, Crosscheck of CE1 Test 2.1 (JCTVC-S0142) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0162, CE1 Test 2.2: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0226, Crosscheck of CE1 Test 2.2 (JCTVC-S0162) [C. Rosewarne, M. Maeda (Canon)]

## Test 3: MVD and BVD entropy coding for Inter and IBC modes

- JCTVC-S0142, CE1 Test 3.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0059, Crosscheck of CE1 Test 3.1 (JCTVC-S0142) [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

- JCTVC-S0061, CE1 Test 3.2: CE1: Results of Test 3.2 on MVD and BVD coding, G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)

  - JCTVC-S0146, Crosscheck of CE1 Test 3.2 (JCTVC-S0061) [K. Rapaka (Qualcomm)]

- JCTVC-S0162, CE1 Test 3.3: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0076, Crosscheck of CE1 Test 3.3 (JCTVC-S0162) [G. Laroche (Canon)]

## Test 4: Combination tests on BVD entropy coding for IBC mode

- JCTVC-S0057, CE1 Test 4.1: combination of Test 1.1 + Test 1.3, CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0167, Crosscheck of CE1 Test 4.1 (JCTVC-S0057) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0057, CE1 Test 4.3: combination of Test 1.3 + Test 1.4, CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0147, Crosscheck of CE1 Test 4.3 (JCTVC-S0057) [K. Rapaka (Qualcomm)]

## Summary of CE1 contributions

| Test | Description | Proponent | JCTVC # | Status | Crosscheck | JCTVC # |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 | BVD coding | Qualcomm | S0142 | Provided | MediaTek | S0166 |
| 1.3 | BVD coding | Mitsubishi | S0057 | Provided | ITRI | S0210 |
| 1.4 | BVD coding | MediaTek | S0162 | Provided | Mitsubishi | S0058 |
| 2.1 | MVD coding | Qualcomm | S0142 | Provided | MediaTek | S0166 |
| 2.2 | MVD coding | MediaTek | S0162 | Provided | Canon (Australia) | S0226 |
| 3.1 | BVD&MVD coding | Qualcomm | S0142 | Provided | Mitsubishi | S0059 |
| 3.2 | BVD&MVD coding | Canon (France) | S0061 | Provided | Qualcomm | S0146 |
| 3.3 | BVD&MVD coding | MediaTek | S0162 | Provided | Canon (France) | S0162 |
| 4.1 | 1.1 + 1.3 BVD coding | Mitsubishi | S0057 | Provided | MediaTek | S0167 |
| 4.3 | 1.3 + 1.4 BVD coding | Mitsubishi | S0057 | Provided | Qualcomm | S0147 |

# Related non-CE1 contributions

There is 1 contribution related to CE1 topics:

- JCTVC-S0143, Non-SCCE1: Non-CE1 :Block vector coding for Intra block copy, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm), K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0225, ‘Non-CE1 :Block vector coding for Intra block copy (JCTVC-S0143) [C. Rosewarne (Canon)]

# Summary of Results

The proposed methods were implemented on top of SCM2.0 and simulated under the SCC common test conditions.

## Test 1 & Test 4: BVD entropy coding for IBC mode

- Test 1.1: BVD coding for IBC (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than 4. If BVD is greater than 4, the remaining absolute level is coded using bypass 4th-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the BVD.

- Test 1.3: Adaptive switching between differential and direct coding for intra block copy vectors (JCTVC-S0057)

In this test, one flag is added to specify whether the BV is predicted or not. If not predicted, BV is coded without prediction. The flag is bypass coded and signalled only when the BV component is non-zero. An additional modification of the coding process of sign flags of vectors to reduce redundancy is also tested.

- Test 1.4: BVD coding for IBC using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each BVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

- Test 4.1: Combination of Test 1.1 + Test 1.3 (JCTVC-S0057)

In this test, the combination of Test 1.1 and Test 1.3 is investigated.

- Test 4.3: Combination of Test 1.3 + Test 1.4 (JCTVC-S0057)

In this test, the combination of Test 1.3 and Test 1.4 is investigated.

### Results for Test 1 & Test 4 on BVD coding for IBC mode

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) |  | Non-CE1 (1.1 + 4.1) |  |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.6% | -0.4% | -0.5% | -0.9% | -0.6% |  | -0.9% |  | -1.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% |  | -0.6% |  | -0.5% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% |  | -0.3% |  | -0.4% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.3% | -0.2% |  | -0.4% |  | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.7% | -0.5% | -0.6% | -1.0% | -0.7% |  | -1.1% |  | -1.2% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.3% | -0.4% | -0.2% | -0.5% | -0.4% |  | -0.6% |  | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.4% | -0.2% |  | -0.4% |  | -0.5% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.4% | -0.1% | -0.2% | -0.5% | -0.3% |  | -0.5% |  | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 96% | 100% | 100% | 100% | 102% | 102% | 100% | 100% |  | 106% |  |
| Dec Time[%] | 100% | 99% | 99% | 100% | 100% | 100% | 100% | 99% |  | 108% |  |

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) | Non-CE1 (1.1 + 4.1) |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.4% | -0.1% | -0.3% | -0.5% | -0.2% | -0.4% | -0.6% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.4% | -0.1% | -0.3% | -0.5% | -0.2% | -0.5% | -0.6% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% | 0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% | -0.3% | 0.0% | -0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 100% | 100% | 99% | 101% | 101% | 101% | 100% | 92% |
| Dec Time[%] | 100% | 100% | 99% | 101% | 100% | 99% | 100% | 100% | 96% |

|  | Low Delay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) | Non-CE1 (1.1 + 4.1) |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.4% | 0.0% | -0.3% | -0.5% | -0.2% | -0.4% | -0.5% |
| RGB, text & graphics with motion,720p | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.2% | 0.2% | 0.1% | -0.1% | -0.1% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.5% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.3% | 0.0% | -0.2% | -0.4% | -0.1% | -0.3% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | 0.0% | -0.2% | -0.4% |
| YUV, mixed content, 1080p | 0.1% | -0.5% | -0.4% | 0.3% | -0.2% | -0.1% | 0.0% | 0.0% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 84% | 100% | 100% | 100% | 101% | 100% | 101% | 101% | 89% |
| Dec Time[%] | 100% | 99% | 97% | 100% | 100% | 98% | 99% | 99% | 95% |

### Complexity analysis

|  | Complexity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) |
| # additional regular bins per BV | 2 | 0 | 0 | 12 | same as 1.1 | same as 1.1 | same as 1.4 | same as 1.4 |
| # additional contexts for BV | 2 | 0 | 0 | 4 for AI 2 for RA/LD | same as 1.1 | same as 1.1 | same as 1.4 | same as 1.4 |

## Test 2: MVD entropy coding for Inter mode

- Test 2.1: MVD coding for Inter (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than 4. If MVD is greater than 4, the remaining absolute level is coded using bypass 4-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the MVD.

- Test 2.2: MVD coding using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

### Results for Test 2 on MVD entropy coding for Inter mode

|  | Random Access |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.8% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% |
| RGB, mixed content, 1440p | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.3% |
| YUV, text & graphics with motion, 1080p | -2.6% | -3.1% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.2% |
| Enc Time[%] | 95% | 95% |
| Dec Time[%] | 100% | 100% |

|  | Low Delay |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| RGB, text & graphics with motion, 1080p | -3.0% | -3.6% |
| RGB, text & graphics with motion,720p | -1.1% | -1.3% |
| RGB, mixed content, 1440p | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -3.3% | -4.0% |
| YUV, text & graphics with motion,720p | -1.3% | -1.7% |
| YUV, mixed content, 1440p | 0.0% | -0.3% |
| YUV, mixed content, 1080p | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | -0.2% |
| Enc Time[%] | 93% | 94% |
| Dec Time[%] | 100% | 100% |

### Complexity analysis

|  | Complexity |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| # additional regular bins per MV | 0 | 10 |
| # additional contexts for MV | 2 | 4 |
| modification to binarization | yes | no |

## Test 3: MVD and BVD entropy coding for Inter and IBC modes

- Test 3.1: BVD coding for IBC and MVD coding for Inter (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate BVD/MVD is zero. When BVD/MVD is not zero, another flag is coded to indicate if absolute level of BVD/MVD is greater than 4. If BVD/MVD is greater than 4, the remaining absolute level is coded using bypass 4-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the BVD/MVD.

- Test 3.2: MVD and BVD entropy coding for Inter and IBC modes (JCTVC-S0061)

In this test, a flag at slice level is used to determine the residual coding method for both Inter and IBC modes. The current SCM2.0 BVD and MVD coding should be considered with this method or with one of the proposed BVD methods in section 1.1.

- Test 3.3: Unified BVD & MVD coding using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each BVD and MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

### Results for Test 3 on MVD and BVD entropy coding for Inter and IBC modes

- Test 3.3* further provides the results with the constraint on the max number of regular bins per CU compared with those of the proposed method in test 3.3 without the constraint.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -0.3% | 0.0% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | 0.0% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | 0.0% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% | 98% | 99% |
| Dec Time[%] | 100% | 100% | 101% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -2.4% | -1.5% | -2.7% |
| RGB, text & graphics with motion,720p | -0.6% | 0.1% | -0.9% |
| RGB, mixed content, 1440p | -0.1% | 0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -2.7% | -1.8% | -3.0% |
| YUV, text & graphics with motion,720p | -0.7% | 0.0% | -0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.2% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | 0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% | 99% | 95% |
| Dec Time[%] | 100% | 99% | 100% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -3.1% | -1.8% | -3.4% |
| RGB, text & graphics with motion,720p | -1.2% | -0.2% | -1.5% |
| RGB, mixed content, 1440p | -0.2% | 0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.6% | 0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -3.4% | -2.1% | -3.9% |
| YUV, text & graphics with motion,720p | -1.4% | -0.4% | -1.8% |
| YUV, mixed content, 1440p | -0.3% | 0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.4% | 0.2% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 87% | 98% | 94% |
| Dec Time[%] | 100% | 98% | 99% |

### Complexity analysis

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3* |
| # additional regular bins per BV | 2 | 0 | 12 |
| # additional regular bins per MV | 0 | 0 | 10 |
| # additional contexts for MV & BV | 4 | 0 | 2 |
| unification of MVD & BVD coding |  |  | same binarization & same contexts |

- Test 3.3 reportedly shows the results generated without violating the current CABAC throughput constraint for each CU.