| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0022 |
| --- | --- |

| Title: | CE2: Summary report for Core Experiment 2 on intra block copy signalling and partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jizheng Xu Shan Liu Krishna Rapaka Xiaoyu Xiu | Email: | jzxu@microsoft.com shan.liu@mediatek.com krapaka@qti.qualcomm.com xiaoyu.xiu@interdigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes CE2 test results on intra block copy signalling and partitioning. Several tests and combinations were tested, as described in the CE description document JCTVC-R1102.

# Introduction

At the 18th JCT-VC meeting, a CE was created to evaluate various proposals on intra block copy (IBC) signalling and partitioning designs for intra block copy. The tests in this CE, i.e. CE2, include using inter signalling and portioning design for intra block copy, PU-based intra block copy, block vector derivation for merge mode and related combinations. SCM2.0 was used as the base software for CE2 and the test condition follows the common test condition described in JCTVC-R1015.

# List of CE proposals

| Document number | Proponent | Title | Cross-check report |
| --- | --- | --- | --- |
| JCTVC-S0080 | MediaTek Microsoft | CE2: Result of Test 1 | JCTVC-S0127 |
| JCTVC-S0121 | MediaTek | CE2: Test 2 – Intra BC signalled at PU level | JCTVC-S0183 JCTVC-S0234 |
| JCTVC-S0131 | InterDigital | CE2: Test3 – IBC with block vector derivation | JCTVC-S0124 |
| JCTVC-S0122 | Media Tek Microsoft | CE2: Test 5 – Intra BC unified with Inter using intra_bc_flag | JCTVC-S0171 |
| JCTVC-S0081 | Microsoft | CE2: Result of Test 6 | JCTVC-S0125 |

# Test description and result

## Test 1 Unification of IBC to inter (JCTVC-S0080)

In this test, IBC block is treated as an inter block with reference to the current picture so that IBC can reuse the design for the inter mode. The modifications are

- When Intra BC mode is enabled, adding the current picture (before deblocking and SAO) to the last position of list 0, marking as long-term reference picture.

- If the current slice is I slice and Intra BC mode is enabled, the syntax parsing process and decoding process of P slice is followed.

- When Intra BC mode is enabled, after decoding the current picture, mark the current picture as short-term reference picture.

The summarized results are showed below –

## Test 2 - PU-based IBC using intra_bc_flag signalling (JCTVC-S0121)

In this test, IBC flag is signalled at PU level instead of CU level. Two sub-tests were conducted. In test 2a, IBC mode is coded as an intra mode and in test 2b, IBC mode is coded as an inter mode.

The summarized results of test 2a are shown below-

The summarized results of test 2b are shown below-

## Test 3 - IBC with block vector derivation (JCTVC-S0131)

In this test, five spatially neighboring BVs and their possible BV derivations are used as merge candidates for the current IBC block. The neighboring BVs and BV derivation are illustrated below –

Two sub tests were conducted. In test 3a, BV derivation is used as to construct more merge candidates for an IBC block. In test 3b, BV derivation is further applied to derive a BV/MV for an IBC block.

The summarized results of test 3a are shown below –

And the summarized results of test 3b are shown below –

## Test 5 – Combination of test 1 and test 2 (JCTVC-S0122)

The test is based on test 1. The modification is to signal IBC using intra_bc_flag at PU level. Two sub-tests were conducted. In test 5a, coding tools designed for Inter mode, such as skip mode, merge mode, MVD coding and AMVP are applied to IBC blocks. In test 5b, skip mode and merge mode in HEVC Inter coding are applied to IBC blocks, i.e., BVD and BVP coding are the same as the current IBC design.

The summarized results of test 5a are shown below –

And the summarized results of test 5b are shown below –

Test 5b with an encoder improvement as used in Test 3 is also provided. The summarized results are –

## Test 6 – Combination of test 1 and the current IBC design (JCTVC-S0081)

In this test, a CU-level flag is sent to indicate IBC flag. Two sub-tests were conducted. In test 6a, both IBC merge mode and IBC AMVP mode are unified with inter mode. In test 6b, only IBC merge mode is unified with inter mode, i.e., BVD and BVP coding are the same as the current IBC design.

The summarized results of test 6a are shown below –

The summarized results of test 6b are shown below –