| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0023_r1 |
| --- | --- |

| Title: | CE3: Summary report on Core Experiment 3 on sub-block partitioning and flipping for Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shan Liu | Email: | shan.liu@mediatek.com |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document summarizes the activities in Core Experiment 3 on sub-block partitioning and flipping for Intra block copy. A set of individual coding tools and their combinations were evaluated. The evaluated tools and the test conditions are described in document JCTVC-R1103 [1].

# Introduction

At the 18th JCT-VC meeting, it was shown in JCTVC-R0050 [2], JCTVC-R0117 [3] and JCTVC-R0204 [4], that improvements in coding efficiency for screen contents may be achieved by performing Intra block copy with sub-block partitioning, flipping, masking and segmentation. This core experiment was established to further investigate these coding tools, their combinations and interactions.

# List of participants

| Company/ Organization | Participants | Email Address | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Canon | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |  |  |
| Ericsson | Martin Pettersson | martin.m.pettersson@ericsson.com |  | X |
| Intel | Wenhao Zhang | Wenhao.zhang@intel.com |  | X |
| ITRI | Chun-Lung Lin Ching-Chieh Lin | chunlung@itri.org.tw jacklin@itri.org.tw (jacklin.itri@gmail.com) |  |  |
| MediaTek | Xiaozhong Xu Jing Ye Shan Liu Kai Zhang Xianguo Zhang | xiaozhong.xu@mediatek.com jing.ye@mediatek.com shan.liu@mediatek.com kai.zhang@mediatek.com xianguo.zhang@mediatek.com | X |  |
| Microsoft | Ji-zheng Xu | jzxu@microsoft.com |  | X |
| Nokia | Jani Lainema DöneBugdayci Sansli | Jani.lainema@nokia.com ext-done.bugdayci@nokia.com | X |  |
| Qualcomm | Krishna Rapaka Chao Pang Ying Chen Joel Sole | krapaka@qti.qualcomm.com cpang@qti.qualcomm.com cheny@qti.qualcomm.com joels@qti.qualcomm.com |  |  |

# List of CE contributions

| Doc No. | Participants | Title | Type |
| --- | --- | --- | --- |
| JCTVC-S0032 | Nokia | CE3: Test 1.1 – Intra block copy masking | Proposal |
| JCTVC-S0107 | MediaTek | CE3 Test 1.2: Segmental prediction for intra block copy | Proposal |
| JCTVC-S0117 | MediaTek | CE3 Test 1.3: Intra block copy with flipping | Proposal |
| JCTVC-S0118 | MediaTek, Nokia | CE3 Test 2.1: Combined test of test 1.1 and test 1.3 | Proposal |
| JCTVC-S0119 | MediaTek | CE3 Test 2.2: Combined test of test 1.2 and test 1.3 | Proposal |
| JCTVC-S0077 | Ericsson | CE3: Cross-check of test 1.1 | Crosscheck |
| JCTVC-S0263 | Intel | CE3 Test 1.2: Crosscheck for Segmental Prediction for Intra Block Copy (JCTVC-S0107) | Crosscheck |
| JCTVC-S0251 | Microsoft | CE3: cross-check of Test 1.3: Intra block copy with flipping (JCTVC-S0117) | Crosscheck |
| JCTVC-S0252 | Microsoft | CE3: cross-check of Test 2.1 Combined test of test 1.1 and test 1.3 (JCTVC-S0118) | Crosscheck |
| JCTVC-S0235 | Intel | CE3: Cross-check of test 2.2 | Crosscheck |

# Summary of proposals

## Test 1.1: Intra block copy masking

The mask is covering a square area extending from one of the borders of the PU and having a width varying from 1 to 3 sample rows or columns. All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black samples in figure below illustrate the location of the sample which value is used in the substitution process.

Figure 1: Example of masks on different borders of the prediction block

## Test 1.2: Segmental prediction for intra block copy

For an IBC CU, a flag is signaled to indicate whether segIBC is applied. If segIBC is on, another flag is signaled to indicate the number of segments, 2 or 3, and then followed by the coded offsets of the segments. segIBC is only applied for lossy coding.

When segIBC is applied, two steps are applied to modify the IBC prediction block before it is used to predict the current block. Figure 2 illustrates an example of segIBC with 2 segments for an 8x8 block.

In the first step, samples in an IBC prediction block are classified into two or three segments. When the prediction block is divided into two segments, a sample is classified to segment 0 if its value is smaller than the average value of all the samples in the IBC prediction block. Otherwise, it is classified into segment 1. When the prediction block is divided into three segments, two thresholds T1 and T2 are calculated as T1= (T+Vmin)/2 and T2= (Vmax+T)/2, where T is the average value of all the samples in the IBC prediction block. Vmax and Vmin are the minimum sample value and the maximum sample value in the IBC prediction block respectively. A sample is classified into segment 0 if its value is smaller than T1; it is classified into segment 2 if its value is larger than T2; otherwise it is classified into segment 1.

In the second step, a single value is assigned to all samples in each segment of the IBC prediction block. The single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the IBC prediction block and O is an offset signaled from the encoder to the decoder. E = (Vsegmax+Vsegmin)/2, where Vsegmax and Vsegmin are the minimum sample value and the maximum sample value in the segment of the IBC prediction block respectively.

At encoder, the offset for the segment of the IBC prediction block is calculated by subtracting the average value of all the original sample values in the corresponding segment of the current block and E.

For an IBC CU, a flag is signaled to indicate whether segIBC is applied. If segIBC is on, another flag is signaled to indicate the number of segments, 2 or 3, and then followed by the coded offsets of the segments. segIBC is only applied for lossy coding.

There are four types of configurations. The results below have 2 segmentations for each CU and with all residues set to be 0 when segIBC is applied.

Figure 2:  An example of segIBC with 2 segments for an 8x8 block.

## Test 1.3: Intra block copy with flipping

In was proposed in and JCTVC-R0204 [4], JCTVC-R0097 [5] and JCTVC-R0116 [6] that the reference block can be flipped for Intra block copy prediction for higher coding gain. When Intra block copy with flipping is enabled, the reference block is flipped either vertically or horizontally specified by a flipping direction flag. Figure 3 illustrates the examples for vertical flipping and horizontal flipping in Intra block copy.

In this test, Intra block copy flipping may be applied to 2Nx2N, 2NxN, Nx2N, and NxN blocks. An ibc_flipping_flag is signalled for each Intra block copy prediction unit to indicate whether its prediction is flipped or not. Two results are provided for ibc_flag signalled at CU and PU level, respectively.

Figure 3: Vertical flipping and horizontal flipping in Intra block copy

## Test 2.1: Combination of test 1.1 and test 1.3

In this test, Intra block copy with masking and Intra block copy with flipping are combined.

## Test 2.2: Combinations of test 1.2 and test 1.3

In this test, segmental prediction for Intra block copy and Intra block copy with flipping are combined.

# Results

The proposed methods were implemented on top of SCM2.0 and simulated under the SCC common test conditions defined in JCTVC-R1015 [7].

## Results of test 1.1, test 1.2 and test 1.3

### Lossy results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.9% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.7% | -0.6% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -1.3% | -1.3% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -1.1% | -1.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.1% | -1.4% | -1.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.6% | -0.7% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -1.3% | -1.3% |
| YUV, mixed content, 1080p | -0.3% | -0.3% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 104% | 115% | 115% |
| Dec Time[%] | 100% | 101% | 101% | 101% |
|  | Random Access |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.8% | -1.7% |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.4% | -0.7% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.4% | -0.1% | -0.6% | -0.7% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.8% | -1.6% |
| YUV, text & graphics with motion,720p | -0.1% | -0.4% | -0.4% | -0.6% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.6% | -0.8% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.5% | -0.8% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 102% | 103% |
| Dec Time[%] | 97% | 100% | 101% | 102% |
|  | Low delay B |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.6% | -1.6% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.4% | -0.6% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.2% | -0.5% |
| RGB, mixed content, 1080p | -0.2% | -0.1% | 0.0% | -0.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.6% | -1.6% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.5% | -0.8% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.3% | -0.6% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | 0.0% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 99% | 102% | 103% |
| Dec Time[%] | 100% | 99% | 100% | 98% |

### Lossless results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.8% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | -0.2% | 0.0% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.0% | 1.0% | 1.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.4% | 0.4% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 101% | 112% | 114% |
| Dec Time[%] | 100% | 99% | 98% | 100% |
|  | Random Access |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.2% | 1.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.0% | 0.4% | 1.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 103% | 106% |
| Dec Time[%] | 100% | 101% | 101% | 102% |
|  | Low delay B |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.2% | 1.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.4% | 1.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 101% | 103% | 106% |
| Dec Time[%] | 100% | 99% | 100% | 99% |

## Results of test 2.1 and test 2.2

### Lossy results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | -1.8% | -1.8% | -2.0% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -1.2% | -1.3% | -1.4% |
| RGB, mixed content, 1080p | -1.4% | -1.4% | -1.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.8% | -1.8% | -2.3% |
| YUV, text & graphics with motion,720p | -0.9% | -0.8% | -1.1% |
| YUV, mixed content, 1440p | -1.3% | -1.3% | -1.5% |
| YUV, mixed content, 1080p | -1.3% | -1.3% | -1.3% |
| YUV, Animation, 720p | 0.2% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116% | 115% | 125% |
| Dec Time[%] | 101% | 102% | 102% |
|  | Random Access |  |  |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | -1.2% | -2.0% | -2.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.9% | -1.0% |
| RGB, mixed content, 1440p | -0.7% | -0.9% | -1.1% |
| RGB, mixed content, 1080p | -0.8% | -1.0% | -0.9% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.9% | -2.4% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -1.0% |
| YUV, mixed content, 1440p | -0.7% | -0.8% | -1.1% |
| YUV, mixed content, 1080p | -0.8% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.1% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 102% | 104% | 106% |
| Dec Time[%] | 101% | 100% | 106% |
|  | Low delay B |  |  |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.9% | -2.3% |
| RGB, text & graphics with motion,720p | -0.5% | -0.7% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.4% | -1.0% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.8% | -2.3% |
| YUV, text & graphics with motion,720p | -0.6% | -0.8% | -1.1% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -0.9% |
| YUV, mixed content, 1080p | -0.1% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% | 104% | 105% |
| Dec Time[%] | 101% | 101% | 104% |

### Lossless results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.9% |
| RGB, text & graphics with motion,720p | 0.4% | 0.4% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 1.3% | 1.0% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.4% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% | 114% | 121% |
| Dec Time[%] | 99% | 100% | 102% |
|  | Random Access |  |  |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | 0.5% | 1.3% | 1.1% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 1.4% | 1.2% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% | 107% | 107% |
| Dec Time[%] | 105% | 103% | 105% |
|  | Low delay B |  |  |
|  | 2.1 CU | 2.1 PU | 2.2 PU |
| RGB, text & graphics with motion, 1080p | 0.4% | 1.4% | 1.2% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 1.5% | 1.3% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% | 106% | 107% |
| Dec Time[%] | 99% | 102% | 103% |

# Reference

- S. Liu, “Description of Core Experiment 3 (CE3): Sub-Block Partitioning and Flipping for Intra Block Copy”, JCTVC-R1103, Sapporo, JP, July 2014.

- J. Lainema, K. Ugur and M. M. Hannuksela, “Non-SCCE1: Intra block copy masking”, JCTVC-R0050, Sapporo, JP, July 2014.

- K. Zhang, J. An, X. Zhang, H. Huang and S. Lei, “Non-SCCE1: Segmental prediction for intra block copy”, JCTVC-R0117, Sapporo, JP, July 2014.

- X. Xu, S. Liu, J. Ye, K. Zhang and S. Lei, “Non-SCCE1: PU Intra block copy with flipping mode”, JCTVC-R0204, Sapporo, JP, July 2014.

- B. Li and J. Xu, “SCCE1: Result of Test 2.2 2.3 and 4.2”, JCTVC-R0097, Sapporo, JP, July 2014.

- K. Zhang, J. An, X. Zhang, H. Huang and S. Lei, “SCCE1 Test 4.1: Symmetric intra block copy”, JCTVC-R0117, Sapporo, JP, July 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Test Conditions for Screen Content Coding”, JCTVC-R1015, Sapporo, JP, July 2014.