| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-S0024 |
| --- | --- |

| Title: | CE4: Summary report for Core Experiment 4 on Intra Line Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Xiaozhong Xu Li Zhang Tao Lin | Email: | cheerchen.cs98g@g2.nctu.edu.tw xiaozhong.xu@mediatek.com lizhang@qti.qualcomm.com lintao@tongji.edu.cn |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This is a summary report on Core Experiment 4 (CE4) on intra line copy. Three experiments as planned in JCTVC-R1104 were investigated in this CE: (Test A) search range of intra line copy, (Test B) intra line copy with constrained BVs, and (Test C) self-matching intra line copy.

# Document List

## CE4 Contributions

- JCTVC-S0135, CE4: Results of Test A on Search Range of Intra Line Copy [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

- JCTVC-S0248, CE4: Cross-check of S0135 (Results of Test A on Search Range of Intra Line Copy) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)]

- JCTVC-S0194, CE4: Results of test C [L. Zhao, X. Chen, T. Lin (Tongji)]

- JCTVC-S0233, Crosscheck of JCTVC-S0194: CE4: Results of test C [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

## CE4-related Contributions

- JCTVC-S0136, Non-CE4: On CABAC Throughput of Intra Line Copy [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

- JCTVC-S0249, Non-CE4: Cross-check of S0136 (On CABAC Throughput of Intra Line Copy) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)]

- JCTVC-S0137, Non-CE4: Intra Line Copy with Extended Full-frame Search for Test A [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

- JCTVC-S0126, Cross check of Non-CE4 Intra Line Copy with Extended Full-frame Search for Test A (JCTVC-S0137) [X. Xu (MediaTek)]

# Test Conditions

All the proposed methods were implemented on top of the HM-15.0+RExt-8.0+SCM-2.0 software. Two test conditions formed based on the common test conditions (JCTVC-R1015) for screen content coding are used to evaluate the coding results for both the lossy and lossless operating points.

- Test Condition 1: Full-frame search range for IBC.

- Test Condition 2: 4-CTU (1 current + 3 left) search range for IBC.

Techniques to be tested under this test condition should follow the same restriction on the search range as that of IBC.

# Test Description and Results

## Test A: Search Range of Intra Line Copy

- Proponent: NCTU/ITRI (JCTVC-S0135)

- X-checker: Huawei USA R&D (JCTVC-S0248)

- Description: It is proposed to performing intra-copying operation for lines at PU level with 2-D BV for each line. In addition, a flag is present at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use. Each line has its own BV shared across its three colour components. Pixels within the current CU cannot be used as reference.

- BD-rate and Processing Time (4-CTU configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -6.4% | -6.1% | -6.2% | -3.7% | -3.3% | -3.4% | -2.4% | -2.1% | -2.2% |
| RGB, text & graphics w/ motion,720p | -3.3% | -3.0% | -3.0% | -2.6% | -2.3% | -2.3% | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -2.5% | -2.3% | -2.4% | -1.7% | -1.5% | -1.6% | -1.0% | -0.8% | -0.9% |
| RGB, mixed content, 1080p | -3.7% | -3.4% | -3.4% | -3.0% | -2.4% | -2.5% | -1.5% | -1.2% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.2% | -5.7% | -5.8% | -3.3% | -2.9% | -2.9% | -2.0% | -1.7% | -1.8% |
| YUV, text & graphics w/ motion,720p | -3.0% | -2.7% | -2.9% | -2.5% | -2.1% | -2.2% | -0.9% | -0.7% | -0.5% |
| YUV, mixed content, 1440p | -2.4% | -2.4% | -2.4% | -1.8% | -1.4% | -1.7% | -0.9% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | -3.7% | -3.1% | -3.4% | -2.9% | -2.4% | -2.7% | -1.6% | -1.1% | -1.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 119% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  | 102% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 7.1% | 4.8% | 11.1% | 5.3% | 2.7% | 8.3% | 4.4% | 2.5% | 6.0% |
| RGB, text & graphics w/ motion,720p | 2.1% | 0.5% | 4.6% | 1.2% | 0.3% | 3.6% | 0.6% | 0.2% | 1.7% |
| RGB, mixed content, 1440p | 1.1% | 0.4% | 1.7% | 0.2% | 0.1% | 0.3% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 7.4% | 5.2% | 11.2% | 5.6% | 3.3% | 8.5% | 4.9% | 3.1% | 6.4% |
| YUV, text & graphics w/ motion,720p | 2.5% | 0.6% | 5.8% | 1.5% | 0.3% | 4.6% | 0.8% | 0.2% | 2.2% |
| YUV, mixed content, 1440p | 1.2% | 0.5% | 1.9% | 0.2% | 0.2% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 100% |  |  |

- BD-rate and Processing Time (Full-frame Configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -3.8% | -3.6% | -3.6% | -2.2% | -1.9% | -2.0% | -1.5% | -1.3% | -1.3% |
| RGB, text & graphics w/ motion,720p | -1.3% | -1.1% | -1.2% | -1.1% | -0.9% | -0.9% | -0.7% | -0.6% | -0.4% |
| RGB, mixed content, 1440p | -1.5% | -1.3% | -1.4% | -1.1% | -0.9% | -1.0% | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -2.2% | -1.9% | -2.0% | -1.6% | -1.3% | -1.4% | -0.6% | 0.1% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -3.7% | -3.4% | -3.4% | -2.0% | -1.7% | -1.8% | -1.3% | -1.0% | -1.1% |
| YUV, text & graphics w/ motion,720p | -1.2% | -1.1% | -1.2% | -0.9% | -1.1% | -1.0% | -0.5% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -1.4% | -1.3% | -1.5% | -1.0% | -1.2% | -0.9% | -0.7% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -2.1% | -1.7% | -2.0% | -1.6% | -1.4% | -1.6% | -0.7% | -0.4% | -1.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 103% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 4.6% | 3.0% | 7.6% | 3.2% | 1.9% | 5.3% | 2.7% | 1.8% | 3.7% |
| RGB, text & graphics w/ motion,720p | 1.0% | 0.3% | 1.7% | 0.5% | 0.2% | 1.0% | 0.3% | 0.2% | 0.7% |
| RGB, mixed content, 1440p | 0.7% | 0.3% | 1.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 4.6% | 3.3% | 7.1% | 3.4% | 2.1% | 5.2% | 2.8% | 2.0% | 3.8% |
| YUV, text & graphics w/ motion,720p | 1.1% | 0.4% | 1.8% | 0.6% | 0.2% | 1.3% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.7% | 0.3% | 1.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

## Worst-case Memory Access Bandwidth

|  | Per-pixel Memory Access Bandwidth (P) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Memory Patterns Prediction Mode | 4x1 | 8x1 | 4x2 | 8x2 | 4x4 |
| 8x8, Bi-prediction | 9.375 | 11.25 | 10 | 12 | 12.5 |
| 4x4, IBC | 2 | 4 | 3 | 6 | 4 |
| 1x4, ILC | 4 | 8 | 6 | 12 | 8 |

## Memory Access Bandwidth:

  - Lossy Results:

| Lossy (Memory bandwidth) | All Intra (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 112.2% | 112.2% | 114.0% | 114.0% | 123.3% | 131.3% | 100.0% | 100.0% |
| RGB, text & graphics with motion, 720p | 105.9% | 105.9% | 106.8% | 106.8% | 111.6% | 115.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 105.7% | 105.7% | 106.7% | 106.7% | 111.6% | 115.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 108.9% | 108.9% | 110.4% | 110.4% | 118.0% | 123.8% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.5% | 100.5% | 100.6% | 100.6% | 100.9% | 101.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 111.1% | 111.1% | 112.6% | 112.6% | 120.4% | 127.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 720p | 105.1% | 105.1% | 105.9% | 105.9% | 109.8% | 113.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 104.9% | 104.9% | 105.7% | 105.7% | 110.0% | 113.8% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 108.0% | 108.0% | 109.4% | 109.4% | 116.1% | 121.8% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.8% | 101.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 105.2% | 105.2% | 106.1% | 106.1% | 110.2% | 113.7% | 100.0% | 100.0% |

| Lossy (Memory bandwidth) | Random Access (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.8% | 100.8% | 100.9% | 100.9% | 101.6% | 102.6% | 100.1% | 100.1% |
| RGB, text & graphics with motion, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.4% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.2% | 100.5% | 99.9% | 99.9% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.6% | 100.6% | 100.7% | 100.7% | 101.3% | 102.1% | 100.1% | 100.1% |
| YUV, text & graphics with motion, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.1% | 100.1% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.6% | 100.0% | 100.0% |

| Lossy (Memory bandwidth) | Low Delay B (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.8% | 100.8% | 100.9% | 100.9% | 101.7% | 102.7% | 100.0% | 100.0% |
| RGB, text & graphics with motion, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.3% | 100.3% | 100.1% | 100.1% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.6% | 100.6% | 100.7% | 100.7% | 101.2% | 102.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 99.9% | 99.9% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 99.9% | 99.9% |
| Average | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.6% | 100.0% | 100.0% |

| Lossy (Memory bandwidth) | All Intra (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 105.8% | 105.8% | 106.6% | 106.6% | 111.1% | 115.2% | 99.6% | 99.6% |
| RGB, text & graphics with motion, 720p | 102.9% | 102.9% | 103.3% | 103.3% | 105.6% | 107.6% | 99.8% | 99.8% |
| RGB, mixed content, 1440p | 103.5% | 103.5% | 104.1% | 104.1% | 107.3% | 109.8% | 99.8% | 99.7% |
| RGB, mixed content, 1080p | 104.9% | 104.9% | 105.7% | 105.7% | 110.1% | 113.4% | 99.6% | 99.5% |
| RGB, animation, 720p | 100.5% | 100.5% | 100.6% | 100.6% | 100.9% | 101.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 105.0% | 105.0% | 105.7% | 105.7% | 109.3% | 112.4% | 99.6% | 99.5% |
| YUV, text & graphics with motion, 720p | 102.5% | 102.5% | 102.8% | 102.8% | 104.7% | 106.4% | 99.9% | 99.9% |
| YUV, mixed content, 1440p | 102.8% | 102.8% | 103.3% | 103.3% | 105.9% | 108.2% | 99.8% | 99.7% |
| YUV, mixed content, 1080p | 104.4% | 104.4% | 105.1% | 105.1% | 108.9% | 112.0% | 99.6% | 99.5% |
| YUV, animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.8% | 101.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 102.7% | 102.7% | 103.2% | 103.2% | 105.4% | 107.2% | 99.8% | 99.8% |

| Lossy (Memory bandwidth) | Random Access (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.5% | 100.5% | 100.9% | 101.5% | 100.0% | 100.0% |
| RGB, text & graphics with motion, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.1% | 100.1% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.4% | 100.0% | 100.0% |

| Lossy (Memory bandwidth) | Low Delay B (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.5% | 100.5% | 100.6% | 100.6% | 101.0% | 101.7% | 100.0% | 100.0% |
| RGB, text & graphics with motion, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.2% | 100.1% | 100.1% |
| RGB, mixed content, 1080p | 99.9% | 99.9% | 99.9% | 99.9% | 100.0% | 100.0% | 99.9% | 99.9% |
| RGB, animation, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.4% | 100.4% | 100.8% | 101.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% |
| YUV, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.4% | 100.4% | 100.4% | 100.4% | 100.4% | 100.4% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 99.9% |
| Average | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.4% | 100.0% | 100.0% |

  - Lossless Results:

| Lossless (Memory bandwidth) | All Intra (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 116.1% | 116.1% | 118.2% | 118.2% | 129.2% | 138.8% | 100.0% | 100.0% |
| RGB, text & graphics with motion, 720p | 110.2% | 110.2% | 111.6% | 111.6% | 119.0% | 125.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 110.2% | 110.2% | 111.7% | 111.7% | 118.4% | 122.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 112.4% | 112.4% | 114.2% | 114.2% | 121.7% | 125.3% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.7% | 100.7% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 115.5% | 115.5% | 117.5% | 117.5% | 127.6% | 136.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 720p | 109.8% | 109.8% | 111.2% | 111.2% | 118.1% | 124.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 109.5% | 109.5% | 110.9% | 110.9% | 116.9% | 120.4% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 112.4% | 112.4% | 114.1% | 114.1% | 121.5% | 124.9% | 100.0% | 100.0% |
| YUV, animation, 720p | 101.4% | 101.4% | 101.6% | 101.6% | 102.2% | 102.4% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 108.2% | 108.2% | 109.3% | 109.3% | 114.6% | 118.5% | 100.0% | 100.0% |

| Lossless (Memory bandwidth) | Random Access (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 103.1% | 103.1% | 103.5% | 103.5% | 105.8% | 108.3% | 99.9% | 99.9% |
| RGB, text & graphics with motion, 720p | 100.3% | 100.3% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.2% | 100.2% | 100.2% | 100.2% | 100.5% | 100.8% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 103.3% | 103.3% | 103.7% | 103.7% | 106.2% | 108.7% | 99.7% | 99.8% |
| YUV, text & graphics with motion, 720p | 100.3% | 100.3% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.4% | 100.8% | 99.9% | 99.9% |
| YUV, mixed content, 1080p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.0% | 100.0% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 100.7% | 100.7% | 100.7% | 100.7% | 101.3% | 101.9% | 100.0% | 100.0% |

| Lossless (Memory bandwidth) | Low Delay B (4-CTU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 103.9% | 103.9% | 104.5% | 104.5% | 107.1% | 109.5% | 99.7% | 99.8% |
| RGB, text & graphics with motion, 720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.6% | 101.0% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 103.7% | 103.7% | 104.2% | 104.2% | 106.7% | 109.1% | 99.7% | 99.7% |
| YUV, text & graphics with motion, 720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.8% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 99.9% | 99.9% | 100.0% | 100.0% | 99.9% | 99.9% |
| Average | 100.7% | 100.7% | 100.8% | 100.8% | 101.4% | 101.9% | 99.9% | 99.9% |

| Lossless (Memory bandwidth) | All Intra (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 108.1% | 108.1% | 109.1% | 109.1% | 114.9% | 120.2% | 99.4% | 99.4% |
| RGB, text & graphics with motion, 720p | 105.9% | 105.9% | 106.7% | 106.7% | 110.8% | 114.6% | 99.8% | 99.8% |
| RGB, mixed content, 1440p | 106.4% | 106.4% | 107.3% | 107.3% | 111.9% | 114.9% | 99.7% | 99.7% |
| RGB, mixed content, 1080p | 107.4% | 107.4% | 108.6% | 108.6% | 113.5% | 115.9% | 99.6% | 99.6% |
| RGB, animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.7% | 100.7% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 107.7% | 107.7% | 108.6% | 108.6% | 113.9% | 118.5% | 99.5% | 99.5% |
| YUV, text & graphics with motion, 720p | 105.8% | 105.8% | 106.6% | 106.6% | 110.6% | 114.2% | 99.8% | 99.7% |
| YUV, mixed content, 1440p | 105.9% | 105.9% | 106.8% | 106.8% | 111.0% | 113.5% | 99.6% | 99.5% |
| YUV, mixed content, 1080p | 107.1% | 107.1% | 108.2% | 108.2% | 112.9% | 115.2% | 99.5% | 99.5% |
| YUV, animation, 720p | 101.4% | 101.4% | 101.6% | 101.6% | 102.2% | 102.3% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 104.7% | 104.7% | 105.3% | 105.3% | 108.5% | 110.8% | 99.7% | 99.7% |

| Lossless (Memory bandwidth) | Random Access (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 101.7% | 101.7% | 101.9% | 101.9% | 103.2% | 104.7% | 99.8% | 99.8% |
| RGB, text & graphics with motion, 720p | 100.2% | 100.2% | 100.3% | 100.3% | 100.4% | 100.8% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 101.9% | 101.9% | 102.2% | 102.2% | 103.6% | 105.1% | 99.7% | 99.7% |
| YUV, text & graphics with motion, 720p | 100.2% | 100.2% | 100.3% | 100.3% | 100.4% | 100.8% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| YUV, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| Average | 100.4% | 100.4% | 100.4% | 100.4% | 100.8% | 101.1% | 99.9% | 99.9% |

| Lossless (Memory bandwidth) | Low Delay B (Full-frame) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 102.2% | 102.2% | 102.4% | 102.4% | 104.0% | 105.5% | 99.7% | 99.7% |
| RGB, text & graphics with motion, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 99.9% | 99.9% |
| YUV, text & graphics with motion, 1080p | 102.1% | 102.1% | 102.4% | 102.4% | 103.9% | 105.4% | 99.6% | 99.6% |
| YUV, text & graphics with motion, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.7% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 99.9% |
| YUV, animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.1% |
| Average | 100.4% | 100.4% | 100.5% | 100.5% | 100.8% | 101.2% | 99.9% | 99.9% |

## Worst-case Number of Context-coded Bins:

## Test B: Intra Line Copy with Constrained BVs (Withdrawn)

## Test C: Self-matching Intra Line Copy

- Proponent: Tongji University (JCTVC-S0194)

- X-checker: NCTU/ITRI (JCTVC-S0233)

- Description: In contrast to the intra line copy proposed of CE4 Test A (JCTVC-S0135), this test additionally relaxes the constraint that the reference lines overlapped with the current CU cannot be used as reference. Two prediction modes are proposed. First, when the reference line is overlapped entirely with the current CU, pixels within the overlapped area is predicted by using the predictor of the reference line. Second, when partial overlapping occurs, pixels of the non-overlapped part will be replicated to predict the pixels in the overlapped part.

- BD-rate and Processing Time (Full-frame Configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -3.8% | -3.5% | -3.6% | -2.2% | -1.9% | -2.0% | -1.4% | -1.3% | -1.3% |
| RGB, text & graphics w/ motion,720p | -1.4% | -1.1% | -1.2% | -1.0% | -0.8% | -0.8% | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.5% | -1.4% | -1.4% | -1.1% | -1.0% | -1.0% | -0.7% | -0.6% | -0.7% |
| RGB, mixed content, 1080p | -2.2% | -1.9% | -2.0% | -1.6% | -1.3% | -1.5% | -0.8% | 0.3% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -3.7% | -3.4% | -3.4% | -2.0% | -1.7% | -1.7% | -1.3% | -1.0% | -1.1% |
| YUV, text & graphics w/ motion,720p | -1.2% | -1.1% | -1.3% | -1.0% | -1.2% | -1.1% | -0.5% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | -1.4% | -1.4% | -1.4% | -1.1% | -1.0% | -0.9% | -0.7% | -0.6% | -0.3% |
| YUV, mixed content, 1080p | -2.1% | -1.6% | -1.9% | -1.7% | -1.7% | -1.6% | -0.8% | 0.1% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 104% |  |  | 148% |  |  |
| Dec Time[%] | 166% |  |  | 157% |  |  | 171% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 4.5% | 2.9% | 7.5% | 3.2% | 1.8% | 5.4% | 2.6% | 1.7% | 3.8% |
| RGB, text & graphics w/ motion,720p | 0.9% | 0.3% | 1.7% | 0.4% | 0.2% | 1.0% | 0.3% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.7% | 0.3% | 1.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 4.6% | 3.2% | 7.3% | 3.4% | 2.1% | 5.4% | 2.9% | 2.0% | 4.0% |
| YUV, text & graphics w/ motion,720p | 1.1% | 0.3% | 1.9% | 0.5% | 0.2% | 1.3% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.8% | 0.3% | 1.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 136% |  |  | 177% |  |  | 153% |  |  |