| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0025 |
| --- | --- |

| Title: | CE5: Summary report of core experiment on investigation of maximum palette size and maximum palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rajan Joshi Xiaoyu Xiu | Emails: | rajanj@qti.qualcomm.com xiaoyu.xiu@interdigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the Core Experiment 5 (CE5) on investigation of maximum palette size and maximum palette predictor size. The BD-rate impact of changing the maximum palette size and maximum palette predictor size was studied.

# Introduction

In SCM-2.0, the maximum palette size is 31 and maximum palette predictor size is 64. In the 18th JCT-VC meeting in Sapporo, core Experiment 5 (CE5) [1] was formed to investigate the impact of varying the maximum palette size and the maximum palette predictor size on BD-rate. Both increase and decrease in palette size and palette predictor size was considered. This contribution describes test conditions, test results, and comparisons.

# Test conditions

All tests were conducted under SCM-2.0 common test conditions [2], which enabled full frame intra block copy (FF-IBC) as anchor.

# Test descriptions and results

Since palette is an intra tool, BD-results are presented and compared only for All-Intra (AI) lossy configuration. The trend for RA and LD-B configurations is similar to what is seen for AI configuration.

## Maximum palette size

Tester: Qualcomm, JCTVC-S0037

Crosschecker: InterDigital (JCTVC-S0271)

Maximum palette sizes of 16, 48, and 64 will be tested. The palette size will be signalled in the PPS or SPS. Setting and signalling the palette size based on the statistics of the previous picture will also be tested.

Results:

Table 1: BD-rate results for maximum palette size of 15 (left), 47 (middle) and 63 (right). Maximum palette predictor size 64, All-Intra lossy configuration

Table 2: BD-rate results for maximum palette size of 15 (left), 47 (middle) and 63 (right). Maximum palette predictor size 64, All-Intra lossless configuration

Note: The results were obtained by setting the macro MAX_PLT_SIZE to 15, 47, and 63, respectively in SCM-2.0.

## Maximum palette predictor size

Tester: InterDigital, JCTVC-S0189

Crosschecker: Qualcomm (JCTVC-S0222)

Maximum palette predictor sizes of 32, 96 and 128 will be tested for the default palette size in SCM 2.0 (31). Additionally, maximum palette predictor size equal to the maximum palette size (32) will also be tested. The palette predictor size will be signalled in the PPS.

Results:

Table 3: BD-rate results for maximum palette predictor size of 96 (left) and 128 (right). Maximum palette size 31, All-Intra lossy configuration

|  | All Intra |  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.3% | 0.7% | 0.7% | 0.7% | 0.4% | 1.1% |
| RGB, text & graphics with motion,720p | 0.7% | 0.5% | 0.0% | 1.6% | 0.9% | 0.7% | 0.0% | 2.3% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.3% | 2.4% | 1.8% | 2.0% | 0.4% | 4.2% |
| YUV, text & graphics with motion,720p | 0.7% | 0.5% | 0.0% | 1.8% | 1.0% | 0.7% | 0.0% | 2.5% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  |

Table 4: Bit-rate results for maximum palette predictor size of 96 (left) and 128 (right). Maximum palette size 31, All-Intra lossless configuration

## Combination

Tester: MediaTek, JCTVC-S0097

Crosschecker: No crosscheck

Various combinations of maximum palette sizes and maximum palette predictor sizes will be tested.

Results:

Table 5: BD-rate results for maximum palette size of 15, maximum predictor size of 32 (left) and maximum palette size of 15, maximum predictor size of 48 (right). All-Intra lossy configuration

Table 6: BD-rate results for maximum palette size of 15, maximum predictor size of 32 (left) and maximum palette size of 15, maximum predictor size of 48 (right). All-Intra lossless configuration

Note: Run-times may not be reliable. The results were obtained by setting the macros MAX_PLT_SIZE and MAX_PLT_PRED_SIZE appropriately in SCM-2.0.

# CE5 related non-CE contributions

- CE6: Test D.1 Run-length coding for reuse flags (JCTVC-S0153)

- Crosscheck for CE6 Test D.1 on palette table signalling (JCTVC-S0204) (registered but missing)

- Non-CE5: CU dependent color palette maximum size (JCTVC-S0201)

- CE5-related: Cross-check of JCTVC-S0201 on CU dependent color palette maximum size (S0106) (registered but missing)

# Conclusions

SCM-2.0 uses maximum palette size of 31 and maximum palette predictor size of 64. The BD-rate improvement from using higher maximum palette size or higher maximum palette predictor size is very small for lossy coding. Some bit-rate improvements from using higher maximum palette predictor size are observed for lossless coding. When both maximum palette size and maximum palette predictor size are reduced, BD-rate losses of more than 1.5% are observed for RGB and YUV text & graphics with motion, 1080p classes for All-Intra configurations.

# References

- R. Joshi and X. Xiu “Description of Screen Content Core Experiment 5 (SCCE5): Investigation of maximum palette size and maximum palette predictor size,” JCTVC-R1105, Sapporo, JP, June-July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, June-July 2014.