| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0026 |
| --- | --- |

| Title: | CE6: Summary report of CE on improvements of palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang (Primary) Patrice Onno Robert Cohen Vadim Seregin Xiaoyu Xiu Zhan Ma | Emails: | yuwen.huang@mediatek.com patrice.onno@crf.canon.fr cohen@merl.com vseregin@qti.qualcomm.com xiaoyu.xiu@interdigital.com Zhan.Ma@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the Core Experiment 6 (CE6) on improvements of palette mode. Out of the 15 planned tests, two tests were withdrawn, and the rest tests with some additional tests were conducted. Evidences were provided that palette mode can be further improved with affordable complexity.

# Introduction

In the 18th JCT-VC meeting in Sapporo, a simplified combined palette mode in JCTVC-R0348 [1] was first adopted into SCM-2.0 software and draft text, and then the CU-level escape color flag in JCTVC-R0066 [2] with a simplification in JCTVC-R0051 [3] was adopted. Then, Core Experiment 6 (CE6) [4] was formed to further investigate improvements of the existing palette mode in SCM-2.0. This contribution describes test conditions, test results, and comparisons.

# Test conditions

All tests were conducted under SCM-2.0 common test condition [5], which enabled full frame intra block copy (FF-IBC) as anchor.

# Test descriptions and results

## Category A – Colour index run coding

### Test A.1 – Restricted run coding

- Proponent: Nokia, JCTVC-R0045

- Crosschecker: Qualcomm

- Description:

This test limits the amount of palette_run syntax element decoded for the index mode of the current palette implementation. When the colour index is larger than a threshold, the palette_run is not signalled. The issue of the parsing dependency noted for this contribution will be investigated.

- Results:

Withdrawn.

### Test A.2 – Run coding for two-colour palette

- Proponent: ITRI, JCTVC-R0076

- Crosschecker: Sharp

- Description:

This test evaluates modifications in the two-colour case of palette mode. The first modification skips coding colour index in “copy left run” mode. Encoder and other related modifications in the two-colour case of palette mode will be also evaluated.

- Results:

Withdrawn.

### Test A.3 – Restricted run coding

- Proponent: Canon, JCTVC-R0085, JCTVC-S0062

- Crosschecker: Qualcomm, JCTVC-S0034

- Description:

This test limits the amount of palette_run syntax element decoded for the index mode of the current palette implementation. When the colour index is larger than a threshold, the palette_run is not signalled. The issue of the parsing dependency noted for this contribution will be investigated.

- Results:

Note: Parsing depends on reconstructed palette index values.

### Test A.4 – Binarization and context modeling for run coding

- Proponent: MediaTek, JCTVC-R0136, JCTVC-S0163

- Crosschecker: Qualcomm, JCTVC-S0035

- Description:

In this test, the syntax element, palette_run, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The context selection is conditioned on the palette_type_flag, bin index, and palette index. Both decoded palette index and reconstructed palette index values will be tested for context selection. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy coded in the bypass mode.

- Results:

[Test A.4.1]

Note: Parsing depends on reconstructed palette index values.

[Test A.4.2]

Note: Parsing is independent of reconstructed palette index values.

### Test A.5 – Contexts for run coding

- Proponent: Qualcomm, JCTVC-R0174 part 1, JCTVC-S0038

- Crosschecker: Canon, JCTVC-S0071

- Description:

In this test, use of additional contexts depending on the index for coding index runs is tested. Additional contexts for greater than 0, greater than 1 and greater than 2 flags will be tested. The context is dependent on the index value (decoded) or the coded index (parsed). Additionally choice of context based on index value and the palette size will be tested.

- Results:

[Test A.5.1]

Note: Parsing depends on reconstructed palette index values.

Note: Run time percentages are inaccurate.

[Test A.5.2]

Note: Parsing is independent of reconstructed palette index values.

Note: Run time percentages are inaccurate.

### Test A.6 – Contexts for run coding

- Proponent: Qualcomm, JCTVC-R0174 part 2, JCTVC-S0039

- Crosschecker: Canon, JCTVC-S0072

- Description:

In this test, exponential Golomb and Golomb Rice binarizations and their truncated versions for the palette run values will be tested. Combination of tests A.5 and A.6 will also be tested.

- Results:

Note: Run time percentages are inaccurate.

- Additional results:

Note: Test A.5.1 and Test A.6 are combined.

Note: Parsing depends on reconstructed palette index values.

Note: Run time percentages are inaccurate.

## Category B – Colour index and escape colour coding

### Test B.1 – Context coded CU-level escape colour flag

- Proponent: Qualcomm, JCTVC-R0066, JCTVC-S0154

- Crosschecker: InterDigital, JCTVC-S0195

- Description:

In this test, use of regular CABAC bin to code the CU level escape flag will be tested. Context determination methods dependent on palette size and block size will be tested.

- Results:

[Test B.1.1]

Note: Two additional contexts are used.

Note: Run time percentages are inaccurate.

[Test B.1.2]

Note: One additional context is used.

Note: Run time percentages are inaccurate.

### Test B.2 – Encoder modification related to CU-based escape colour flag

- Proponent: ITRI, JCTVC-R0075, JCTVC-S0048

- Crosschecker: InterDigital, JCTVC-S0128

- Description:

If the number of the pixels quantized to a major colour is smaller than or equal to a pre-assigned threshold, the pixels are changed to escape colour pixels. Then the corresponding major colour is removed. An adaptive threshold method will also be tested.

- Results:

Note: The proposed method is non-normative and encoder-only.

Note: In addition to the the original size of major colour table (M), the proposed method searches M-1, M-2, and M-3 using rate-distortion optimization (RDO).

### Test B.3 – Colour index coding with contextualization

- Proponent: University of Hanover, JCTVC-R0113, JCTVC-S0074

- Crosschecker: MediaTek, JCTVC-S0168

- Description:

In this test, a modified colour index signalling is evaluated. In particular, the most significant bin of the palette_index syntax element is coded with a context instead of bypass.

- Results:

### Test B.4 – Binarization and context modeling for index coding

- Proponent: MediaTek, JCTVC-R0135, JCTVC-S0164

- Crosschecker: Fujitsu, JCTVC-S0215

- Description:

In this test, the syntax element, palette_index, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the truncated binary code and the resulting bin string is entropy coded in the bypass mode.

- Results:

### Test B.5 – Escape colour prediction

- Proponent: InterDigital, JCTVC-R0170, JCTVC-S0132

- Crosschecker: ITRI, JCTVC-S0211

- Description:

In this test, an escape colour prediction method will be tested. A flag is signalled to indicate if all colour components of the current escape colour pixel are the same as that of the previous escape colour pixel. If not, the current escape colour value is predicted from an entry in the palette table. The index value of the table entry used for prediction and the prediction difference are coded.

- Results:

Note: In addition to entries in the palette table, previous coded escape colours can also be used for predicting the current escape colour.

## Category C – Additional colour index representation modes

### Test C.1 – Transition copy mode

- Proponent: Canon, JCTVC-R0084, JCTVC-S0063

- Crosschecker: MediaTek, JCTVC-S0205

- Description:

In this test, an index prediction method based on transitions will be evaluated. The following index value for the last occurrence of a given index value is stored and used as the inferred index value in the new prediction mode.

- Results:

Note: Parsing depends on reconstructed palette index values.

Note: An encoder-only parameter change is applied (without changing the RDO algorithm flow) to select index, copy-above, or transition copy.

### Test C.2 – Transition copy mode

- Proponent: MediaTek, JCTVC-R0120, JCTVC-S0078

- Crosschecker: Canon, JCTVC-S0073

- Description:

In this test, a new index coding mode, transition copy (TC) run mode will be tested. In the TC run mode, the coded adjacent colour index patterns are recorded in the TC table. The TC table can be inherited across CUs. The recoded colour indices are used as the colour index predictors in the TC run mode.

- Results:

Note: An encoder-only parameter change is applied (without changing the RDO algorithm flow) to select index, copy-above, or transition copy.

### Test C.3 – Copy-from-previous-row mode

- Proponent: Qualcomm, JCTVC-R0202, JCTVC-S0174

- Crosschecker: MediaTek, JCTVC-S0203

- Description:

In this test, a mode of “copy from previous rows” for the palette coding will be tested. This method enables copying pixels from previously coded rows beyond the row above in the current CU. This is defined as an additional palette mode besides the existing “copy from left”, “copy above” and “escape” modes. The reference row index is coded in the bitstream to indicate which row is copied from. Restrictions on the number of rows available for ‘copy from previous rows” will be tested.

- Results:

Note: An encoder-only RDO algorithm change is applied to estimate the run bits more precisely.

## Category D – Palette table coding

### Test D.1 – Palette table coding

- Proponent: Qualcomm, JCTVC-R0228, JCTVC-S0153

- Crosschecker: MediaTek, JCTVC-S0204

- Description:

In this test, a method that uses run-length coding to signal the binary vector predictor for palette will be tested, where the run value, indicating the number of zero elements between the ones, is signalled as described in JCTVC-R0228. Additionally, sending the number of non-zero elements explicitly in the beginning will be tested. Run values are coded using Exponential Golomb coded or Golomb Rice codes or their truncated versions. Additionally, increased palette predictor size will be tested with the run-length coding.

- Results:

- Additional results:

[Palette predictor length = 128]

Left: palette predictor length increased from 64 to 128

Right: Combination of palette predictor length increased from 64 to 128 and Test D.1

Note:

# Comparisons

To compare different tests, for each coding structures (AI, RA, and LB), the average BD-rate of all sequences from the first four RGB or YUV classes is calculated color component-wise, and the obtained average BD-rates of the last two color components are further averaged. The reason of skipping the last two RGB or YUV classes is that all the tests show close-to-zero BD-rates for these classes.

## Comparison of category A (colour index run coding) proposals

- BD-rate savings of category A tests

## Comparison of category B (colour index and escape colour coding) proposals

- BD-rate savings of category B tests

## Comparison of category C (additional colour index representation modes) proposals

- BD-rate savings of category C tests

## Category D (palette table coding) proposal

- BD-rate savings of category D test

Note: P128 represents increasing palette predictor size from 64 to 128.

# Conclusions

Quite a few tests were conducted in CE6. Evidences were provided that palette mode can be further improved with affordable complexity. Combining promising tests in CE6 may bring around 4% BD-rate savings in all intra condition and around 2% BD-rate savings in random access and low delay B conditions for test video classes of text and graphics with motion, assuming most of the gains are additive. It is suggested to discuss and test the combinations in the Strasbourg meeting.

# List of CE6-related non-CE documents

JCTVC-S0043 [6]

JCTVC-S0047 [7]

JCTVC-S0051 [8] (Although the title says “Non-CE8 …,” it is more CE6-related than CE8-related.)

JCTVC-S0052 [9]

JCTVC-S0053 [10]

JCTVC-S0054 [11]

JCTVC-S0055 [12]

JCTVC-S0060 [13]

JCTVC-S0064 [14]

JCTVC-S0066 [15]

JCTVC-S0079 [16]

JCTVC-S0099 [17]

JCTVC-S0105 [18]

JCTVC-S0108 [19]

JCTVC-S0110 [20]

JCTVC-S0111 [21]

JCTVC-S0114 [22]

JCTVC-S0115 [23]

JCTVC-S0116 [24]

JCTVC-S0120 [25]

JCTVC-S0134 [26]

JCTVC-S0138 [27]

JCTVC-S0150 [28]

JCTVC-S0151 [29]

JCTVC-S0152 [30]

JCTVC-S0155 [31]

JCTVC-S0156 [32]

JCTVC-S0157 [33]

JCTVC-S0169 [34] (crosscheck of JCTVC-S0052)

JCTVC-S0170 [35] (Crosscheck of JCTVC-S0053)

JCTVC-S0173 [36]

JCTVC-S0178 [37]

JCTVC-S0181 [38]

JCTVC-S0186 [39]

JCTVC-S0188 [40]

JCTVC-S0200 [41] (Crosscheck of JCTVC-S0173)

JCTVC-S0209 [42] (Crosscheck of JCTVC-S0051)

JCTVC-S0213 [43] (Crosscheck of JCTVC-S0134)

JCTVC-S0214 [44] (Crosscheck of JCTVC-S0055)

JCTVC-S0217 [45] (Crosscheck of JCTVC-S0110)

JCTVC-S0223 [46] (Crosscheck of JCTVC-S0064)

JCTVC-S0228 [47] (Crosscheck of JCTVC-S0111)

JCTVC-S0229 [48] (Crosscheck of JCTVC-S0138)

JCTVC-S0232 [49] (Crosscheck of JCTVC-S0150)

JCTVC-S0246 [50] (Crosscheck of JCTVC-S0054)

JCTVC-S0247 [51] (Crosscheck of JCTVC-S0060)

JCTVC-S0253 [52] (Crosscheck of JCTVC-S0120)

JCTVC-S0255 [53] (Crosscheck of JCTVC-S0114)

JCTVC-S0256 [54] (Crosscheck of JCTVC-S0181)

JCTVC-S0258 [55]

JCTVC-S0259 [56] (Crosscheck of JCTVC-S0157)

JCTVC-S0260 [57] (Crosscheck of JCTVC-S0047)

JCTVC-S0266 [58] (Crosscheck of JCTVC-S0108)

JCTVC-S0267 [59] (Crosscheck of JCTVC-S0064)

JCTVC-S0269 [60]

JCTVC-S0270 [61] (Crosscheck of JCTVC-S0115)

JCTVC-S0275 [62] (Crosscheck of JCTVC-S0178)

JCTVC-S0276 [63] (Crosscheck of JCTVC-S0186)

JCTVC-S0277 [64] (Crosscheck of JCTVC-S0258)

JCTVC-S0278 [65] (Crosscheck of JCTVC-S0066)

JCTVC-S0279 [66] (Crosscheck of JCTVC-S0115)

JCTVC-S0280 [67] (Crosscheck of JCTVC-S0105)

JCTVC-S0282 [68] (Crosscheck of JCTVC-S0067)

# References

- P. Onno, X. Xiu, Y.-W. Huang, R. Joshi, “Suggested combined software and text for run-based palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0348, 18th Meeting: Sapporo, JP, June-July 2014.

- W. Pu, R. Joshi, M. Karczewicz, J. Sole, V. Seregin, “SCCE3: Test B.13 – Flag to indicate escape and index coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0066, 18th Meeting: Sapporo, JP, June-July 2014.

- Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung, “Non-SCCE3: Adaptive escape scheme for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0051, 18th Meeting: Sapporo, JP, June-July 2014.

- Y.-W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z, Ma “Description of Core Experiment 6 (CE6): Improvements of palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1016, 18th Meeting: Sapporo, JP, June-July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1015, 18th Meeting: Sapporo, JP, June-July 2014.

- J. Sole, W. Pu, R. Joshi, V. Seregin, M. Karczewicz, “Non-CE6: Delta QP signalling for palette,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0043, 19th Meeting: Strasbourg, FR, October 2014.

- J.-S. Tu, C.-L. Lin, C.-H. Hung, C.-C. Lin, Y.-J. Chang, “Non-CE6: Modifications of copy-left and copy-above modes in index coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0047, 19th Meeting: Strasbourg, FR, October 2014.

- Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung, “Non-CE8: Multi-colour intra mode for screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0051, 19th Meeting: Strasbourg, FR, October 2014.

- J. Ye, J. Zhu, “Non-CE6: Escape coded pixel prediction using previous escape coded pixels for palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0052, 19th Meeting: Strasbourg, FR, October 2014.

- J. Ye, J. Zhu, “Non-CE6: Escape pixel prediction using previous escape coded pixels and palette for palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0053, 19th Meeting: Strasbourg, FR, October 2014.

- J. Ye, J. Zhu, “Non-CE6: Escape coded pixel prediction using previous palette for palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0054, 19th Meeting: Strasbourg, FR, October 2014.

- C. -C Lin, C.-L. Lin, Y.-J. Chang, J.-S. Tu, C.-H. Hung, “Non-CE6: Escape colour signalling,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0055, 19th Meeting: Strasbourg, FR, October 2014.

- Z. Wang, J. Zhu, “Non-CE6: Modified unpredicted palette entries coding in palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0060, 19th Meeting: Strasbourg, FR, October 2014.

- G. Laroche, T. Poirier, C. Gisquet, P. Onno, “Non-CE6: Last run flag for Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0064, 19th Meeting: Strasbourg, FR, October 2014.

- C. Gisquet, G. Laroche, P. Onno, “Non-CE6: Palette encoder improvements for SCM2.0,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0066, 19th Meeting: Strasbourg, FR, October 2014.

- Y.-C. Sun, J. Kim, T.-D. Chuang, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei, “Non-CE6: Cross-CU palette colour index prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0079, 19th Meeting: Strasbourg, FR, October 2014.

- P. Lai, J. Kim, S. Liu, S. Lei, “Non-CE6: Syntax redundancy removal for palette mode with one index value,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0099, 19th Meeting: Strasbourg, FR, October 2014.

- K. Zhang, T.-D. Chuang, S. Liu, J. An, X. Zhang, S. Lei, “CE6-related: Syntax fixes for zero palette in palette coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0105, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, M. Karczewicz, R. Joshi, F. Zou, V. Seregin, J. Sole, “Non-CE6: Improvement on palette sharing mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0108, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, M. Karczewicz, V. Seregin, F. Zou, R. Joshi, J. Sole, “Non-CE6: Syntax redundancy fixes for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0110, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin, J. Sole, “Non-CE6: Improvement on palette run coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0111, 19th Meeting: Strasbourg, FR, October 2014.

- J. Kim, Y.-C. Sun, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE6-related : Enabling copy above mode prediction at the boundary of CU,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0114, 19th Meeting: Strasbourg, FR, October 2014.

- J. Kim, S. Liu, T.-D. Chuang, Y.-W Huang, S. Lei, “CE6-related: Clarifying decoder’s ambiguous behaviour for escape index in palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0115, 19th Meeting: Strasbourg, FR, October 2014.

- J. Kim, S. Liu, S. Lei, “CE6-related: Signalling palette size in palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0116, 19th Meeting: Strasbourg, FR, October 2014.

- J. Ye, J. Kim, S. Liu, P. Lai, S. Lei, “Non-CE6: Copy previous mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0120, 19th Meeting: Strasbourg, FR, October 2014.

- J. Zhao, S. H. Kim, K. Misra, A. Segall, “Non-CE6: Simplified palette size coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0134, 19th Meeting: Strasbourg, FR, October 2014.

- S. H. Kim, K. Misra, J. Zhao, A. Segall, “	Non-CE6: Exponential Golomb binarization for palette run,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0138, 19th Meeting: Strasbourg, FR, October 2014.

- M. Xu, Z. Ma, W. Wang, H. Yu, “	Non-CE6: Simplification on escape coding of palette mode in HEVC SCC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0150, 19th Meeting: Strasbourg, FR, October 2014.

- W. Wang, Z. Ma, M. Xu, H. Yu, “Non-CE6: 2-D index map coding of palette mode in HEVC SCC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0151, 19th Meeting: Strasbourg, FR, October 2014.

- W. Wang, Z. Ma, M. Xu, H. Yu, “Non-CE6: Index coding group (ICG) for 8x8 CU of palette mode in HEVC SCC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0152, 19th Meeting: Strasbourg, FR, October 2014.

- V. Seregin, M. Karczewicz, W. Pu, R. Joshi, J. Sole, “Non-CE6: Palette copy above mode for the first row,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0155, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, F. Zou, V. Seregin, R. Joshi, M. Karczewicz, J. Sole, “Non-CE6: Palette parsing dependency and palette encoder improvement,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0156, 19th Meeting: Strasbourg, FR, October 2014.

- V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole, “Non-CE6: Copy mode and escape coded sample,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0157, 19th Meeting: Strasbourg, FR, October 2014.

- S.-T. Hsiang, “Non-CE6: Cross check of JCTVC-S0052 on escape coded pixel prediction using previous escape coded pixels for palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0169, 19th Meeting: Strasbourg, FR, October 2014.

- S.-T. Hsiang, “Non-CE6: Cross check of JCTVC-S0053 on escape pixel prediction using previous escape coded pixels and palette for palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0170, 19th Meeting: Strasbourg, FR, October 2014.

- Y. He, X. Xiu, Y. Ye, “Non-CE6: Redundancy removal and simplification for palette coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0173, 19th Meeting: Strasbourg, FR, October 2014.

- M. Karczewicz, F. Zou, R. Joshi, V. Seregin, “Non-CE6: Improved binarization and signaling of index coding for transition copy mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0178, 19th Meeting: Strasbourg, FR, October 2014.

- X. Xiu, Y. He, Y. Ye, “Non-CE6: Removal of parsing dependency in palette-based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0181, 19th Meeting: Strasbourg, FR, October 2014.

- X. Xiu, Y. He, Y. Ye, “Non-CE6: On context modeling of palette_transpose_flag,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0186, 19th Meeting: Strasbourg, FR, October 2014.

- Y.-C. Sun, T.-D. Chuang, Y.-W. Huang, S. Lei, “Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) - copy-from-previous-row mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0188, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, “Cross check Non-CE6: Redundancy removal and simplification for palette coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0200, 19th Meeting: Strasbourg, FR, October 2014.

- Y.-W. Chen, “CE8-related: Crosscheck for Multi-colour intra mode for screen content coding (JCTVC-S0051),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0209, 19th Meeting: Strasbourg, FR, October 2014.

- J. Kim, “CE6-related: Crosscheck report of JCTVC-S0134,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0213, 19th Meeting: Strasbourg, FR, October 2014.

- W. Pu, “Cross check non-CE6: Escape colour signalling,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0214, 19th Meeting: Strasbourg, FR, October 2014.

- Y. He, X. Xiu, Y. Ye, “Non-CE6: Cross-check of JCTVC-S0110 on syntax redundancy fixes for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0217, 19th Meeting: Strasbourg, FR, October 2014.

- P. Wu, “Cross check non-CE6: last run flag for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0223, 19th Meeting: Strasbourg, FR, October 2014.

- C.-C. Lin, “Crosscheck of non-CE6 on improvement on palette run coding (JCTVC-S0111),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0228, 19th Meeting: Strasbourg, FR, October 2014.

- C.-C. Lin, “Crosscheck of non-CE6 on exponential Golomb binarization for palette run (JCTVC-S0138),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0229, 19th Meeting: Strasbourg, FR, October 2014.

- R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang, “Crosscheck of JCTVC-S0150: non-CE6: simplification on escape coding of palette mode in HEVC SCC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0232, 19th Meeting: Strasbourg, FR, October 2014.

- M. Xu, W. Wang, Z. Ma, H. Yu, “Non-CE6: Cross-check of S0054 (Escape coded pixel prediction using previous palette for palette based coding),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0246, 19th Meeting: Strasbourg, FR, October 2014.

- M. Xu, W. Wang, Z. Ma, H. Yu, “Non-CE6: Cross-check of S0060 (Modified unpredicted palette entries coding in palette based coding),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0247, 19th Meeting: Strasbourg, FR, October 2014.

- W. Zhang, L. Xu, Y. Chiu, “Non-CE6: Crosscheck for Copy Previous Mode (JCTVC-S0120),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0253, 19th Meeting: Strasbourg, FR, October 2014.

- V. Seregin, “Non-CE6: Cross-check of enabling copy above mode prediction at the boundary of CU (JCTVC-S0114),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0255, 19th Meeting: Strasbourg, FR, October 2014.

- V. Seregin, “Non-CE6: Cross-check of removal of parsing dependency in palette-based coding (JCTVC-S0181),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0256, 19th Meeting: Strasbourg, FR, October 2014.

- X. Xiu, Y. He, Y. Ye, V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole, “Non-CE6: Unification of coding of escape indices and other palette indices,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0258, 19th Meeting: Strasbourg, FR, October 2014.

- J. Kim, S. Liu, “CE6-related: Crosscheck report of JCTVC-S0157,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0259, 19th Meeting: Strasbourg, FR, October 2014.

- S.H. Kim, A. Segall, “Cross-check of Non-CE6: Modifications of copy-left and copy-above modes in index coding (JCTVC-S0047),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0260, 19th Meeting: Strasbourg, FR, October 2014.

- B. Li, J. Xu, “Non-CE6: cross-check of improvement on palette sharing mode (JCTVC-S0108),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0266, 19th Meeting: Strasbourg, FR, October 2014.

- X. Xiu, Y. He, Y. Ye, “Cross-verification of JCTVC-S0064 on Last run flag for Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0267, 19th Meeting: Strasbourg, FR, October 2014.

- S.-T. Hsiang, T.-D. Chuang, S. Lei, R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, J. Sole, “CE6-related: Harmonization of CE6 Tests A.4, A.5, and A.6,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0269, 19th Meeting: Strasbourg, FR, October 2014.

- J. Zhao, S. H Kim, “CE6-related: Cross check of JCTVC-S0115 Test 2.1,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0270, 19th Meeting: Strasbourg, FR, October 2014.

- P. Onno, “Non-CE6: Cross-check of JCTVC-S0178 on improved binarization and signaling of index coding for transition copy mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0275, 19th Meeting: Strasbourg, FR, October 2014.

- P. Onno, “Non-CE6: Cross-check of JCVC-S0186 on context modeling of palette_transpose_flag,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0276, 19th Meeting: Strasbourg, FR, October 2014.

- P. Onno, “Non-CE6: Crosscheck of JCTVC-S0258 on unification of coding of escape indices and other palette indices,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0277, 19th Meeting: Strasbourg, FR, October 2014.

- P. Wu, “Cross check Non-CE6: Palette encoder improvements for SCM2.0,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0278, 19th Meeting: Strasbourg, FR, October 2014.

- F. Zhou, “Crosscheck of JCTVC-S0115 method 2,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0279, 19th Meeting: Strasbourg, FR, October 2014.

- F. Zhou, “Crosscheck of JCTVC-S0105,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0280, 19th Meeting: Strasbourg, FR, October 2014.

- F. Zhou, “Crosscheck of JCTVC-S0067,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-S0282, 19th Meeting: Strasbourg, FR, October 2014.