| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0027 |
| --- | --- |

| Title: | CE7: Summary report for Core Experiment 7 on String Matching for Palette Index Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document To JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhan Ma Y.-W. Huang | zhan.ma@outlook.com yuwen.huang@mediatek.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the summary of Core Experiment 7 (CE7) on String Matching for Palette Index Coding. CE7 includes three tests:

- Test 1 on 1-D string-based index coding;

- Test 2 on constrained 1-D string-based index coding;

- Test 3 on hybrid 1-D/2-D string-based index coding

# Introduction

At the 18th JCT-VC meeting, it was shown in JCTVC-R0268 [1] andJCTVC-R0304 Error: Reference source not found that improvements in coding performance for screen content coding could be obtained by using string-based index coding, either 1-D, constrained 1-D or hybrid 1-D/2-D scheme. Core Experiment 7 on string-based index coding was defined Error: Reference source not found to study these proposed techniques. This report summarizes and compares the results of the tests performed under this core experiment. Combined summary tables are included in Section 8.

# Core experiment testing conditions

The proposed modifications were incorporated on top of SCM-2.0. The common conditions for screen content coding tests from JCTVC-R1015 Error: Reference source not found were used for these experiments. In addition, SCM-2.0 common test condition with intra block copy using 4-CTU search (i.e., current CTU and left 3 CTUs) is also evaluated as the anchor for second test point.

# Test conditions

The common conditions for screen content coding tests from JCTVC-R1015 shall be used for these experiments.

# Description of tests

This core experiment tested three string-based index coding techniques: 1-D, constrained 1-D and hybrid 1-D/2-D schemes.

## Test 1 – 1-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0268 using 1D string match based index compression. 1-D string match is performed within current CU with the representation of matched pairs..

## Constrained 1-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0304 using constrained 1D string match based index compression. This constrained 1-D string match is performed within current CU with constrained matched distance and length is inferred as the CU width. More details could be found JCTVC-R0304

## Hybrid 1-D/2-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0268/R304 using hybrid 1D/2D string match based index compression. 1-D string match is performed within current CU, while hybrid 1D/2D search can be extended to the left 3 CTUs. More details could be found JCTVC-R0268/R0304

# CE7 document summary

| Test | Mode | Current Proposal | Crosscheck(s) | Tested Proposal(s) |
| --- | --- | --- | --- | --- |
| 1 | 1-D | JCTVC-S0158 | JCTVC-S0216 | JCTVC-R0268 |
| 2 | Constrained 1-D | JCTVC-S0159 | JCTVC-S0091 | JCTVC-R0304 |
| 3 | Hybrid 1-D/2-D | JCTVC-S0160 | JCTVC-S0130 | JCTVC-R0304/R0268 |

Current proposals:

| JCTVC-S0158 | m34786 | CE 7 Test 1: 1-D String based Index Coding for Palette Index Coding | M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D) |
| --- | --- | --- | --- |
| JCTVC-S0159 | m34787 | CE 7 Test 2: Constrained 1-D String based Index Coding for Palette Index Coding | M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D) |
| JCTVC-S0160 | m34788 | CE 7 Test 3: Hybrid 1-D/2-D String-based Index Coding for Palette Index Coding | M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D) |

Current crosschecks:

| JCTVC-S0216 | m34892 | CE7: crosscheck report of CE7 test 1 | Zhanglei Xu, Jianqing Zhu(Fujitsu) |
| --- | --- | --- | --- |
| JCTVC-S0091 | m34713 | CE7: Cross-check of test 2 (JCTVC-S0159) | B. Li, J. Xu (Microsoft) |
| JCTVC-S0130 | m34755 | CE7: Cross check results for Test 3( JCTVC-S0160) | J. Ye, S. Liu (MediaTek) |

# Complexity analysis of palette modes

The above 3 tests do not require any off-chip memory bandwidth. Test 2 requires 38.75B additional on-chip memory; Test 3 requires 36KB additional on-chip memory. Worst context coded bins per 3-component pixel is 13.296875, and No parsing dependency issue is found.

# Test Results

This section includes separate BD-Rate summary tables for each CE7 test. Encoder or decoder run-time ratios that may not be accurate due to variations in computing platforms are so noted.

## Test 1 – 1-D string-based index coding

Lossy test conditions (anchor is full frame IBC):

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -1.0% | -1.3% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.8% | -2.4% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 123% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.9% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.6% |
| YUV, text & graphics with motion,720p | -0.4% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.7% | -0.3% |
| YUV, mixed content, 1080p | -0.6% | -2.6% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.2% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.6% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.5% | -0.9% |
| YUV, mixed content, 1440p | -0.3% | -0.7% | -0.4% |
| YUV, mixed content, 1080p | -1.0% | -3.5% | -3.0% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 134% |  |  |

Lossless test conditions (anchor is full frame IBC):

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.3% | 0.9% |
| RGB, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.1% | 0.9% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 122% |  |  |  |

Lossy test conditions (anchor is 1x4 IBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.6% | -0.7% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -1.1% | -1.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.4% |
| YUV, mixed content, 1080p | -0.3% | -1.7% | -1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 127% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.6% | -0.8% | -1.0% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -1.7% | -1.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.6% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.3% |
| RGB, mixed content, 1080p | -0.9% | -1.1% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.9% | -0.9% |
| YUV, text & graphics with motion,720p | -0.4% | -0.9% | -1.1% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | -0.4% | -1.9% | -1.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 134% |  |  |

Lossless test conditions (anchor is 1x4 IBC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.9% | 0.6% | 1.1% |
| RGB, text & graphics with motion,720p | 0.4% | 0.5% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.7% | 0.8% | 0.2% | 1.1% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.2% | 0.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.3% | 0.1% | 0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 122% |  |  |  |

## Test 2 – Constrained 1-D String-based index coding

Lossy test conditions (full frame IBC):

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.6% | -1.6% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.6% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -0.6% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.1% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -1.0% | -0.9% |
| YUV, text & graphics with motion,720p | -0.7% | -1.0% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.4% |
| YUV, mixed content, 1080p | 0.0% | -0.8% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% |
| RGB, text & graphics with motion,720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.8% | -0.7% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -1.0% |
| YUV, mixed content, 1440p | -0.4% | -0.8% | -0.7% |
| YUV, mixed content, 1080p | -0.4% | -1.1% | -1.3% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 133% |  |  |

Lossless test conditions (full frame IBC):

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.0% | 0.4% | 1.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.1% | 0.2% | 1.6% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.2% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.1% | 1.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 120% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 123% |  |  |  |

Lossy test conditions (1x4 IBC):

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.5% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -1.6% | -1.7% | -1.6% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.4% | -2.5% | -2.5% |
| YUV, text & graphics with motion,720p | -1.2% | -1.4% | -1.5% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 129% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.1% | -1.1% |
| RGB, text & graphics with motion,720p | -1.6% | -1.6% | -1.5% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -1.1% | -1.2% | -1.4% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.6% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.3% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.3% | -1.0% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 134% |  |  |

Lossless test conditions (1x4 IBC):

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.7% | 0.6% | 2.6% |
| RGB, text & graphics with motion,720p | 0.5% | 0.8% | 0.1% | 2.4% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.6% | 1.8% | 0.3% | 3.1% |
| YUV, text & graphics with motion,720p | 0.5% | 0.9% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |  |
| Dec Time[%] | 120% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.9% | 0.3% | 1.8% |
| RGB, text & graphics with motion,720p | 0.1% | 0.5% | 0.0% | 1.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.6% | 0.0% | 2.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 117% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 1.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 121% |  |  |  |

## Test 2 – Hybrid 1-D/2-D String-based index coding

Lossy test conditions (full frame IBC):

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.3% | -2.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.5% | -1.5% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -1.2% | -1.5% | -1.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.7% | -2.2% | -2.1% |
| YUV, text & graphics with motion,720p | -0.8% | -1.3% | -1.2% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -1.1% | -3.0% | -2.5% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 124% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.5% | -1.5% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.9% | -1.3% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -0.8% | -1.3% | -1.3% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.8% | -3.1% | -2.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.4% | -1.3% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -1.1% | -0.9% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.5% | -1.5% |
| YUV, text & graphics with motion,720p | -0.7% | -0.8% | -1.3% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.3% |
| YUV, mixed content, 1080p | -1.1% | -3.1% | -2.6% |
| YUV, Animation, 720p | 0.0% | -0.5% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 133% |  |  |

Lossless test conditions (full frame IBC):

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.1% | 1.3% | 0.3% | 1.9% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 1.3% | 0.1% | 2.0% |
| YUV, text & graphics with motion,720p | 0.3% | 0.5% | 0.1% | 1.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 125% |  |  |  |
| Dec Time[%] | 115% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.7% | 0.1% | 1.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.8% | 0.1% | 1.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.1% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.5% | 0.0% | 0.9% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 122% |  |  |  |

Lossy test conditions (1x4 IBC):

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.4% | -4.2% | -4.0% |
| RGB, text & graphics with motion,720p | -3.2% | -3.9% | -3.7% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.9% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.9% | -3.7% | -3.7% |
| YUV, text & graphics with motion,720p | -2.1% | -2.9% | -2.9% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.6% |
| YUV, mixed content, 1080p | -0.6% | -2.7% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 122% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.0% | -2.7% | -2.5% |
| RGB, text & graphics with motion,720p | -3.0% | -3.6% | -3.5% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.7% | -0.9% | -0.8% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.9% | -2.7% | -2.7% |
| YUV, text & graphics with motion,720p | -2.1% | -2.8% | -3.2% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | -0.5% | -2.6% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.5% | -2.0% | -1.8% |
| RGB, text & graphics with motion,720p | -1.3% | -1.7% | -1.6% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -0.6% | -0.9% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.4% | -2.2% | -2.2% |
| YUV, text & graphics with motion,720p | -0.8% | -1.6% | -1.7% |
| YUV, mixed content, 1440p | -0.4% | -0.9% | -0.8% |
| YUV, mixed content, 1080p | -0.6% | -2.6% | -1.6% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 133% |  |  |

Lossless test conditions (1x4 IBC):

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.5% | 2.7% | 0.6% | 4.7% |
| RGB, text & graphics with motion,720p | 0.7% | 1.6% | 0.1% | 5.5% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.7% | 2.9% | 0.2% | 5.8% |
| YUV, text & graphics with motion,720p | 0.8% | 2.0% | 0.1% | 7.2% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 1.5% | 0.2% | 3.3% |
| RGB, text & graphics with motion,720p | 0.1% | 1.1% | 0.0% | 4.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 1.8% | 0.1% | 4.0% |
| YUV, text & graphics with motion,720p | 0.1% | 1.4% | 0.0% | 5.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 1.2% | 0.1% | 2.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 121% |  |  |  |

# Combined summary tables

This section contains combined BD-Rate summary tables.

To reduce table size, the following notations for sequence types are used:

| R1 | RGB, text & graphics with motion, 1080p |
| --- | --- |
| R2 | RGB, text & graphics with motion,720p |
| R3 | RGB, mixed content, 1440p |
| R4 | RGB, mixed content, 1080p |
| R5 | RGB, Animation, 720p |
| R6 | RGB, camera captured, 1080p |
| Y1 | YUV, text & graphics with motion, 1080p |
| Y2 | YUV, text & graphics with motion,720p |
| Y3 | YUV, mixed content, 1440p |
| Y4 | YUV, mixed content, 1080p |
| Y5 | YUV, Animation, 720p |
| Y6 | YUV, camera captured, 1080p |

## Lossy test conditions

### All-intra

|  | Test 1 FF IBC |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -0.6% | -0.7% | -0.7% |  | -1.0% | -1.2% | -1.2% |
| R2 | -0.7% | -0.7% | -0.7% |  | -0.9% | -1.0% | -1.0% |
| R3 | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.2% | -0.2% |
| R4 | -1.0% | -1.3% | -1.1% |  | -0.6% | -0.7% | -0.5% |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Y1 | -0.6% | -0.8% | -0.8% |  | -0.8% | -1.1% | -1.1% |
| Y2 | -0.2% | -0.4% | -0.4% |  | -0.5% | -0.8% | -0.9% |
| Y3 | 0.0% | -0.4% | -0.3% |  | -0.1% | -0.5% | -0.4% |
| Y4 | -0.8% | -2.4% | -2.0% |  | -0.3% | -1.7% | -1.3% |
| Y5 | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
|  | Test 2: FF IBC |  |  |  | Test 2: 1x4 IBC |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -1.6% | -1.6% | -1.6% |  | -2.5% | -2.6% | -2.5% |
| R2 | -1.0% | -1.0% | -1.0% |  | -1.6% | -1.7% | -1.6% |
| R3 | -0.2% | -0.1% | -0.2% |  | -0.1% | -0.1% | -0.2% |
| R4 | -0.2% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.2% |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Y1 | -1.6% | -1.7% | -1.7% |  | -2.4% | -2.5% | -2.5% |
| Y2 | -0.6% | -0.8% | -0.9% |  | -1.2% | -1.4% | -1.5% |
| Y3 | -0.1% | -0.3% | -0.3% |  | -0.1% | -0.3% | -0.2% |
| Y4 | -0.1% | -0.4% | -0.3% |  | -0.2% | -0.4% | -0.4% |
| Y5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |

|  | Test 3: FF IBC |  |  | Test 3: 1x4 IBC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -1.9% | -2.3% | -2.3% | -3.4% | -4.2% | -4.0% |
| R2 | -1.3% | -1.5% | -1.5% | -3.2% | -3.9% | -3.7% |
| R3 | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| R4 | -1.2% | -1.5% | -1.2% | -0.8% | -0.9% | -0.8% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | -1.7% | -2.2% | -2.1% | -2.9% | -3.7% | -3.7% |
| Y2 | -0.8% | -1.3% | -1.2% | -2.1% | -2.9% | -2.9% |
| Y3 | -0.1% | -0.4% | -0.3% | -0.2% | -0.7% | -0.6% |
| Y4 | -1.1% | -3.0% | -2.5% | -0.6% | -2.7% | -2.2% |
| Y5 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

### Random Access

|  | Test 1 FF IBC |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -0.4% | -0.4% | -0.4% |  | -0.6% | -0.7% | -0.6% |
| R2 | -0.7% | -0.7% | -0.7% |  | -0.9% | -1.0% | -1.0% |
| R3 | -0.2% | -0.2% | -0.2% |  | -0.1% | -0.3% | -0.2% |
| R4 | -0.8% | -0.9% | -1.0% |  | -0.5% | -0.5% | -0.6% |
| R5 | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |
| R6 | 0.1% | 0.0% | 0.0% |  | 0.1% | 0.1% | 0.0% |
| Y1 | -0.3% | -0.6% | -0.6% |  | -0.5% | -0.8% | -0.8% |
| Y2 | -0.4% | -0.7% | -0.7% |  | -0.6% | -0.8% | -1.0% |
| Y3 | -0.1% | -0.7% | -0.3% |  | -0.2% | -0.7% | -0.7% |
| Y4 | -0.6% | -2.6% | -2.0% |  | -0.3% | -1.7% | -1.5% |
| Y5 | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |
| Y6 | 0.1% | 0.2% | 0.0% |  | 0.0% | 0.0% | 0.1% |
|  | Test 2: FF IBC |  |  |  | Test 2: 1x4 IBC |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -0.8% | -0.8% | -0.8% |  | -1.1% | -1.1% | -1.1% |
| R2 | -1.0% | -1.0% | -1.1% |  | -1.6% | -1.6% | -1.5% |
| R3 | -0.3% | -0.3% | -0.3% |  | -0.2% | -0.3% | -0.2% |
| R4 | -0.1% | -0.3% | -0.5% |  | -0.2% | -0.3% | -0.3% |
| R5 | 0.0% | 0.1% | 0.0% |  | 0.0% | -0.1% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.1% | 0.0% |
| Y1 | -0.7% | -1.0% | -0.9% |  | -1.1% | -1.3% | -1.3% |
| Y2 | -0.7% | -1.0% | -0.9% |  | -1.1% | -1.2% | -1.4% |
| Y3 | -0.2% | -0.6% | -0.4% |  | -0.1% | -0.4% | -0.3% |
| Y4 | 0.0% | -0.8% | -0.3% |  | -0.1% | -0.5% | -0.4% |
| Y5 | 0.1% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |
| Y6 | 0.1% | 0.2% | 0.1% |  | 0.0% | 0.0% | 0.2% |

|  | Test 3: FF IBC |  |  | Test 3: 1x4 IBC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -1.2% | -1.5% | -1.5% | -2.0% | -2.7% | -2.5% |
| R2 | -1.3% | -1.4% | -1.4% | -3.0% | -3.6% | -3.5% |
| R3 | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |
| R4 | -0.9% | -1.3% | -1.1% | -0.7% | -0.9% | -0.8% |
| R5 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | -1.2% | -1.7% | -1.7% | -1.9% | -2.7% | -2.7% |
| Y2 | -0.8% | -1.3% | -1.3% | -2.1% | -2.8% | -3.2% |
| Y3 | -0.1% | -0.5% | -0.3% | -0.2% | -0.6% | -0.6% |
| Y4 | -0.8% | -3.1% | -2.5% | -0.5% | -2.6% | -2.2% |
| Y5 | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.2% |

### Low Delay

|  | Test 1 FF IBC |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -0.3% | -0.4% | -0.4% |  | -0.4% | -0.5% | -0.5% |
| R2 | -0.6% | -0.6% | -0.6% |  | -0.6% | -0.8% | -0.8% |
| R3 | -0.2% | -0.4% | -0.2% |  | 0.0% | -0.1% | -0.3% |
| R4 | -0.8% | -0.6% | -0.4% |  | -0.9% | -1.1% | -0.7% |
| R5 | 0.0% | 0.1% | 0.1% |  | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.1% | 0.1% |
| Y1 | -0.3% | -0.6% | -0.5% |  | -0.4% | -0.9% | -0.9% |
| Y2 | -0.3% | -0.5% | -0.9% |  | -0.4% | -0.9% | -1.1% |
| Y3 | -0.3% | -0.7% | -0.4% |  | -0.3% | -0.6% | -0.6% |
| Y4 | -1.0% | -3.5% | -3.0% |  | -0.4% | -1.9% | -1.2% |
| Y5 | 0.1% | -0.1% | 0.2% |  | 0.1% | -0.1% | 0.0% |
| Y6 | 0.0% | -0.1% | -0.1% |  | 0.0% | -0.1% | 0.1% |
|  | Test 2: FF IBC |  |  |  | Test 2: 1x4 IBC |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |
| R1 | -0.6% | -0.7% | -0.6% |  | -0.8% | -0.8% | -0.8% |
| R2 | -0.7% | -0.6% | -0.6% |  | -0.9% | -1.0% | -1.0% |
| R3 | -0.4% | -0.4% | -0.2% |  | -0.3% | -0.3% | -0.3% |
| R4 | -0.1% | 0.1% | 0.2% |  | -0.3% | -0.6% | -0.3% |
| R5 | 0.0% | 0.1% | 0.0% |  | 0.1% | 0.1% | 0.1% |
| R6 | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.1% | 0.1% |
| Y1 | -0.5% | -0.8% | -0.7% |  | -0.6% | -1.0% | -1.0% |
| Y2 | -0.4% | -0.5% | -1.0% |  | -0.3% | -0.4% | -0.5% |
| Y3 | -0.4% | -0.8% | -0.7% |  | -0.2% | -0.4% | -0.3% |
| Y4 | -0.4% | -1.1% | -1.3% |  | -0.3% | -1.0% | -0.3% |
| Y5 | 0.1% | -0.4% | 0.1% |  | 0.1% | -0.3% | 0.0% |
| Y6 | 0.0% | -0.1% | -0.1% |  | 0.0% | -0.1% | 0.0% |

|  | Test 3: FF IBC |  |  | Test 3: 1x4 IBC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -1.0% | -1.4% | -1.3% | -1.5% | -2.0% | -1.8% |
| R2 | -0.9% | -0.8% | -0.9% | -1.3% | -1.7% | -1.6% |
| R3 | -0.2% | -0.3% | -0.1% | -0.3% | -0.4% | -0.5% |
| R4 | -1.1% | -0.9% | -0.2% | -0.6% | -0.9% | -0.4% |
| R5 | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Y1 | -1.0% | -1.5% | -1.5% | -1.4% | -2.2% | -2.2% |
| Y2 | -0.7% | -0.8% | -1.3% | -0.8% | -1.6% | -1.7% |
| Y3 | -0.3% | -0.6% | -0.3% | -0.4% | -0.9% | -0.8% |
| Y4 | -1.1% | -3.1% | -2.6% | -0.6% | -2.6% | -1.6% |
| Y5 | 0.0% | -0.5% | 0.0% | 0.0% | -0.1% | -0.2% |
| Y6 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |

## Lossless test conditions

### All-intra

|  | Test 1 FF IBC |  |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.5% | 0.5% | 0.3% | 0.9% | 0.8% | 0.9% | 0.6% | 1.1% |
| R2 | 0.2% | 0.2% | 0.0% | 0.4% | 0.4% | 0.5% | 0.0% | 1.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.5% | 0.5% | 0.1% | 0.9% | 0.7% | 0.8% | 0.2% | 1.1% |
| Y2 | 0.2% | 0.2% | 0.0% | 0.4% | 0.3% | 0.4% | 0.0% | 1.1% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test 2 FF IBC |  |  |  | Test 2 1x4 IBC |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.9% | 1.0% | 0.4% | 1.4% | 1.6% | 1.7% | 0.6% | 2.6% |
| R2 | 0.3% | 0.4% | 0.1% | 1.0% | 0.5% | 0.8% | 0.1% | 2.4% |
| R3 | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
| R4 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 1.0% | 1.1% | 0.2% | 1.6% | 1.6% | 1.8% | 0.3% | 3.1% |
| Y2 | 0.3% | 0.4% | 0.1% | 1.1% | 0.5% | 0.9% | 0.2% | 2.9% |
| Y3 | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% |
| Y4 | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Test 3 FF IBC |  |  |  | Test 3 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 1.1% | 1.3% | 0.3% | 1.9% | 2.5% | 2.7% | 0.6% | 4.7% |
| R2 | 0.3% | 0.4% | 0.0% | 1.0% | 0.7% | 1.6% | 0.1% | 5.5% |
| R3 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 1.2% | 1.3% | 0.1% | 2.0% | 2.7% | 2.9% | 0.2% | 5.8% |
| Y2 | 0.3% | 0.5% | 0.1% | 1.3% | 0.8% | 2.0% | 0.1% | 7.2% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

### Random Access

|  | Test 1 FF IBC |  |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.2% | 0.2% | 0.1% | 0.4% | 0.2% | 0.4% | 0.2% | 0.6% |
| R2 | 0.0% | 0.1% | 0.0% | 0.3% | 0.1% | 0.2% | 0.0% | 0.7% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.1% | 0.2% | 0.1% | 0.5% | 0.2% | 0.4% | 0.1% | 0.7% |
| Y2 | 0.0% | 0.1% | 0.0% | 0.3% | 0.0% | 0.2% | 0.0% | 0.8% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test 2 FF IBC |  |  |  | Test 2 1x4 IBC |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.3% | 0.5% | 0.2% | 0.9% | 0.4% | 0.9% | 0.3% | 1.8% |
| R2 | 0.1% | 0.3% | 0.0% | 0.8% | 0.1% | 0.5% | 0.0% | 1.9% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.2% | 0.6% | 0.1% | 1.0% | 0.3% | 1.0% | 0.2% | 2.0% |
| Y2 | 0.1% | 0.3% | 0.0% | 0.9% | 0.1% | 0.6% | 0.0% | 2.3% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Test 3 FF IBC |  |  |  | Test 3 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.3% | 0.7% | 0.1% | 1.2% | 0.4% | 1.5% | 0.2% | 3.3% |
| R2 | 0.0% | 0.2% | 0.0% | 0.8% | 0.1% | 1.1% | 0.0% | 4.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.2% | 0.8% | 0.1% | 1.4% | 0.4% | 1.8% | 0.1% | 4.0% |
| Y2 | 0.0% | 0.3% | 0.0% | 1.0% | 0.1% | 1.4% | 0.0% | 5.3% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

### Low Delay

|  | Test 1 FF IBC |  |  |  | Test 1 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.1% | 0.2% | 0.1% | 0.3% | 0.2% | 0.2% | 0.1% | 0.3% |
| R2 | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.3% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% | 0.3% | 0.1% | 0.4% |
| Y2 | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.4% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test 2 FF IBC |  |  |  | Test 2 1x4 IBC |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.2% | 0.4% | 0.1% | 0.6% | 0.2% | 0.5% | 0.2% | 1.0% |
| R2 | 0.0% | 0.2% | 0.0% | 0.5% | 0.1% | 0.3% | 0.0% | 0.9% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.2% | 0.4% | 0.1% | 0.6% | 0.2% | 0.6% | 0.1% | 1.1% |
| Y2 | 0.0% | 0.2% | 0.0% | 0.6% | 0.0% | 0.3% | 0.0% | 1.1% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Test 3 FF IBC |  |  |  | Test 3 1x4 IBC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |
| R1 | 0.2% | 0.5% | 0.1% | 0.8% | 0.3% | 1.0% | 0.2% | 2.1% |
| R2 | 0.0% | 0.1% | 0.0% | 0.3% | 0.0% | 0.3% | 0.0% | 1.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.1% | 0.5% | 0.0% | 0.9% | 0.2% | 1.2% | 0.1% | 2.7% |
| Y2 | 0.0% | 0.1% | 0.0% | 0.4% | 0.0% | 0.3% | 0.0% | 1.2% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, Japan, July 2014

- Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu, “Description of screen content coding technology proposal by Huawei Technologies (USA)”, JCTVC-Q0034, Valencia, Spain, April 2014

- W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu, “AHG8: String match in coding of screen content”, JCTVC-Q0176, Valencia, Spain, April 2014.

- Z. Ma, W. Wang, M. Xu, H. Yu, “Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer”, JCTVC-R0268, Sapporo, Japan, July 2014.

- Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu, “Non-SCCE3: Improvements for SCCE3 Test C 3”, JCTVC-R0304, Sapporo, Japan, July 2014.

- Z. Ma, Y.-W. Huang, “Description of Core Experiment 7 (CE7): String Matching for Palette Index Coding”, JCTVC-R1107, Sapporo, Japan, July 2014.