| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0028 |
| --- | --- |

| Title: | CE8: Summary report for Core Experiment 8 on Uni- and bi-colour mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document To JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen Tzu-Der Chuang Ching-Chieh Lin Krishna Rapaka | cohen@merl.com peter.chuang@mediatek.com JackLin@itri.org.tw krapaka@qti.qualcomm.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the summary of Core Experiment 8 (CE8) on uni- and bi-colour modes. CE8 includes three tests: Test A on bi-colour intra mode, Test B on single colour intra mode, and Test C on independent uniform prediction intra mode. A list of proposals related to CE8 that were not part of the CE8 description is also included.

# Introduction

At the 18th JCT-VC meeting, it was shown in JCTVC-R0112 [1], JCTVC-R0198 [2] (and JCTVC-R0058 [3]), and JCTVC-R0200 [4] that improvements in coding performance for screen content coding may be obtained by reconstructing or predicting blocks using one or two colours, independently of palette mode. Core Experiment 8 on unit- and bi-colour mode was defined [5] to study these proposed techniques. This report summarizes and compares the results of the tests performed under this core experiment. Combined summary tables are included in Section 7.

# Core experiment testing conditions

The proposed modifications were incorporated on top of SCM-2.0. The common conditions for screen content coding tests from JCTVC-R1015 [6] were used for these experiments.

# Test conditions

The common conditions for screen content coding tests from JCTVC-R1015 shall be used for these experiments.

# Description of tests

This core experiment tested three techniques: bi-colour intra mode, single colour intra mode, and independent uniform prediction intra mode.

## Test A – Bi-colour intra mode

Bi-colour intra mode first transmits an index to specify which two colours are selected out of available spatial neighboring pixels of the current CU. Then an index map for the CU is established by the selected two colours. An index map coding is used to transmit the index map. Two methods are tested: Method 1 uses two diverse colors selected from five neighboring samples, and Method 2 uses two diverse colors selected from the above two and left two lines.

## Test B – Single colour intra mode

A CU-level flag is signalled to indicate whether a CU is coded as single colour mode. When a CU is coded as single colour mode, it is reconstructed by filling this CU with one single value. Additionally, an index is transmitted to select the colour from the colour sample candidates derived from the spatial neighboring and / or previous CU samples to fill in the CU. Two methods are tested: Method 1 evaluates two spatial sample candidates from immediate neighboring samples, and Method 2 evaluates four spatial sample candidates from both four spatial neighboring samples plus colour candidates from the palette predictor table.

## Test C – Independent uniform prediction intra mode

Independent uniform prediction uses a uniform colour value (triplet) as prediction samples for a block. The number of candidate uniform colours and their values are signalled on a slice level. A CU-level flag indicates whether the CU uses this mode. If there is more than one colour, an index indicates which colour to use.

# CE8 document summary

| Test | Mode | Current Proposal | Crosscheck(s) | Tested Proposal(s) |
| --- | --- | --- | --- | --- |
| A | Bi-colour | JCTVC-S0049 | JCTVC-S0103 | JCTVC-R0112/Q0094 |
| B | Single colour | JCTVC-S0098 | JCTVC-S0190 | JCTVC-R0198/R0058 |
| C | Independent Uniform Prediction | JCTVC-S0176 | JCTVC-S0212 | JCTVC-R0200 |

Current proposals:

| Test A | JCTVC-S0049 | CE8 Test A: Bi-colour intra mode for screen content coding | Y.-J. Chang, C.-C. Lin, J.-S. Tu, C.-L. Lin, C.-H. Hung (ITRI) |
| --- | --- | --- | --- |
| Test B | JCTVC-S0098 | CE8 Test B: Single colour intra mode, with supplementary results | P. Lai, S. Liu, Y.-W. Chen, T.-D. Chuang, Y.-C. Sun, S. Lei (MediaTek) |
| Test C | JCTVC-S0176 | CE8 Test C: Independent Uniform Prediction Intra Mode (IUP) | R. Cohen, X. Zhang (MERL) |

Current crosschecks:

| Test A | JCTVC-S0103 | CE8: Cross-check of Test A (JCTVC-S0049) Bi-colour intra mode for screen content coding | P. Lai (MediaTek) |
| --- | --- | --- | --- |
| Test B | JCTVC-S0190 | CE8: Cross-check of JCTVC-S0098, CE8 Test B: Single colour intra mode | R. Cohen (MERL) |
| Test C | JCTVC-S0212 | CE8: Cross-Check Results of Test C | J.-S. Tu, C.-H. Hung, C. -C Lin, Y.-J. Chang, C.-L. Lin (ITRI) |

Proposals for tests related to CE8 that were not part of the CE8 description (“Non-CE8):

| JCTVC-S0050 | Non-CE8: Combination of CE8 Test A and CE8 Test B for screen content coding | Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung (ITRI) |
| --- | --- | --- |
| JCTVC-S0051 | Non-CE8: Multi-colour intra mode for screen content coding | Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung (ITRI) |

# Test Results

This section includes separate BD-Rate summary tables for each CE8 test. Encoder or decoder run-time ratios that may not be accurate due to variations in computing platforms are so noted.

## Test A – Bi-colour intra mode

### Method 1: Two diverse colors selected from five neighboring samples

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | 0.1% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.2% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | 0.2% | -0.4% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 101% |  |  |

Lossless test conditions:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

### Method 2: Two diverse colors selected from above two and left two lines

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.3% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | 0.4% |
| YUV, mixed content, 1080p | -0.1% | 0.5% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |

Lossless test conditions:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.2% | 0.2% | 0.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

## Test B – Single colour intra mode

### Method 1: Candidate list size 2, spatial neighboring samples

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.7% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.6% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.5% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | 0.1% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -0.3% | -0.1% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.5% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.3% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

Lossless test conditions:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

### Method 2: Candidate list size 4, spatial neighboring samples, plus colour candidates from palette predictor table

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.6% | -0.6% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.3% | -0.5% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 113% (not accurate) |  |  |
| Dec Time[%] | 123% (not accurate) |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | 0.2% | 0.0% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -0.4% | -0.2% | -0.6% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.4% |
| YUV, mixed content, 1080p | -0.2% | 0.3% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.4% | -0.2% |
| Enc Time[%] | 111% (not accurate) |  |  |
| Dec Time[%] | 125% (not accurate) |  |  |

Lossless test conditions:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.2% | 0.2% | 0.1% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% (not accurate) |  |  |  |
| Dec Time[%] | 117% (not accurate) |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% (not accurate) |  |  |  |
| Dec Time[%] | 127% (not accurate) |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% (not accurate) |  |  |  |
| Dec Time[%] | 130% (not accurate) |  |  |  |

## Test C – Independent uniform prediction intra mode

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.5% |
| YUV, text & graphics with motion,720p | -0.1% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 95% (may be slightly inaccurate) |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | 0.2% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% (may be slightly inaccurate) |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.3% | 1.4% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 96% (may be slightly inaccurate) |  |  |

Lossless test conditions:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.3% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 99% (may be slightly inaccurate) |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% (may be slightly inaccurate) |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.2% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.2% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% (may be slightly inaccurate) |  |  |  |

# Combined summary tables

This section contains combined BD-Rate summary tables.

To reduce table size, the following notations for sequence types are used:

| R1 | RGB, text & graphics with motion, 1080p |
| --- | --- |
| R2 | RGB, text & graphics with motion,720p |
| R3 | RGB, mixed content, 1440p |
| R4 | RGB, mixed content, 1080p |
| R5 | RGB, Animation, 720p |
| R6 | RGB, camera captured, 1080p |
| Y1 | YUV, text & graphics with motion, 1080p |
| Y2 | YUV, text & graphics with motion,720p |
| Y3 | YUV, mixed content, 1440p |
| Y4 | YUV, mixed content, 1080p |
| Y5 | YUV, Animation, 720p |
| Y6 | YUV, camera captured, 1080p |

## Lossy test conditions

### All-intra

|  | Test A: Bi-colour Method 1 |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  | G/Y | B/U |  | R/V |  |
| R1 | -0.4% | -0.3% | -0.3% |  | -0.4% | -0.4% | -0.4% |  | R1 | -0.3% |  | -0.3% |  | -0.3% |
| R2 | -0.4% | -0.4% | -0.4% |  | -0.4% | -0.3% | -0.3% |  | R2 | -0.4% |  | -0.5% |  | -0.4% |
| R3 | -0.2% | -0.1% | -0.2% |  | -0.1% | -0.1% | -0.1% |  | R3 | -0.1% |  | -0.1% |  | -0.1% |
| R4 | -0.2% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  | R4 | 0.0% |  | 0.0% |  | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | R5 | 0.0% |  | 0.0% |  | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | R6 | 0.0% |  | 0.0% |  | 0.0% |
| Y1 | -0.2% | -0.1% | -0.1% |  | -0.3% | -0.2% | -0.2% |  | Y1 | 0.5% |  | 0.5% |  | 0.5% |
| Y2 | -0.1% | -0.3% | -0.2% |  | -0.1% | -0.3% | 0.0% |  | Y2 | -0.1% |  | -0.4% |  | -0.4% |
| Y3 | -0.1% | -0.3% | -0.4% |  | -0.1% | -0.1% | -0.2% |  | Y3 | -0.1% |  | -0.1% |  | 0.0% |
| Y4 | -0.1% | -0.1% | -0.1% |  | -0.1% | 0.0% | -0.1% |  | Y4 | 0.0% |  | 0.0% |  | 0.0% |
| Y5 | 0.0% | -0.1% | -0.1% |  | 0.0% | -0.1% | -0.1% |  | Y5 | 0.0% |  | 0.0% |  | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | Y6 | 0.0% |  | 0.0% |  | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  |  |  |  |  |  |
| R1 | -0.6% | -0.5% | -0.5% |  | -0.5% | -0.5% | -0.5% |  |  |  |  |  |  |  |
| R2 | -0.8% | -0.8% | -0.7% |  | -0.8% | -0.8% | -0.8% |  |  |  |  |  |  |  |
| R3 | -0.3% | -0.2% | -0.3% |  | -0.3% | -0.2% | -0.3% |  |  |  |  |  |  |  |
| R4 | -0.3% | -0.2% | -0.3% |  | -0.3% | -0.2% | -0.3% |  |  |  |  |  |  |  |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |  |
| Y1 | -0.1% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% |  |  |  |  |  |  |  |
| Y2 | -0.5% | -0.6% | -0.5% |  | -0.6% | -0.6% | -0.5% |  |  |  |  |  |  |  |
| Y3 | -0.3% | -0.3% | -0.4% |  | -0.3% | -0.3% | -0.4% |  |  |  |  |  |  |  |
| Y4 | -0.3% | -0.2% | -0.2% |  | -0.3% | -0.2% | -0.2% |  |  |  |  |  |  |  |
| Y5 | 0.0% | -0.2% | -0.2% |  | 0.0% | -0.2% | -0.2% |  |  |  |  |  |  |  |
| Y6 | -0.1% | -0.1% | -0.2% |  | 0.0% | -0.1% | -0.1% |  |  |  |  |  |  |  |

### Random Access

|  | Test A: Bi-colour Method 1 |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  | G/Y | B/U |  | R/V |  |
| R1 | -0.1% | 0.0% | -0.1% |  | -0.1% | -0.1% | -0.1% |  | R1 | -0.1% |  | 0.0% |  | 0.0% |
| R2 | -0.2% | -0.3% | -0.2% |  | -0.2% | -0.2% | -0.2% |  | R2 | -0.2% |  | -0.2% |  | -0.1% |
| R3 | -0.1% | -0.2% | -0.2% |  | -0.1% | -0.2% | -0.1% |  | R3 | -0.1% |  | -0.2% |  | -0.2% |
| R4 | 0.0% | 0.1% | -0.1% |  | -0.1% | -0.1% | -0.2% |  | R4 | 0.2% |  | 0.1% |  | 0.1% |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | R5 | 0.0% |  | 0.0% |  | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | R6 | 0.0% |  | 0.0% |  | 0.0% |
| Y1 | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |  | Y1 | 0.2% |  | 0.1% |  | 0.1% |
| Y2 | 0.0% | -0.2% | -0.1% |  | 0.0% | -0.1% | 0.0% |  | Y2 | 0.1% |  | -0.1% |  | 0.1% |
| Y3 | -0.1% | -0.3% | -0.2% |  | -0.1% | -0.1% | -0.1% |  | Y3 | -0.1% |  | -0.2% |  | -0.2% |
| Y4 | 0.0% | -0.1% | -0.1% |  | 0.0% | -0.2% | -0.1% |  | Y4 | 0.1% |  | 0.0% |  | 0.0% |
| Y5 | 0.1% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | Y5 | 0.0% |  | 0.0% |  | 0.0% |
| Y6 | 0.0% | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | Y6 | 0.0% |  | 0.0% |  | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  |  |  |  |  |  |
| R1 | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |  |  |  |  |  |  |
| R2 | -0.5% | -0.5% | -0.6% |  | -0.5% | -0.5% | -0.5% |  |  |  |  |  |  |  |
| R3 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | -0.1% |  |  |  |  |  |  |  |
| R4 | 0.0% | -0.1% | -0.2% |  | 0.0% | -0.2% | -0.3% |  |  |  |  |  |  |  |
| R5 | 0.0% | 0.0% | 0.0% |  | 0.1% | 0.1% | -0.1% |  |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |  |
| Y1 | 0.0% | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |  |
| Y2 | -0.3% | -0.6% | -0.5% |  | -0.3% | -0.5% | -0.4% |  |  |  |  |  |  |  |
| Y3 | 0.0% | -0.3% | -0.2% |  | 0.0% | -0.4% | -0.2% |  |  |  |  |  |  |  |
| Y4 | -0.1% | -0.3% | -0.4% |  | -0.1% | -0.1% | -0.2% |  |  |  |  |  |  |  |
| Y5 | 0.0% | -0.1% | -0.1% |  | 0.0% | -0.1% | -0.2% |  |  |  |  |  |  |  |
| Y6 | 0.0% | 0.0% | -0.2% |  | 0.0% | -0.1% | -0.2% |  |  |  |  |  |  |  |

### Low Delay

|  | Test A: Bi-colour Method 1 |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  | G/Y | B/U |  | R/V |  |
| R1 | 0.0% | 0.0% | 0.0% |  | -0.1% | -0.1% | 0.0% |  | R1 | 0.1% |  | 0.1% |  | 0.1% |
| R2 | -0.1% | -0.2% | 0.0% |  | -0.1% | 0.0% | 0.1% |  | R2 | 0.3% |  | 0.2% |  | 0.3% |
| R3 | -0.2% | -0.2% | -0.3% |  | 0.0% | -0.3% | -0.1% |  | R3 | 0.1% |  | 0.0% |  | -0.2% |
| R4 | -0.3% | 0.1% | 0.3% |  | -0.1% | 0.3% | 0.4% |  | R4 | 0.1% |  | 0.3% |  | 0.3% |
| R5 | 0.0% | 0.1% | 0.0% |  | 0.0% | 0.1% | 0.1% |  | R5 | 0.0% |  | 0.0% |  | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | -0.1% | 0.0% |  | R6 | 0.0% |  | 0.0% |  | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% |  | 0.0% | -0.1% | 0.0% |  | Y1 | 0.1% |  | 0.1% |  | 0.2% |
| Y2 | -0.1% | 0.2% | 0.0% |  | 0.0% | 0.3% | 0.1% |  | Y2 | 0.3% |  | 0.4% |  | 0.0% |
| Y3 | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.2% | 0.4% |  | Y3 | -0.1% |  | -0.1% |  | 0.3% |
| Y4 | -0.1% | 0.2% | -0.4% |  | -0.1% | 0.5% | 0.0% |  | Y4 | 0.3% |  | 1.4% |  | 0.1% |
| Y5 | 0.1% | -0.3% | 0.1% |  | 0.1% | -0.4% | 0.3% |  | Y5 | 0.0% |  | 0.0% |  | 0.0% |
| Y6 | 0.0% | -0.1% | 0.0% |  | 0.0% | -0.1% | 0.2% |  | Y6 | 0.0% |  | 0.0% |  | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V |  |  |  |  |  |  |  |
| R1 | -0.1% | 0.0% | 0.0% |  | -0.1% | 0.0% | 0.1% |  |  |  |  |  |  |  |
| R2 | -0.2% | -0.1% | 0.1% |  | -0.2% | -0.2% | -0.1% |  |  |  |  |  |  |  |
| R3 | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.4% | -0.2% |  |  |  |  |  |  |  |
| R4 | -0.4% | 0.1% | 0.3% |  | 0.2% | 0.0% | 0.3% |  |  |  |  |  |  |  |
| R5 | 0.0% | 0.1% | 0.0% |  | 0.0% | 0.1% | 0.0% |  |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |  |
| Y1 | 0.0% | 0.1% | 0.1% |  | 0.0% | 0.1% | 0.1% |  |  |  |  |  |  |  |
| Y2 | -0.3% | -0.1% | -0.5% |  | -0.4% | -0.2% | -0.6% |  |  |  |  |  |  |  |
| Y3 | -0.3% | -0.1% | -0.3% |  | -0.1% | -0.2% | -0.4% |  |  |  |  |  |  |  |
| Y4 | 0.0% | 0.1% | 0.0% |  | -0.2% | 0.3% | 0.1% |  |  |  |  |  |  |  |
| Y5 | 0.1% | -0.5% | 0.1% |  | 0.0% | -0.4% | 0.0% |  |  |  |  |  |  |  |
| Y6 | 0.0% | -0.3% | 0.0% |  | 0.0% | -0.4% | -0.2% |  |  |  |  |  |  |  |

## Lossless test conditions

### All-intra

|  | Test A: Bi-colour Method 1 |  |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  | Total | Avg. | Min | Max |
| R1 | 0.2% | 0.2% | 0.1% | 0.3% | 0.3% | 0.2% | 0.2% | 0.3% |  | R1 | 0.1% | 0.2% | 0.1% | 0.3% |
| R2 | 0.1% | 0.2% | 0.1% | 0.3% | 0.1% | 0.2% | 0.1% | 0.3% |  | R2 | 0.1% | 0.1% | 0.1% | 0.4% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  | R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% |  | Y1 | -0.2% | -0.2% | -0.3% | -0.1% |
| Y2 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  | Y2 | 0.0% | 0.0% | 0.0% | 0.1% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y6 | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  |  |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  |  |  |  |  |
| R1 | 0.3% | 0.3% | 0.1% | 0.4% | 0.3% | 0.3% | 0.1% | 0.4% |  |  |  |  |  |  |
| R2 | 0.1% | 0.2% | 0.1% | 0.6% | 0.2% | 0.2% | 0.1% | 0.6% |  |  |  |  |  |  |
| R3 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |  |  |  |
| R4 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |  |  |  |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y1 | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |  |  |  |  |  |  |
| Y2 | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |  |  |  |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y4 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |  |  |  |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |

### Random Access

|  | Test A: Bi-colour Method 1 |  |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  | Total | Avg. | Min | Max |
| R1 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  | R1 | 0.1% | 0.1% | 0.0% | 0.1% |
| R2 | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% |  | R2 | 0.0% | 0.0% | 0.0% | 0.2% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  | Y1 | 0.0% | -0.1% | -0.2% | 0.0% |
| Y2 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  | Y2 | 0.0% | 0.0% | -0.1% | 0.0% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y6 | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  |  |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  |  |  |  |  |
| R1 | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |  |  |  |
| R2 | 0.0% | 0.2% | 0.0% | 0.4% | 0.0% | 0.2% | 0.0% | 0.4% |  |  |  |  |  |  |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y1 | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |  |  |  |  |  |  |
| Y2 | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% |  |  |  |  |  |  |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |

### Low Delay

|  | Test A: Bi-colour Method 1 |  |  |  | Test A: Bi-colour Method 2 |  |  |  |  |  | Test C: Indep. Unif. Pred. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  | Bit-rate saving |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  | Total | Avg. | Min | Max |
| R1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R1 | 0.0% | 0.0% | -0.2% | 0.0% |
| R2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R2 | 0.0% | 0.0% | -0.1% | 0.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y1 | 0.0% | -0.1% | -0.2% | 0.1% |
| Y2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y2 | 0.0% | -0.1% | -0.2% | 0.0% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | Y6 | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Test B: Single-colour Method 1 |  |  |  | Test B: Single-colour Method 2 |  |  |  |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  | Bit-rate saving |  |  |  |  |  |  |  |  |  |
|  | Total | Avg. | Min | Max | Total | Avg. | Min | Max |  |  |  |  |  |  |
| R1 | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |  |  |  |
| R2 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |  |  |  |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y2 | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |  |  |  |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |

# References

- Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung, “Non-SCCE3: Bi-color intra mode for screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0112, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- P. Lai, S. Liu, Y.-W. Chen, S. Lei, “Non-SCCE3 Test D.1 Modified single color mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0112, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei, “SCCE3 Test D.1: Single color intra mode for screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0058, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- X. Zhang, R. Cohen, “SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0112, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- R. Cohen, T.-D. Chuang, C.-C. Lin, K. Rapaka, “Description of Core Experiment 8 (CE8): Uni- and bi-colour mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1108, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu ,	“Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1108, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.