| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0029_r1 |
| --- | --- |

| Title: | CE9: Summary report for Core Experiment 9 on IBF/CCP interdependency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document To JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen Shan Liu Jizheng Xu Li Zhang | cohen@merl.com shan.liu@mediatek.com jzxu@microsoft.com lizhang@qti.qualcomm.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the summary of Core Experiment 9 (CE9) on intra boundary filtering (IBF) and cross-component prediction (CCP) interdependency. CE9 includes three individual tests: Test A.1 on optionally disabling the usage of the intra boundary filters, Test A.2 on optionally enabling the usage of the intra boundary filters on all components, and Test A.3 on modifying cross-component prediction to compensate for intra boundary filtering. Combination tests are also discussed.

# Introduction

At the 18th JCT-VC meeting, it was shown in JCTVC-R0114 [1], JCTVC-R0314 [2], and JCTVC-R0219 [3] that improvements in coding performance for screen content coding may be obtained by disabling intra boundary filtering for the first (G/Y) component, by enabling intra boundary filtering for all three components, or by compensating for interdependencies between intra boundary filtering (IBF) and cross-component prediction (CCP), because the existing CCP is uses a boundary-filtered first component to predict the non-filtered remaining components. Core Experiment 9 on IBF/CCP interdependency was defined [4] to study these proposed techniques. This report summarizes and compares the results of the tests performed under this core experiment. Combined summary tables are included in Section 7.

# Core experiment testing conditions

The proposed modifications were incorporated on top of SCM-2.0. The common conditions for screen content coding tests from JCTVC-R1015 [5] were used for these experiments.

# Test conditions

The common conditions for screen content coding tests from JCTVC-R1015 shall be used for these experiments.

# Description of tests

This core experiment tested three techniques: Optionally disabling the usage of the intra boundary filters, optionally enabling the usage of the intra boundary filters on all components, and modifying cross-component prediction to compensate for intra boundary filtering.

## Individual tool tests

### Test A.1 – Optionally disabling the usage of the intra boundary filters

It is proposed to conditionally disable the boundary filters for intra horizontal, vertical, and DC modes with flag controls in SPS and slice header. Two methods are tested: Method 1 disables all the intra boundary filters. For Method 2, if the proportion of 2nd and 3rd component 16x16 blocks having sharp edges exceeds a threshold, then the intra boundary filters are disabled.

### Test A.2 – Optionally enabling the usage of the intra boundary filters on all components

It is proposed to enable the intra boundary filter for the second and third components in the 444 chroma sampling format. An enable flag is added at the SPS or PPS level.

### Test A.3 – Modifying cross-component prediction to compensate for intra boundary filtering

It is proposed to modify the CCP process by adding an offset block to the reconstructed luma (or first component) residual block. This offset block is the difference between the luma prediction block after intra boundary filtering is applied and the same block before intra boundary filtering is applied. For blocks in which the adaptive color transform has been applied, the luma offset is also subtracted from the CCP computations for the Cg component.

## Combination and additional tests

### Test B.1 – Combination of tests A.1 and A.3

This test was postponed pending discussion.

### Supplemental test: Combination of tests A.1 and A.2

Results from a supplemental test, which was not defined in the original CE9 description, are included here for completeness and to facilitate discussion. In this test, if the number of 16x16 sharp-edge blocks exceeds that of smooth-edge blocks, IBF is disabled for all components; Otherwise, IBF is enabled for all components.

# CE9 document summary

| Test | Mode | Current Proposal | Crosscheck(s) | Tested Proposal(s) |
| --- | --- | --- | --- | --- |
| A.1 | Optionally disabling the usage of the intra boundary filters | JCTVC-S0102 | JCTVC-S0191 | JCTVC-R0114 |
| A.2 | Optionally enabling the usage of the intra boundary filters on all components | JCTVC-S0082 | JCTVC-S0268 | JCTVC-R0314 |
| A.3 | Modifying cross-component prediction to compensate for intra boundary filtering | JCTVC-S0177 | JCTVC-S0221 | JCTVC-R0219 |

Current proposals:

| Test A.1 | JCTVC-S0102 | CE9 Test A.1: Optionally disabling the usage of the intra boundary filters | X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek) |
| --- | --- | --- | --- |
| Test A.2 | JCTVC-S0082 | CE9: Result of Test A.2 | B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen (MERL) |
| Test A.3 | JCTVC-S0177 | CE9 Test A.3 Modifying cross-component prediction to compensate for intra boundary filtering | R. Cohen, X. Zhang (MERL) |

Current crosschecks:

| Test A.1 | JCTVC-S0191 | CE9: Cross-check of JCTVC-S0102, CE9 Test A.1: Optionally disabling the usage of the intra boundary filters | R. Cohen (MERL) |
| --- | --- | --- | --- |
| Test A.2 | JCTVC-S0268 | CE9: Cross-check of JCTVC-S0082 Test A.2 | X. Zhang (MediaTek) |
| Test A.3 | JCTVC-S0221 | CE9: crosscheck report of CE9 Test A.3 (JCTVC-S0177) | L. Zhang (Qualcomm) |

Non-CE9 proposals:

| JCTVC-S0046 | Non-CE9: Intra-boundary filter control for non-camera captured content | C. Rosewarne, M. Maeda (Canon) |
| --- | --- | --- |
| JCTVC-S0199 | Non-CE9: On chroma boundary filtering | F. Zou, R. Joshi, W. Pu, V. Seregin, M. Karczewicz (Qualcomm) |

# Test Results

This section includes separate BD-Rate summary tables for each test. Encoder or decoder run-time ratios that may not be accurate due to variations in computing platforms are so noted.

## Test A.1 – Optionally disabling the usage of the intra boundary filters

### Method 1: Disable all the intra boundary filters

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.2% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.1% | -0.2% | 0.1% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Lossless test conditions:

|  |  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |  |  |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 103% |  |  |  |  |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |  |  |  |  |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |
| Enc Time[%] |  | 101% |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |  |  |

### Method 2: If the proportion of 2nd and 3rd component 16x16 blocks having sharp edges exceeds a threshold, then the intra boundary filters are disabled

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Lossless test conditions:

|  |  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 98% |  |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |  |  |

## Test A.2 – Optionally enabling the usage of the intra boundary filters on all components

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.8% | -0.4% | -0.5% |
| RGB, mixed content, 1440p | -1.5% | -0.9% | -1.0% |
| RGB, mixed content, 1080p | -1.3% | -0.9% | -0.8% |
| RGB, Animation, 720p | -0.5% | -0.3% | -0.4% |
| RGB, camera captured, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -1.0% | -0.7% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.2% | -0.3% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -1.2% | -0.4% | -0.7% |
| RGB, mixed content, 1080p | -0.8% | 0.1% | 0.5% |
| RGB, Animation, 720p | -0.2% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | -0.2% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Lossless test conditions:

|  |  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  |
| Enc Time[%] |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 104% |  |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |  |  |

## Test A.3 – Modifying cross-component prediction to compensate for intra boundary filtering

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 105% (may be inaccurate) |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | -0.4% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% (may be inaccurate) |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.1% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.8% |
| YUV, Animation, 720p | 0.1% | -0.6% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 105% (may be inaccurate) |  |  |

Lossless test conditions:

|  |  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  |  | Bit-rate saving (Min) |  |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| Enc Time[%] |  | 102% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 110% (may be inaccurate) |  |  |  |  |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |  |  |  |  |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  |  | Bit-rate saving (Min) |  |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 103% (may be inaccurate) |  |  |  |  |  |  |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |  |  |  |  |  |  |
|  | Bit-rate saving (Total) |  |  | Bit-rate saving (Average) |  |  | Bit-rate saving (Min) |  |  | Bit-rate saving (Max) |  |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  | 104% (may be inaccurate) |  |  |  |  |  |  |  |  |  |  |

## Supplemental test results: Combination of tests A.1 and A.2.

Lossy test conditions:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.2% | -0.3% |
| RGB, text & graphics with motion,720p | -0.8% | -0.4% | -0.5% |
| RGB, mixed content, 1440p | -1.5% | -0.9% | -1.0% |
| RGB, mixed content, 1080p | -1.3% | -0.9% | -0.8% |
| RGB, Animation, 720p | -0.5% | -0.3% | -0.4% |
| RGB, camera captured, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -1.0% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.2% | -0.3% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.2% |
| RGB, mixed content, 1440p | -1.2% | -0.4% | -0.7% |
| RGB, mixed content, 1080p | -0.8% | 0.1% | 0.5% |
| RGB, Animation, 720p | -0.2% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | -0.2% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Lossless test conditions:

|  |  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 99% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 103% |  |  |  |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |  |  |  |
|  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion,720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1440p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, mixed content, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, Animation, 720p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  | 98% |  |  |  |  |  |  |  |

# Combined summary tables

This section contains combined BD-Rate summary tables.

To reduce table size, the following notations for sequence types are used:

| R1 | RGB, text & graphics with motion, 1080p |
| --- | --- |
| R2 | RGB, text & graphics with motion,720p |
| R3 | RGB, mixed content, 1440p |
| R4 | RGB, mixed content, 1080p |
| R5 | RGB, Animation, 720p |
| R6 | RGB, camera captured, 1080p |
| Y1 | YUV, text & graphics with motion, 1080p |
| Y2 | YUV, text & graphics with motion,720p |
| Y3 | YUV, mixed content, 1440p |
| Y4 | YUV, mixed content, 1080p |
| Y5 | YUV, Animation, 720p |
| Y6 | YUV, camera captured, 1080p |

## Lossy test conditions

### All-intra

|  | Test A.1: Disable IBF Method 1 (always) |  |  | Test A.1: Disable IBF Method 2 (threshold) |  |  |  | Test A.2: Enable IBF for all components |  |  | Test A.3: Modify CCP to compensate for IBF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -0.5% | -0.3% | -0.4% |  | -0.5% | -0.3% | -0.4% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| R2 | -0.6% | -0.3% | -0.4% |  | -0.6% | -0.3% | -0.4% | -0.8% | -0.4% | -0.5% | -0.1% | 0.0% | 0.0% |
| R3 | -1.2% | -0.7% | -0.8% |  | -1.2% | -0.7% | -0.8% | -1.5% | -0.9% | -1.0% | 0.0% | -0.1% | -0.1% |
| R4 | -0.9% | -0.6% | -0.6% |  | -0.9% | -0.6% | -0.6% | -1.3% | -0.9% | -0.8% | -0.1% | -0.1% | 0.0% |
| R5 | -0.2% | 0.0% | -0.1% |  | -0.2% | 0.0% | -0.1% | -0.5% | -0.3% | -0.4% | -0.1% | 0.0% | -0.1% |
| R6 | -0.1% | 0.0% | -0.1% |  | -0.1% | -0.1% | -0.1% | -0.4% | -0.4% | -0.4% | 0.0% | 0.0% | 0.0% |
| Y1 | -0.2% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y2 | 0.1% | 0.2% | 0.1% |  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Y3 | 0.2% | 0.1% | 0.2% |  | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Y4 | 0.2% | 0.1% | 0.2% |  | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Y5 | 0.2% | 0.0% | 0.2% |  | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% | 0.1% | -0.1% | -0.2% | -0.1% |
| Y6 | 0.3% | 0.3% | 0.1% |  | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% | -0.1% | -0.3% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| R1 | -0.4% | -0.2% | -0.3% |
| R2 | -0.8% | -0.4% | -0.5% |
| R3 | -1.5% | -0.9% | -1.0% |
| R4 | -1.3% | -0.9% | -0.8% |
| R5 | -0.5% | -0.3% | -0.4% |
| R6 | -0.4% | -0.4% | -0.4% |
| Y1 | -0.1% | 0.0% | 0.0% |
| Y2 | -0.1% | 0.0% | 0.1% |
| Y3 | -0.2% | -0.1% | 0.0% |
| Y4 | -0.2% | -0.1% | 0.0% |
| Y5 | -0.2% | 0.1% | 0.1% |
| Y6 | -0.1% | 0.1% | -0.1% |

### Random Access

|  | Test A.1: Disable IBF Method 1 (always) |  |  | Test A.1: Disable IBF Method 2 (threshold) |  |  |  | Test A.2: Enable IBF for all components |  |  | Test A.3: Modify CCP to compensate for IBF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -0.2% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| R2 | -0.4% | -0.2% | -0.3% |  | -0.4% | -0.2% | -0.3% | -0.6% | -0.4% | -0.4% | -0.1% | -0.1% | -0.1% |
| R3 | -1.1% | -0.7% | -0.6% |  | -1.1% | -0.7% | -0.6% | -1.2% | -0.7% | -0.7% | -0.1% | -0.1% | -0.1% |
| R4 | -0.6% | -0.4% | -0.4% |  | -0.6% | -0.4% | -0.4% | -1.0% | -0.7% | -0.6% | 0.0% | -0.1% | -0.2% |
| R5 | -0.1% | -0.1% | -0.2% |  | -0.1% | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| R6 | -0.1% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y2 | 0.1% | 0.0% | 0.2% |  | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% | -0.2% | 0.0% |
| Y3 | 0.1% | -0.2% | 0.1% |  | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Y4 | 0.2% | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.1% | 0.0% | -0.4% | -0.2% |
| Y5 | 0.1% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% |
| Y6 | 0.1% | 0.2% | 0.0% |  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.2% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| R1 | -0.1% | 0.0% | -0.1% |
| R2 | -0.6% | -0.4% | -0.4% |
| R3 | -1.2% | -0.7% | -0.7% |
| R4 | -1.0% | -0.6% | -0.6% |
| R5 | -0.3% | -0.2% | -0.3% |
| R6 | -0.2% | -0.1% | -0.2% |
| Y1 | 0.0% | 0.0% | 0.0% |
| Y2 | -0.1% | 0.0% | 0.2% |
| Y3 | -0.2% | -0.2% | 0.0% |
| Y4 | -0.1% | -0.3% | -0.1% |
| Y5 | -0.1% | 0.0% | 0.1% |
| Y6 | 0.0% | 0.2% | 0.0% |

### Low Delay

|  | Test A.1: Disable IBF Method 1 (always) |  |  | Test A.1: Disable IBF Method 2 (threshold) |  |  |  | Test A.2: Enable IBF for all components |  |  | Test A.3: Modify CCP to compensate for IBF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V |  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| R1 | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| R2 | -0.5% | -0.3% | -0.3% |  | -0.5% | -0.3% | -0.3% | -0.5% | -0.4% | -0.3% | 0.0% | 0.1% | 0.1% |
| R3 | -0.9% | -0.6% | -0.6% |  | -0.9% | -0.6% | -0.6% | -1.2% | -0.4% | -0.7% | -0.2% | -0.1% | -0.1% |
| R4 | -0.5% | -0.4% | 0.5% |  | -0.5% | -0.4% | 0.5% | -0.8% | 0.1% | 0.5% | -0.2% | 0.1% | 0.5% |
| R5 | -0.1% | 0.1% | 0.0% |  | -0.1% | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| R6 | 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Y2 | -0.1% | -0.1% | -0.2% |  | -0.1% | 0.1% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | -0.1% | -0.3% |
| Y3 | -0.3% | 0.0% | 0.2% |  | -0.3% | -0.1% | 0.2% | -0.2% | -0.1% | 0.3% | -0.1% | -0.4% | 0.0% |
| Y4 | -0.1% | -0.1% | -0.3% |  | 0.0% | 0.2% | -0.4% | -0.2% | 0.1% | 0.1% | -0.1% | -0.1% | -0.8% |
| Y5 | 0.1% | -0.3% | 0.2% |  | 0.0% | -0.3% | -0.1% | 0.1% | -0.4% | 0.2% | 0.1% | -0.6% | -0.2% |
| Y6 | 0.0% | 0.0% | 0.2% |  | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| R1 | 0.0% | 0.0% | 0.0% |
| R2 | -0.5% | -0.4% | -0.2% |
| R3 | -1.2% | -0.4% | -0.7% |
| R4 | -0.8% | 0.1% | 0.5% |
| R5 | -0.2% | 0.0% | 0.0% |
| R6 | 0.0% | -0.1% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.1% |
| Y2 | -0.3% | 0.0% | -0.1% |
| Y3 | -0.2% | -0.1% | 0.3% |
| Y4 | -0.2% | 0.1% | 0.1% |
| Y5 | 0.1% | -0.4% | 0.2% |
| Y6 | 0.0% | 0.0% | 0.0% |

## Lossless test conditions

### All-intra

|  | Test A.1: Disable IBF Method 1 (always) | Test A.1: Disable IBF Method 2 (threshold) | Test A.2: Enable IBF for all components | Test A.3: Modify CCP to compensate for IBF |
| --- | --- | --- | --- | --- |
| R1 | 0.0% | 0.0% | 0.0% | 0.0% |
| R2 | 0.0% | 0.0% | 0.0% | 0.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y2 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.1% | 0.0% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |
| --- | --- |
| R1 | 0.0% |
| R2 | 0.0% |
| R3 | 0.0% |
| R4 | 0.0% |
| R5 | 0.0% |
| R6 | 0.0% |
| Y1 | 0.0% |
| Y2 | 0.0% |
| Y3 | 0.0% |
| Y4 | 0.0% |
| Y5 | 0.0% |
| Y6 | 0.0% |

### Random Access

|  | Test A.1: Disable IBF Method 1 (always) | Test A.1: Disable IBF Method 2 (threshold) | Test A.2: Enable IBF for all components | Test A.3: Modify CCP to compensate for IBF |
| --- | --- | --- | --- | --- |
| R1 | 0.0% | 0.0% | 0.0% | 0.0% |
| R2 | 0.0% | 0.0% | 0.0% | 0.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y2 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |
| --- | --- |
| R1 | 0.0% |
| R2 | 0.0% |
| R3 | 0.0% |
| R4 | 0.0% |
| R5 | 0.0% |
| R6 | 0.0% |
| Y1 | 0.0% |
| Y2 | 0.0% |
| Y3 | 0.0% |
| Y4 | 0.0% |
| Y5 | 0.0% |
| Y6 | 0.0% |

### Low Delay

|  | Test A.1: Disable IBF Method 1 (always) | Test A.1: Disable IBF Method 2 (threshold) | Test A.2: Enable IBF for all components | Test A.3: Modify CCP to compensate for IBF |
| --- | --- | --- | --- | --- |
| R1 | 0.0% | 0.0% | 0.0% | 0.0% |
| R2 | 0.0% | 0.0% | 0.0% | 0.0% |
| R3 | 0.0% | 0.0% | 0.0% | 0.0% |
| R4 | 0.0% | 0.0% | 0.0% | 0.0% |
| R5 | 0.0% | 0.0% | 0.0% | 0.0% |
| R6 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y1 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y2 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y3 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y4 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y5 | 0.0% | 0.0% | 0.0% | 0.0% |
| Y6 | 0.0% | 0.0% | 0.0% | 0.0% |

Supplemental results:

|  | Supplemental test: Combining A.1 and A.2 |
| --- | --- |
| R1 | 0.0% |
| R2 | 0.0% |
| R3 | 0.0% |
| R4 | 0.0% |
| R5 | 0.0% |
| R6 | 0.0% |
| Y1 | 0.0% |
| Y2 | 0.0% |
| Y3 | 0.0% |
| Y4 | 0.0% |
| Y5 | 0.0% |
| Y6 | 0.0% |

# References

- X. Zhang, K. Zhang, J. An, H. Huang, S. Lei, “Optionally disabling the usage of the intra boundary filters for SCC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0114, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen, “On intra chroma boundary filtering,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0314, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- X. Zhang, R. Cohen, “Improvement of cross-component prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0219, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- R. Cohen, T.-D. Chuang, C.-C. Lin, K. Rapaka, “Description of Core Experiment 8 (CE8): Uni- and bi-colour mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1108, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu ,	“Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1108, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.