| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, Fr, 17 October – 24 October 2014 | Document: JCTVC-S0031 |
| --- | --- |

| Title: | Additional Definitions of FPA SEI Message for Inclusion of Centralized Color-Depth Packing (CCDP) Formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jar-Ferr Yang Ke-Ying Liao Ya-Han Hu Hung-Ming Wang | Tel: Email: | +886-6-2757575#62362 jefyang@mail.ncku.edu.tw |
| Source: | National Cheng Kung University (NCKU), 1 University Road, Tainan, Taiwan 701 |  |  |

_____________________________

# Abstract

The most popular frame compatible formats for transmitting stereoscopic views are side-by-side (SbS) and top-and-bottom (TaB). Currently, the defined Frame Packing Arrangement (FPA) SEI Message supports these two formats and some stereo packing ones. However, there are still no related standard packing formats for 3D video represent by color and depth information. This contribution proposes new FPA SEI Message to cover a series of centralized color-depth packing (CCDP) formats for representation of 3D videos with color and depth information. The 3D video encoded with the CCDP formats could be directly viewed in 2DTV displays without any pre-processing. Besides, the CCDP formats show better performances in both color-depth coding and multiviews rendering results compared to the frame-compatible color-and-depth SbS packing method based on HM 13.0. The proposed additional FPA SEI Message includes the CCDP formats for delivery of 3D video services in the existing HEVC and AVC video coding standards.

# Introduction

Current several well-known 2D frame compatible side-by-side, top-and-bottom, and checkerboard stereopacking formats exist allowing broadcasters to deliver 3D television programs with left and right views through their existing channels. However, the existing frame compatible 3D stereo packing formats have two problems preventing broadcasters from delivering more 3D contents: 1) They cannot directly support multiview naked-eye 3D displays with a reasonable cost unless the 3D displays implement real-time stereo matching and multiple depth image-based rendering algorithms; 2) The stereo packing formats produce serious annoying effects while showing two-view video in one frame for the traditional 2D displays. In the commercial market, stereo 3D TV sets only occupy a small fraction of TV sets. From broadcasting system point of view, the stereopacking 3D videos only can serve a small number of users, who have 3D TV sets such that the stereopacking 3D video services will be very difficult to attract commercial advertisements. The 3D naked-eye displays, which will be a preferable 3D solution, need additional depth information for generating multiview 3D contents. Besides, the various 3D naked-eye displays that have started to be available in the market require several different views. Most surveys show that the 3D naked-eye TVs coming with higher resolution definition LED displays will be the perfect solution for 3D displays.

To generate multi-view content, a possible transmission format of 3DTV broadcasting systems [1] could be the original 2D video associated with its depth information. With 2D-plus-depth format [2], [3], at the display side, the user can view multi-view content if the depth-image-based rendering (DIBR) technique is built in. However, the issues of the bandwidth limitation, frame compatibility and comfortable visualization for old 2D displays still need to be solved for 3D broadcasting services simply. With video and depth map at hands, the first problem to be solved is frame compatible format and comfortable visualization on 2D displays. The various versions of centralized color-depth packing (CCDP) formats and their objective and subjective quality measurements have been introduced in MPEG meeting [4] and AVS standard meetings [5]-[7]. In MPEG video standards, however, there is still no suitable SEI Message, which can properly define the packing formats of the 3D videos, which are delivered with color and depth information. Therefore, how to properly introduce color depth packing formats in MPEG video standards and how to successfully design efficient color depth packing formats, which could maintain the coding performance and comfortable view in existing 2D TV broadcasting systems is an important and urgent problem. In this contribution, in Section 2 we first overview the CCDP concept and then introduce the details of the packing and depacking procedures. In Section 3 we then suggest additional definition of FPA SEI Message to effectively include the CCDP formats. The coding performances of the CCDP formats comparing to that of color-and-depth SbS packing format are exhibited in Section 4.

# Proposed frame compatible CCDP formats

Fig. 1 shows the flowchart of for computation of the color-3/4 CCCD format [4], we could downscale the vertical resolution of color and depth frames into three fourth of the original resolution. Here, we use common bilinear method to resize color frame and depth frame. For the resized depth map, since only Y channel is utilized, the color-based subpixel arrangement can be adopted to make it more compact and further reduce to one third of the resized depth map. To achieve this, we embed three Y values of resized depth map into R, G, and B color subpixel channels in the vertical direction as shown in Fig. 2. The vertical resolution of reduced depth map becomes one third of the resized depth map. After RGB subpixel packing, the resized-and-colored depth map becomes one fourth of the original vertical resolution.

Fig. 1. Flowchart for computation of the proposed color-3/4 CCDP

Fig. 2. Procedure of color subpixels rearranging of depth frame into colored depth frame vertically

After color subpixels rearranging, the RGB color space is transformed to the coding YCbCr color space by the following formula,

YCbCr=0.25680.50410.0979−0.1482−0.29100.43920.4392−0.3678−0.0714RGB+16128128.

By the color subpixels rearrangement, the three fourth (3/4) of the resized depth map frame size now is reduced to one fourth (1/4) of the original vertical resolution. Packing three depth pixels into RGB color subpixels will keep the reduced-and-colored depth map closer to gray color since nearby depth values are almost the same.

Then, we split the reduced depth frame averagely into two separated parts and flip them. The splitting and flipping parts are designed for better line-up of reduced depth map and color frame to achieve better coding performance and better visualization in 2D displays. Finally, we get the packing image by combining the first half of reduced-and-colored depth frame, resized color frame, and the other half of reduced-and-colored depth frame from top to bottom sequentially.

The flow diagram of de-packing the color-3/4 CCDP format is shown in Fig. 3. After 4/3 upscaling depth frame in the vertical direction, we could optionally perform the depth enhancement to get more clear depth frame.

Fig. 3. Flow chart of the proposed Color-3/4 CCDP depacking procedure

For the extension of the color-3/4 CCDP format, we also can set it to be color-7/8 and color-15/16 CCDP formats, by downscaling the color frame sizes into 7/8 and 15/16 vertical resolutions while the depth frame sizes should be downscaling to 3/8 and 3/16 vertical resolutions, respectively. After the color subpixels rearrangements, the horizontal sizes of the reduced depth maps will become 1/8 H and 1/16 H, respectively. After the combination of the first half of reduced depth frame, resized color frame, and the other half of reduced depth frame from top to bottom, Fig. 4 exhibits the outlooks of the packed frames for color-3/4, color-7/8, and color-15/16 CCDP formats. It is noted that the reduced depth could be arranged in left and right side, while the downsizing of color and depth frames and reduction of depth frame are all down in horizontally.

Fig. 4. Packed frames for color-3/4, color-7/8, and color-15/16 CCDP formats

Fig. 5 shows the usage of the proposed frame-compatible color-3/4 CCDP format, which could be adopted in any TV broadcasting networks associated with paired HEVC or AVC video encoder and decoder. Without any supporting hardware, the traditional 2DTV can directly exhibit their 2D programs. Any simple build-in vertical expansion function in 2DTV display, the users can easily remove the depth portions. For 2DTV users, the color-15/16 will show the best visualization performance while 3/4-color CCDP format will have a slightly worse visualization. Even in 3/4-color CCDP format, the subjective test shows that the CCDP format outperforms the other well-known SbS and TaB stereopacking formats in 2D visualization [7].

Fig. 5. The scope of the proposed CCDP formats used in TV broadcasting networks

# Additional definitions of FPA SEI message for CCDP formats

In order to properly signal the CCDP formats and their setups, in subclause D.2.16 [8], we propose to further define the unused values of FPA SEI Message. Thus, we suggest SEI Message Syntax for frame packing arrangement could be further defined. In the following table, it shows the original Frame Packing Arrangement (FPA) SEI Message as:

Original Frame Packing Arrangement (FPA) SEI Message:

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| frame_packing_arrangement_persistence_flag | u(1) |
| } |  |
| upsampled_aspect_ratio_flag | u(1) |
| } |  |

For the CCDP formats, we could simply define some unused values of Frame Packing Arrangement (FPA) SEI Message as:

- We define 3 additional values of frame_packing_arrangement_type, in which we choose 65, 66, and 67 to represent color-3/4, color-7/8, and color-15/16 CCDP formats, respectively.

- We define 2 additional values of content_interpretation_type, in which we choose 33 and 34 to indicate if the depth performs vertical or horizontal downsizing and arranged in “top and bottom” or “left and right” positions, respectively.

- We use spatial_flipping_flag to indicate if the split half reduced depth is flipped or not.

- We use 8-bit frame_packing_arrangement_reserved_byte to express the other CCDP packing information as: 1-bit depth_subpixel_rotation_flag, 3-bit color_sampling_type, and 4-bit depth_sampling_type.

The detailed FPA SEI Message and descriptions by adding the additional definitions (with yellow color marked) of FPA SEI Message are described as:

Frame Packing Arrangement (FPA) SEI Message:

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| if( frame_packing_arrangement_type == 65 ¦¦ frame_packing_arrangement_type == 66 frame_packing_arrangement_type == 67) { |  |
| depth_subpixel_rotation_flag | u(1) |
| color_sampling_type | u(3) |
| if(depth_indication_flag ==33 ¦¦ frame_packing_arrangement_type == 34) |  |
| depth_sampling_type | u(4) |
| } |  |
| else { |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| } |  |
| frame_packing_arrangement_persistence_flag | u(1) |
| } |  |
| upsampled_aspect_ratio_flag | u(1) |
| } |  |

frame_packing_arrangement_type indicates the type of packing arrangement of the frames as specified in Table D-1.

Table D-1 – Definition of frame_packing_arrangement_type

| Value | Interpretation |
| --- | --- |
| 3 | Each component plane of the decoded frames contains a side-by-side packing arrangement of corresponding planes of two constituent frames as illustrated in Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found. |
| 4 | Each component plane of the decoded frames contains a top-bottom packing arrangement of corresponding planes of two constituent frames as illustrated in Error: Reference source not found and Error: Reference source not found. |
| 5 | The component planes of the decoded frames in output order form a temporal interleaving of alternating first and second constituent frames as illustrated in Error: Reference source not found. |
| 65 | Color and depth plane of the decoded frames contains a color-3/4 CCDP arrangement of corresponding planes of 2D and depth frames as illustrated in Figure D 10. |
| 66 | Color and depth plane of the decoded frames contains a color-7/8 CCDP arrangement of corresponding planes of 2D and depth frames as illustrated in Figure D 11. |
| 67 | Color and depth plane of the decoded frames contains a color-15/16 CCDP arrangement of corresponding planes of 2D and depth frames as illustrated in Figure D 12. |

Figure D-10 Frame arrangement of color-3/4 CCDP format

Figure D-11 Frame arrangement of color-7/8 CCDP format

Figure D-11 Frame arrangement of color-15/16 CCDP format

content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-12. Values of content_interpretation_type that do not appear in Table D-12 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore frame packing arrangement SEI messages that contain reserved values of content_interpretation_type.

For frame packing arrangement scheme equal to 3, or 4 and content_interpretation_type equal to 1 or 2, there are two constituent frames that are referred to as frame 0 and frame 1 For frame packing arrangement scheme equal to 65, 66 or 67 and content_interpretation_type equal to 33 or 34, color and depth frames that are arranged by the specific order.

Table D-12 – Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |
| 33 | Indicates that the packed frame is arranged in order of reduced_depth_1, resized_color, and reduced_depth_2 from top to bottom |
| 34 | Indicates that the packed frame is arranged in order of reduced_depth_1, resized_color, and reduced_depth_2 from left to right |

NOTE 1 – The value 2 or 34 for content_interpretation_type is not expected to be prevalent in industry use of this SEI message. However, the value was specified herein for purposes of completeness

When frame_packing_arrangement_type is not equal to 3, 4, 65, 66 or 67, it is a requirement of bitstream conformance that content_interpretation_type shall be equal to 0.

When frame_packing_arrangement_type is not equal to 3 or 4, the value 1 or 2 for content_interpretation_type is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 3 or 4, decoders shall ignore the value 1 or 2 for content_interpretation_type.

When frame_packing_arrangement_type is not equal to 65, 66 or 67, the value 33 or 34 for content_interpretation_type is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 65, 66 or 67, decoders shall ignore the value 33 or 34 for content_interpretation_type.

spatial_flipping_flag equal to 1, when frame_packing_arrangement_type is equal to 3 or 4, indicates that one of the two constituent frames is spatially flipped relative to its intended orientation for display or other such purposes.

When frame_packing_arrangement_type is equal to 3 or 4 and spatial_flipping_flag is equal to 1, the type of spatial flipping that is indicated is as follows:

–	If frame_packing_arrangement_type is equal to 3, the indicated spatial flipping is horizontal flipping.

–	Otherwise (frame_packing_arrangement_type is equal to 4), the indicated spatial flipping is vertical flipping.

spatial_flipping_flag equal to 1, when frame_packing_arrangement_type is equal to 65, 66 or 67 and content_interpretation_type equal 33 or 34, indicates that halves of the reduced colored depth frame is spatially flipped relative to its intended orientation for display or other such purposes.

When frame_packing_arrangement_type is equal to 65, 66 or 67, content_interpretation_type equal 33 or 34, and spatial_flipping_flag is equal to 1, the type of spatial flipping that is indicated is as follows:

–	If content_interpretation_type is equal to 33, the indicated spatial flipping is vertical flipping.

–	If content_interpretation_type is equal to 34, the indicated spatial flipping is horizontal flipping.

–	Otherwise (spatial_flipping_flag is equal to 0), the indicated spatial flipping is not necessary.

When frame_packing_arrangement_type is not equal to 3, 4, 65, 66 or 67, it is a requirement of bitstream conformance that spatial_flipping_flag shall be equal to 0. When frame_packing_arrangement_type is not equal to  3, 4, 65, 66 or 67, the value 1 for spatial_flipping_flag is reserved for future use by ITU-T | ISO/IEC. When frame_packing_arrangement_type is not equal to 3, 4, 65, 66 or 67, decoders shall ignore the value 1 for spatial_flipping_flag.

depth_subpixel_rotation_flag equal to 1 indicates that embedding the Y values into Cb and Cr channels in the depth frame is used a line-offset rotation method. depth_subpixel_arrangement_flag equal to 0 indicates that embedding the Y values into Cb and Cr channels in the depth frame is used by Y, Cb, Cr sequence order.

color_sampling_type indicates that the method of the color sampling is used by an encoder to indicate to a decoder as specified in Table D-13.

Table D-13 – Definition of color_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical resolution of color frame |

NOTE 7–When content_interpretation_type equal to 33, the value 0 for color_sampling_type, the color frame is down scale the vertical resolution by using bilinear method. When content_interpretation_type equal to 34, the value 0 for color_sampling_type, the color frame is down scaled in the horizontal resolution by using bilinear method.

Values of color_sampling_type that do not appear in Table D.13 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignoreframe packing arrangement SEI messages that contain reserved values of content_interpretation_type.

depth_sampling_type indicates that the method of the depth sampling is used by an encoder to indicate to a decoder as specified in Table D-14.

Table D-14 – Definition of depth_sampling_type

| Value | Interpretation |
| --- | --- |
| 0 | Encoder using bilinear method to downscale the vertical or horizontal resolution of depth frame |

NOTE 8–When content_interpretation_type equal to 33, the value 0 for depth_sampling_type, the depth frame is down scale the vertical resolution by using bilinear method. When content_interpretation_type equal to 34, the value 0 for depth_sampling_type, the depth frame is down scaled in the horizontal resolution by using bilinear method.

Values of depth_sampling_type that do not appear in Table D.14 are reserved for future specification by ITU-T |ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore frame packing arrangement SEI messages that contain reserved values of content_interpretation_type.

# Performance Measurements

To verify our CCDP method, we compare direct color-depth SbS packing format [2] with our packing format with three different DCCP formats (with color 3/4, 7/8, and 15/16), which are shown in Fig. 6. We adopted five 3D video sequences, which are Poznan Hall (1920x1088), Poznan Street (1920x1088), Kendo (1024x768), Balloons (1024x768), and Newspaper (1024x768) to conduct the objective measurements. Besides, our experiment is executed on HEVC HM13.0, and compression quality parameters are 22、27、32、37.

Color-3/4 CCDP

Color-7/8 CCDP

Color-15/16 CCDP

Color-depth SbS

Fig. 6. Schematics of different color and depth packaging formats

Tables 1 and 2 respectively show the performances of the averaged BDPSNR in dB and the averaged BDBR in % under three different testing conditions as All Intra (ai), Low Delay (ldp), and Random Access (ra). For comparison of coding performance of color and depth information separately, we calculate the PSNR of reconstructed color and depth frames respectively. To calculate BDPSNR and BDBR, the comparisons are based on the reconstructed versions of original view and original depth. Bitstreams are the consuming bits to restore reconstructed color image and depth map, respectively.

In order to compare the synthesis ability of different packing formats under the new viewpoint images at the receiver side, we utilize the reconstructed image and corresponding camera parameters to get a new viewpoint image through the depth image based rendering (DIBR), and fill the holes by directly extending nearby background.

Table 1. Average BDPSNR (dB) results for different sequences

Table 2. Average BDBR (%) results for different sequences

To verify our CCDP method, we also compare the coding performances of three different DCCP formats (with color 3/4, 7/8, and 15/16) coded by HEVC HM13.0 with the coding performances of 1-view and 1-depth coded by 3D-HEVC HTM8.0. The experiments executed on HM13.0 are with compression quality parameters are 22, 27, 32, and 37 while those executed on HTM8.0 are with compression quality parameters are 25, 30, 35, and 40.

Tables 3 and 4 respectively show the performances of the averaged BDPSNR in dB and the averaged BDBR in % under Random Access (ra) condition. For comparison of coding performance of color and depth information separately, we calculate the PSNR of reconstructed color and depth frames respectively. For the synthesis view, we used VSRS built-in 3D-HEVC HTM8.0 to get the new viewpoint images at the receiver side, while we utilize the depth image based rendering (DIBR), and fill the hole areas by directly extending nearby background. We found that the coding performances of the DCCP formats coded by HEVC HM13.0 are slightly worse than those of view and depth coded by 3D-HEVC HTM8.0. It is noted that we did not perform and depth enhancement method in the enlisted simulation results.

Table 3. Average BDPSNR (dB) results for different sequences

Table 4. Average BDBR (%) results for different sequences

# Conclusions

In this contribution, the objective measurements of the proposed color-3/4, color-7/8, and color-15/16 CCDP methods are comparing to that of the frame compatible color-and-depth (SbS) packing format. To include the CCDP formats and their settings in the AVC and HEVC standard, we propose SEI message syntax for frame packing arrangement. The proposed CCDP format attains better image quality in the receiving side compared to the frame compatible color-and-depth (SbS) packing format for both 2D-TV and 3D-TV displays.

# References

[1]	Digital Video Broadcasting (DVB); Plano-stereoscopic 3DTV; ETSI TS 101 547 V1.1.1

[2]	Philips 3D Solutions: 3D Interface Specifications White Paper

[3]	A. Redert, R.-P. Berretty, C. Varekamp, O. Willemsen1, J. Swillens, H. Driessen , "Philips 3D Solutions: From Content Creation to Visualization", Third International Symposium on 3D Data Processing, Visualization, and Transmission (3DPVT'06).

[4] J.-F. Yang, H.-M. Wang and, A.-T. Chiang, “2D Backwards Compatible Centralized Color-Depth Packing,”, Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, the 6th Meeting: Document: JCT3V-F0087, Geneva, November 2013.

[5] 杨家辉, 王泓民, 江宜安, 廖恪应, “结合颜色和深度信息之2D回放兼容集中颜色包装格式,” AVS-M3225, 数字音视频编解码技术标准工作组, 第四十七次會議，北京 中國, 2013年12月。(Proposed Color-3/4 CCDP format)

[6] 杨家辉, 王泓民, 江宜安, 廖恪应, “结合颜色和深度信息之2D回放兼容颜色置中包装格式于不同包装比例之测试分析,” AVS-M3302, 数字音视频编解码技术标准工作组, 第四十八次會議，深圳 中國, 2014年4月。(Proposed color-3/4, color-7/8, and color-15/16 CCDP formats and their subjective quality measure)

[7] 杨家辉, 王泓民, 江宜安, 廖恪应, “结合颜色和深度信息之2D回放兼容颜色置中包装格式于不同包装比例之主客觀測試分析,” AVS-M3375, 数字音视频编解码技术标准工作组, 第四十九次會議，大連 中國, 2014年6月。(Provided objective and subjective measurements for three CCDP formats)

[8] 杨家辉, 王泓民, 廖恪应, 江宜安, “结合颜色和深度信息之2D回放兼容颜色置中包装格式于AVS2文本标志之建议,” AVS-M3472, 数字音视频编解码技术标准工作组, 第五十次會議，南京 中國, 2014年10月。(Suggested New AVS Syntax Message to include CCDP formats)

[8]	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent), Document no. JCTVC-L1003, Jan. 2013.

# Patent rights declaration(s)

National Cheng Kung University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).