Coding unit syntax:

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) /* Intra BC*/ |  |
| intra_bc_mask_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bc_mask_flag[ x0 ][ y0 ] ) { |  |
| intra_bc_mask_mode_minus1[ x0 ][ y0 ] | u(2) |
| intra_bc_mask_size_flag1[ x0 ][ y0 ] | u(1) |
| if( intra_bc_mask_size_flag1[ x0 ][ y0 ] ) |  |
| intra_bc_mask_size_flag2[ x0 ][ y0 ] | u(1) |
| } |  |
| bvd_coding( x0, y0, 2 ) |  |
| bvp_flag[ x0 ][ y0 ] | ae(v) |
| else { /* MODE_INTER */ |  |

Prediction unit semantics (add the following):

intra_bc_mask_flag[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is coded in intra block copying mode with sample mask mode maskModeIntraBc larger than zero.  intra_bc_mask_flag [ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is coded without a mask and maskModeIntraBc is set to zero.  When not present, the value of intra_bc_mask_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

intra_bc_mask_mode_minus[ x0 ][ y0 ] indicates the applied maskModeIntraBc shall be equal to intra_bc_mask_mode_minus [ x0 ][ y0 ] plus one. When not present, maskModeIntraBc is inferred to be equal to 0.

intra_bc_mask_size_flag1[ x0 ][ y0 ] equal to 0 indicates the width of the masking area for luminance component maskWidthIntraBcLuma is equal to two. intra_bc_mask_size_flag1[ x0 ][ y0 ] equal to 1 indicates that maskWidthIntraBcLuma is defined by intra_bc_mask_size_flag2[ x0 ][ y0 ].

intra_bc_mask_size_flag2[ x0 ][ y0 ] equal to 1 indicates the maskWidthIntraBcLuma is equal to one. intra_bc_mask_size_flag2[ x0 ][ y0 ] equal to 0 indicates maskWidthIntraBcLuma is equal to three.

Specification of general decoding process for coding units coded in intra prediction mode:

### 8.4.1 General decoding process for coding units coded in intra prediction mode

…

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0 ), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for block vector components and masking information in intra block copying prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and variable log2CbSize, variable maskModeIntraBc and variable maskWidthIntraBcLuma as inputs, and the output being bvIntra, ibcMask and ibcMaskLoc.

- If cu_residual_act_flag[ xCb ][ yCb ] is equal to 1, the following applies:

–		For the variable cIdx proceeding over the values 0..2, the following ordered steps apply:

  - The variable comp is set equal to (!cIdx ? L : (cIdx = =1 ? Cb : Cr). [Ed. (GJS): Suggest reformulating to avoid this usage formulation and this variable name.]

  - The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable cIdx, the variable ibcMask, the variable ibcMaskLoc and variable controlParaACT equal to 1 as inputs, and the output is the residual sample array resSamplescomp.

–		The residual modification process for residual blocks using adaptive colour transform as specified in subclause Error: Reference source not found is invoked with the variable blkSize set equal to nCbS, the (nCbS)x(nCbS) array rY set equal to resSamplesL, the (nCbS)x(nCbS) array rCb set equal to resSamplesCb, and the (nCbS)x(nCbS) array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the (nCbS)x(nCbS) arrays resSamplesL, resSamplesCb and resSamplesCr. [Ed. (GJS): Paragraph hanging indentation alignment problem.]

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable cIdx set equal to 0, and variable controlParaACT set to (cu_residual_act_flag[ xCb ][ yCb ] ? 2 : 3 ), the variable ibcMask and the variable ibcMaskLoc as inputs, and the output is a modified reconstructed picture before deblocking filtering. [Ed. (GJS): Variable name controlParaACT seems to not follow editorial convention, by having several capitalized letters in a row. Do we do that elsewhere?]

…

### 8.4.4 Derivation process for block vector components and masking information in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

- variables maskModeIntraBc and maskWidthIntraBcLuma.

Output of this process is the (nCbS)x(nCbX) array of block vectors bvIntra, the mask sample location array ibcMaskLoc and mask array ibcMask.

…

The variable maskWidthIntraBc is set equal to maskWidthIntraBcLuma / cSc, where:

cSc = ( cIdx == 0 ) ? 1 : ( maskModeIntraBc == 1 || maskModeIntraBc == 3 ) ? SubHeightC : SubWidthC

Each value of ibcMask[ xPb + x ][ yPb + y ] with x = 0.. nPbSw − 1, y = 0.. nPbSh – 1 are set to 0.

If maskModeIntraBc is larger than 0, the following additional steps apply:

–	Variables maskFirstX, maskLastX,  maskFirstY, maskLastY, maskRefX, maskRefY are set as follows:

firstX = maskModeIntraBc == 2 ? nPbSw - maskWidthIntraBc: 0;

firstY = maskModeIntraBc == 3 ? nPbSh - maskWidthIntraBc: 0;

lastX  = maskModeIntraBc == 4 ? maskWidthIntraBc - 1 : nPbSw - 1;

lastY  = maskModeIntraBc == 1 ? maskWidthIntraBc - 1 : nPbSh - 1;

–	Variables maskRefX and maskRefY are defined as a function of variables maskModeIntraBc, firstX, firstY, lastX and lastY according to the table below:

| maskModeIntraBc | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| maskRefX | nPbSw / 2 | maskFirstX | nPbSw / 2 | maskLastX |
| maskRefY | maskLastY | nPbSh / 2 | maskFirstY | nPbSh / 2 |

–	The components of the mask location array ibcMaskLoc are set to:

ibcMaskLoc[ xPb + x ][ yPb + y ][ 0 ] =  xPb + maskRefX with x = 0..nPbSw-1, y = 0..nPbSh-1

ibcMaskLoc[ xPb + x ][ yPb + y ][ 1 ] =  yPb + maskRefY with x = 0..nPbSw-1, y = 0..nPbSh-1

- Each value of ibcMask[ xPb + x ][ yPb + y ] with x = maskFirstX..maskLastX, y = maskFirstY.. maskLastY are set to 1.

##### Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable bvIntra specifying the block copying vector,

–	a variable ibcMaskLoc specifying the mask sample location,

–	a variable ibcMask specifying the array of masked samples,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-1)

Depending upon the values of trafoDepth, PartMode and nTbS, the following applies:

–	If trafoDepth is equal to 0, PartMode is not equal to PART_2Nx2N, and nTbS is greater than 4, the following applies, for the variable tempIdx proceeding over the values 0..3:

–	The variable nTbS1 is set equal to nTbS / 2.

–	The variable xTb1 is set equal to xTb0 + nTbS1 * ( blkIdx % 2 ).

–	The variable yTb1 is set equal to yTb0 + nTbS1 * ( blkIdx / 2 ).

–	The general intra block copying process as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable nTbS set equal to nTbS1, the variable bvIntra, the variable trafoDepth is set equal to 1, and the variable cIdx as inputs, and the output is an (nTbS1) x (nTbS1) array predSamples. [Ed.: This should be tempSamples, and then copied into predSamples]

–	Otherwise, the variable bv representing the block vector for prediction in full-sample units is derived as follows:

–	If cIdx is not equal to 0, trafoDepth is equal to 0, and nTbS is equal to 4, the following applies:

–	If ChromaArrayTyp is equal to 1, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY + 4 ].

–	Otherwise, if ChromaArrayType is equal to 2, bv is set equal to bvIntra[ xTbY + 4 ][ yTbY ].

–	bv[ 0 ] = Max( −( xTbY % CtbSizeY + CtbSizeY ), bv[ 0 ] ) >> ( SubWidthC − 1 )

–	bv[ 1 ] = Max( −( yTbY % CtbSizeY ), bv[ 1 ] ) >> ( SubHeightC − 1 )

–	Otherwise, the following applies:

bv[ 0 ] = bvIntra[ xTbY ][ yTbY ][ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-2)

bv[ 1 ] = bvIntra[ xTbY ][ yTbY ][ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-3)

–	If maskModeIntraBc is larger than 0, the mask sample location ( xMaskCmp, yMaskCmp ) representing the mask sample location vector in full-sample units is derived as follows:

xMaskCmp = ibcMaskLoc[ xTb0 ][ yTb0 ][ 0 ] + bv[ 0 ]	(8-xx)

yMaskCmp = ibcMaskLoc[ xTb0 ][ yTb0 ][ 1 ] + bv[ 1 ]	(8-xx)

–	The (nTbS)x(nTbS) array of predicted samples, with x, y = 0..nTbS − 1, is derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-4)

Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ]

Depending on the value of ibcMask[ xTb0  + x ][ yTb0  + y ], the following applies:

–	If ibcMask[ xTb0  + x ][ yTb0  + y ] is equal to 0, each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

–	Otherwise, each sample at the location ( xMaskCmp, yMaskCmp ) is assigned to predSamples[ x ][ y ].

Parsing process

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| prediction_unit( ) | merge_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc[ ][ ] | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0[ ][ ], ref_idx_l1[ ][ ] | Error: Reference source not found |  | 0..1 | 2..3 |
|  | intra_bc_mask_flag[ ][ ] | Table 9-38'' | 0…2 | 0…2 | 0…2 |
|  | mvp_l0_flag[ ][ ], mvp_l1_flag[ ][ ], bvp_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
|  | intra_bc_mask_flag [ ][ ] | FL | cMax = 1 |
|  | bvp_flag[ ][ ] | FL | cMax = 1 |

Table 9-38’' – Values of initValue for ctxIdx of intra_bc_mask_flag

| Initialization variable | ctxIdx of intra_bc_mask_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

| Table 9-43 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| intra_bc_flag | 0 | na | na | na | na | na |
| intra_bc_mask_flag | 0 | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| … |  |  |  |  |  |  |