| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0033 |
| --- | --- |

| Title: | Non-CE2: Intra block vector coding for small PUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema, Miska Hannuksela | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to limit the number of full "unrestricted" intra block copy vectors signalled for a CU to one. In the case the CU is split to multiple PUs, the first PU will have a traditional block vector associated with it, while the rest of the block vectors are restricted to either use the vertical component of the primary block vector or have a zero vertical component. It is reported that the proposed approach achieves -0.4% and -0.5% bitrate reductions for lossy coding of the RGB text and graphics content at 1080p and 720p resolutions, respectively. It is further reported that the reference encoder runtime is improved by 4% in the all intra test due to the significantly reduced search area for the small PUs.

# Introduction

The vertical components of the intra block copy vectors tend to be highly correlated due to the typical content characteristics (e.g. majority of the text is usually aligned on horizontal rows). Thus, it is quite unlikely that optimal block vectors for 8x4, 4x8 and 4x4 PUs within the same CU have more than one non-zero vertical component. Based on this finding, it is proposed to limit the number of full block vectors for CU to one (the primary block vector) and indicate for the rest of the vectors whether their vertical component is the same as the vertical component of the primary block vector or whether it should be set to zero. Coding of the horizontal components of the block vectors are unaltered in order to allow the same amount of flexibility to represent horizontal displacements as SCM 2.0 has.

# Proposed signaling of 2NxN, Nx2N and NxN block vectors

The proposed signalling of the block vectors can be expressed in pseudo code as follows:

for each PU in CU

if (first PU ) {

decode 2D_differential_block_vector

set block_vector = 2D_block_vector_predictor + 2D_differential_block_vector

}

else {

if ( vertical block vector component of first PU != 0 )

decode use_vert_of_first_PU_flag

else

set use_vert_of_first_PU_flag = 0

decode 1D_differential_block_vector

set block_vector_ver = use_vert_of_first_PU_flag ? vertical component of block vector of the first PU : 0

set block_vector_hor = 1D_block_vector_predictor_hor + 1D_differential_block_vector

}

}

# Proposed encoder operation

The block vector search of SCM 2.0 is used as it is for the first PU in the CU. For the remaining PUs we perform one or two 1D searches depending whether there are one or two possible candidates for the vertical component of those block vectors (one candidate if the vertical component of the block vector of the first PU was zero, two candidate otherwise).

It should be noted that the proposed encoding algorithm is suboptimal as the only non-zero candidate for the vertical block vector component is fixed by a search done for the first PU. Better results should be obtainable by removing this restriction.

# Objective coding results

## Results for the common conditions / lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.5% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.6% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.6% | -0.7% |
| YUV, mixed content, 1440p | -0.3% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.6% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.2% | 0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 96 % |  |  |
| Dec Time[%] | 100 % |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, text & graphics with motion,720p | -0.3% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | -0.3% | -0.6% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 98 % |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 100 % |  |  |

## Results for the common conditions / lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.5% | -0.9% | 2.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | -0.2% | 0.8% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.1% | 0.7% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.4% | -0.9% | 2.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | -0.2% | 1.1% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.2% | 0.4% |
| YUV, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94 % |  |  |  |
| Dec Time[%] | 100 % |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.2% | -0.7% | 1.5% |
| RGB, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% | 0.7% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.3% | -0.6% | 1.4% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.3% | 0.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99 % |  |  |  |
| Dec Time[%] | 100 % |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.7% | 0.7% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | -0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | -0.6% | 0.7% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | -0.2% | 0.5% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99 % |  |  |  |
| Dec Time[%] | 100 % |  |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).