| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0042 |
| --- | --- |

| Title: | AhG6: Inclusion of 4:2:0 screen content in the HEVC SCC common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz, Rajan Joshi, Vadim Seregin, Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The tools adopted and proposed for HEVC SCC are not being tested for 4:2:0 material. This document proposes to introduce 4:2:0 screen content sequences in the HEVC SCC common test conditions to assure that the SCC tools properly work for this chroma format. Intra block copying is specified and working for 4:2:0 and providing BD-rate gains of 18.6% for all-intra. Palette is currently undefined for 4:2:0 in the draft specification.

# Introduction

The philosophy of HEVC Range Extensions (RExt) is that of nested profiles. Tools developed for 4:4:4 are also supported and were tested for 4:2:0 (and 4:2:2 & 4:0:0) during the HEVC RExt development.

The working assumption for HEVC SCC is the same as HEVC RExt. However, the active support for 4:2:0 was dropped when moving to the SCC project. At this point, some HEVC SCC tools are not working for 4:2:0 (and possibly, other non-4:4:4 chroma formats).

# Proposal

It is proposed to introduce 4:2:0 screen content sequences (e.g., the class F used during the HEVC development) in the common test conditons set of HEVC SCC to assure that the SCC tools and the new proposals are working properly for this case.

It is noted that the ability to support other non-4:4:4 chroma formats (e.g., 4:2:2 or 4:0:0) might be of interest.

## Status of Intra Block Copying for 4:2:0

The HEVC SCC Draft 1 specification and the SCM2.0 software are supporting intra BC. The gains of intra BC in SCM2.0 for class F using the CTC are as follows:

|  | Class F |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| All Intra | -18.6% | -18.4% | -18.7% |
| Random Access | -13.6% | -14.1% | -14.2% |
| Low delay B | -7.4% | -8.1% | -7.5% |

## Status of Palette for 4:2:0

The palette mode in the current specification and software is not supporting 4:2:0, even though the adopted palette was supporting it. We propose to include the palette support for 4:2:0 in the specification and software.

## Status of Adaptive Color Transform for 4:2:0

The current specification and software are not supporting adaptive color transform for 4:2:0. This tool is generally intended for 4:4:4, as is the case of cross-component prediction. Therefore, no change seems necessary for it.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).