| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0044 |
| --- | --- |

| Title: | AHG13: Chroma deblocking filter control for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony corporation |  |  |

_____________________________

# Abstract

This contribution proposes chroma deblocking filter control for SCC.

First, disabling deblocking filter was tested under SCC CTC. The existing pps_deblocking_filter_disabled_flag was set to 1. It is reported that the bd-rate difference {G/Y, B/U, R/V} is {+1.1, -0.6, -0.2}/{+1.2, -1.6, -1.5}/{+0.8, -1.5, -1.4} for AI/RA/LB, respectively.

Then, disabling chroma deblocking filter was examined. The decoder was changed to skip the chroma process while the luma process was applied as the anchor. It is reported the result is {0.0, -0.6, -0.3}/{0.0, -1.7, -1.6}/{0.0, -1.6, -1.5}.

Visual evaluation was also conducted. It is reportedly shown that the deblocking filter affects the character recognition result of the coded picture.

Finally, the text is proposed to control the chroma deblocking process. The syntax and semantics are presented.

# Introduction

In the 18th Sapporo meeting, the experts discussed if the current deblocking filter process is necessary to modify for SCC, and AHG13 has been established.

This contribution studies the effect of deblocking filter for SCC, especially for the chroma process.

The contribution is structured as follows. Section 2 reports test conditions and the objective results. Section 3 reportedly shows the visual evaluation results. Section 4 proposes the text. Finally, Section 5 presents the conclusion.

# Experiment

## Test1: disabling deblocking filter with the existing control flag

In Test1, pps_deblocking_filter_disabled_flag is set to 1 under SCC CTC [2], in order to measure the impact of the deblocking filter for SCC. It is noted that the flag disables both the luma and the chroma process.

The result for lossy case is shown in Table 1. The lossless case is not considered where the deblocking filter doesn’t affect the result except the control flag signaling.

Below is the detail condition of Test1.

Software: HM-15.0+RExt-8.0+SCM-2.0 (original)

Configuration parameter: modified from CTC as below

DeblockingFilterControlPresent : 1

LoopFilterDisable : 1

Table 1 Impact of disabling deblocking filter

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -2.7% | -2.3% |
| RGB, text & graphics with motion,720p | 0.1% | -1.8% | -1.9% |
| RGB, mixed content, 1440p | 0.1% | -1.0% | -1.1% |
| RGB, mixed content, 1080p | 0.4% | -0.9% | -0.9% |
| RGB, Animation, 720p | 0.0% | -0.4% | -0.7% |
| RGB, camera captured, 1080p | 4.7% | 0.4% | 4.3% |
| YUV, text & graphics with motion, 1080p | -0.1% | -3.2% | -2.9% |
| YUV, text & graphics with motion,720p | 0.5% | -0.8% | -1.5% |
| YUV, mixed content, 1440p | 0.8% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | 0.7% | -1.2% | -1.0% |
| YUV, Animation, 720p | 1.3% | 1.5% | 0.9% |
| YUV, camera captured, 1080p | 5.0% | 2.7% | 4.4% |
| Average | 1.1% | -0.6% | -0.2% |
| Average (except camera captured) | 0.4% | -1.1% | -1.2% |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -3.4% | -2.9% |
| RGB, text & graphics with motion,720p | -0.2% | -2.7% | -3.0% |
| RGB, mixed content, 1440p | -0.1% | -2.2% | -2.4% |
| RGB, mixed content, 1080p | 0.2% | -2.3% | -2.4% |
| RGB, Animation, 720p | -0.4% | -1.0% | -1.1% |
| RGB, camera captured, 1080p | 6.5% | 0.2% | 1.6% |
| YUV, text & graphics with motion, 1080p | -0.4% | -3.8% | -3.5% |
| YUV, text & graphics with motion,720p | 0.4% | -1.7% | -2.7% |
| YUV, mixed content, 1440p | 1.1% | -1.5% | -1.6% |
| YUV, mixed content, 1080p | 0.8% | -2.8% | -2.6% |
| YUV, Animation, 720p | 1.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 6.1% | 2.1% | 2.6% |
| Average | 1.2% | -1.6% | -1.5% |
| Average (except camera captured) | 0.2% | -2.1% | -2.2% |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -3.1% | -2.6% |
| RGB, text & graphics with motion,720p | -0.7% | -1.8% | -2.3% |
| RGB, mixed content, 1440p | -0.4% | -1.8% | -1.8% |
| RGB, mixed content, 1080p | -1.2% | -2.4% | -2.4% |
| RGB, Animation, 720p | -1.4% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 5.9% | 0.2% | 0.5% |
| YUV, text & graphics with motion, 1080p | -0.7% | -3.3% | -2.9% |
| YUV, text & graphics with motion,720p | 0.4% | -1.4% | -2.1% |
| YUV, mixed content, 1440p | 1.5% | -1.1% | -1.2% |
| YUV, mixed content, 1080p | 0.1% | -4.4% | -4.5% |
| YUV, Animation, 720p | 1.0% | 0.8% | 1.0% |
| YUV, camera captured, 1080p | 6.0% | 0.6% | 1.1% |
| Average | 0.8% | -1.5% | -1.4% |
| Average (except camera captured) | -0.3% | -1.9% | -1.9% |

## Test2: disabling chroma deblocking filter by the decoder change

In Test2, decoder/encoder process is modified to skip the chroma deblocking filter process. The luma process is applied as the anchor. The result is shown in Table 2.

Below is the detail condition of Test2.

Software: modified as below on top of HM-15.0+RExt-8.0+SCM-2.0

In l. 231- xDeblockCU () @ TComLoopFilter.cpp,

// comment: the chroma process is simply skipped

// xEdgeFilterChroma   ( pcCU, uiAbsZorderIdx, uiDepth, edgeDir, iEdge );

Configuration parameter setting: same as SCC CTC

Table 2 Impact of disabling chroma deblocking filter

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -2.7% | -2.3% |
| RGB, text & graphics with motion,720p | 0.0% | -1.8% | -1.9% |
| RGB, mixed content, 1440p | 0.0% | -1.0% | -1.1% |
| RGB, mixed content, 1080p | 0.0% | -0.9% | -0.9% |
| RGB, Animation, 720p | 0.0% | -0.4% | -0.7% |
| RGB, camera captured, 1080p | 0.0% | 0.4% | 4.3% |
| YUV, text & graphics with motion, 1080p | 0.0% | -3.2% | -2.9% |
| YUV, text & graphics with motion,720p | 0.0% | -0.9% | -1.5% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -1.2% | -1.0% |
| YUV, Animation, 720p | 0.0% | 1.5% | 0.8% |
| YUV, camera captured, 1080p | 0.1% | 2.6% | 4.3% |
| Average | 0.0% | -0.6% | -0.3% |
| Average (except camera captured) | 0.0% | -1.1% | -1.2% |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -3.3% | -2.8% |
| RGB, text & graphics with motion,720p | 0.0% | -2.7% | -3.0% |
| RGB, mixed content, 1440p | -0.1% | -2.2% | -2.4% |
| RGB, mixed content, 1080p | 0.0% | -2.4% | -2.4% |
| RGB, Animation, 720p | 0.0% | -1.1% | -1.1% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | 1.7% |
| YUV, text & graphics with motion, 1080p | -0.2% | -3.6% | -3.3% |
| YUV, text & graphics with motion,720p | 0.0% | -1.8% | -2.7% |
| YUV, mixed content, 1440p | 0.1% | -1.9% | -1.8% |
| YUV, mixed content, 1080p | 0.0% | -3.1% | -2.8% |
| YUV, Animation, 720p | 0.1% | -0.5% | -0.3% |
| YUV, camera captured, 1080p | 0.1% | 1.9% | 2.2% |
| Average | 0.0% | -1.7% | -1.6% |
| Average (except camera captured) | 0.0% | -2.2% | -2.3% |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -2.8% | -2.4% |
| RGB, text & graphics with motion,720p | 0.0% | -1.7% | -2.1% |
| RGB, mixed content, 1440p | 0.2% | -1.6% | -1.7% |
| RGB, mixed content, 1080p | -0.1% | -2.6% | -2.5% |
| RGB, Animation, 720p | 0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.9% |
| YUV, text & graphics with motion, 1080p | -0.2% | -3.1% | -2.8% |
| YUV, text & graphics with motion,720p | 0.1% | -1.5% | -2.2% |
| YUV, mixed content, 1440p | 0.0% | -1.6% | -1.7% |
| YUV, mixed content, 1080p | -0.3% | -5.3% | -4.9% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.3% | 0.8% |
| Average | 0.0% | -1.6% | -1.5% |
| Average (except camera captured) | -0.1% | -2.0% | -2.0% |

## Discussion

Table 1 shows roughly 5% loss in G/Y component for the camera captured content. It would say disabling the deblocking filter for the natural images is not reasonable.

For the screen content, Table 1 shows roughly 1% gain in G component of RGB coding but 1% loss in Y of YUV. Simply disabling the deblocking filter would make the coding efficiency worse in YUV coding.

Table 1 also presents roughly 1-2% gain in B/U and R/V except for the camera captured content. It is highly likely that the current chroma deblocking filter process does not suit for the screen content in terms of the coding efficiency.

Table 2 shows the impact of disabling chroma deblocking filter. Roughly 1-2% gain is observed in B/U and R/V as Table 1 while no coding loss is observed in G/Y.

From Table 2, it would say disabling the chroma deblocking filter for the screen content is a reasonable approach. The properties of the approach are summarized as below.

- Controllable based on the content characteristic (e. g. screen content)

- No low-level change (e.g. bS decision)

- Independent from the coding mode decision by the encoder

# Visual evaluation

This section reports visual evaluation result of the coded picture discussed in the previous section. All cited pictures are attached in the contribution package.

## Artifact in the CG area

Figure 1 shows a part of the original “sc_programming_1280x720_60_8bit.rgb”, frame 222. The left center 512x512 pixel is cropped where a polygon 3D model is displayed.

Figure 2 reportedly shows the same part of the CTC result. The condition is AI-Main and QP37. In the green highlighted circle, a triangle-shape artifact is observed.

Figure 3 and Figure 4 reportedly show the result of Test1 and Test2, respectively. The artifact is slightly visible in Figure 3 and not visible in Figure 4. Also, the gradation area below the circle seems smoother in Figure 4 compared to Figure 3, thanks to the luma deblocking process. It would say disabling chroma deblocking filter is effective for both objective and subjective result in the case.

Figure 1 Original picture

In the green circle, triangle-shape artifact is observed in the chroma component.

Figure 2 CTC result (AI-Main, QP37)

In the green circle, triangle-shape artifact is slightly observed.

Figure 3 Test1 result (AI-Main, QP37)

In the green circle, no triangle-shape artifact is observed.

Figure 4 Test2 result (AI-Main, QP37)

## Character recognition in the text area

Figure 5 shows a part of the original “MissionControlClip3_1920x1080_60p_8b444.rgb”, frame 525. The bottom center 512x512 pixel is cropped where many characters are displayed. In the green highlighted circle, the characters are recognized as “40, 43, 40, 39, 38, 37” from top to bottom.

Figure 6 reportedly shows the same part of the CTC result. The condition is LB-Main and QP32. The characters in the circle seem “40, 42, 40, 30, 39, 37”, where the red number is different from the original due to the coding artifact.

Figure 7 and Figure 8 reportedly show the result of Test1 and Test2, respectively. The characters in the circle seem “40, 43, 40, 39, 38, 37” as the original. Thus, it is pointed that the deblocking filter process affected the character recognition result in CTC.

However, it is also noted that the visual difference is due to not only the filtering process but also the coding mode decision by the encoder. Since the deblocking filter is in-loop process, the change affects the coding mode in the future picture. Figure 9 reportedly shows the intermediate pixels of the CTC result. The before-deblocking pixel is dumped out for the study. As Figure 6, the characters seem “40, 42, 40, 30, 39, 37.” It shows the character difference is not due to the deblocking filter in the picture.

The point here is that the in-loop filter not optimized for the screen content would affect the visual quality, especially for the character recognition result.

In the green circle, the characters are

“40, 43, 40, 39, 38, 37”

from top to bottom.

Figure 5 Original picture

In the green circle, the characters seem

“40, 42, 40, 30, 39, 37”

where the red numbers are different from the originals.

Figure 6 CTC result (LB-Main, QP32)

In the green circle, the characters seem

“40, 43, 40, 39, 38, 37”

as the original numbers.

Figure 7 Test1 result (LB-Main, QP32)

In the green circle, the characters seem

“40, 43, 40, 39, 38, 37”

as the original numbers.

Figure 8 Test2 result (LB-Main, QP32)

In the green circle, the characters seem

“40, 42, 40, 30, 39, 37”

Figure 9 before deblocking pixels of Figure 6

# Proposal

Considering the results in Section 2 and 3, this contribution proposes chroma deblocking filter disabling flag in SPS, PPS or/and Slice header. The syntax and semantics on top of the latest draft text [3] are attached.

In short, the proposed flag controls the chroma process only if the existing slice_deblocking_filter_disabled_flag is 0.

Finer granularity (e.g. CU-level control) might be desirable for the mixed content, while the decoder increases the burden to check the condition. It is proposed to discuss such capability as well.

It is noted that Ver1/RExt provides the deblocking control functionality in PPS and Slice level. Similar approach would be desirable for the consistency.

##### Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| sps_deblocking_chroma_filter_disabled_flag | u(1) |
| } |  |

sps_deblocking_chroma_filter_disabled_flag equal to 1 specifies that the deblocking filter process is not applied to the chroma component in the CVS. sps_deblocking_chroma_filter_disabled_flag equal to 0 specifies that the deblocking filter process is applied to the chroma component in the CVS. When not present, sps_deblocking_chroma_filter_disabled_flag is inferred to 0.

##### Picture parameter set scc extensions syntax

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_deblocking_ chroma_filter_disabled_flag | u(1) |
| } |  |
| } |  |
| } |  |

pps_deblocking_chroma_filter_disabled_flag equal to 1 specifies that the deblocking filter process is not applied to the chroma component in the CVS. pps_deblocking_chroma_filter_disabled_flag equal to 0 specifies that the deblocking filter process is applied to the chroma component in the CVS. When not present, pps_deblocking_chroma_filter_disabled_flag is inferred to sps_deblocking_chroma_filter_disabled_flag.

##### Picture parameter set scc extensions syntax

| slice_segment_header_scc_extensions( ) { | Descriptor |
| --- | --- |
| if( deblocking_filter_override_flag ) { |  |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_deblocking_ chroma_filter_disabled_flag | u(1) |
| } |  |
| } |  |
| } |  |

slice_deblocking_chroma_filter_disabled_flag equal to 1 specifies that the deblocking filter process is not applied to the chroma component in the CVS. slice_deblocking_chroma_filter_disabled_flag equal to 0 specifies that the deblocking filter process is applied to the chroma component in the CVS. When not present, slice_deblocking_chroma_filter_disabled_flag is inferred to pps_deblocking_chroma_filter_disabled_flag.

8.7.2 Deblocking filter process

8.7.2.1 General

…

When slice_deblocking_filter_disabled_flag of the current slice is equal to 0, for each coding unit with luma coding block size log2CbSize and location of top-left sample of the luma coding block ( xCb, yCb ), the vertical edges are filtered by the following ordered steps:

…

- The vertical edge filtering process for a coding unit as specified in subclause Error: Reference source not found is invoked with the reconstructed picture sample array prior to deblocking recPictureL, and when ChromaArrayType is not equal to 0, the arrays recPictureCb, and recPictureCr, the luma location ( xCb, yCb ), the luma coding block size log2CbSize, and the array verBs as inputs, and the modified reconstructed picture sample array recPictureL, and when ChromaArrayType is not equal to 0 and slice_deblocking_chroma_filter_disabled_flag is equal to 0, the arrays recPictureCb, and recPictureCr as outputs.

…

When slice_deblocking_filter_disabled_flag of the current slice is equal to 0, for each coding unit with luma coding block size log2CbSize and location of top-left sample of the luma coding block ( xCb, yCb ), the horizontal edges are filtered by the following ordered steps:

…

- The horizontal edge filtering process for a coding unit as specified in subclause Error: Reference source not found is invoked with the modified reconstructed picture sample array recPictureL, and when ChromaArrayType is not equal to 0, the arrays recPictureCb, and recPictureCr, the luma location ( xCb, yCb ), the luma coding block size log2CbSize and the array horBs as inputs, and the modified reconstructed picture sample array recPictureL, and when ChromaArrayType is not equal to 0 and slice_deblocking_chroma_filter_disabled_flag is equal to 0, the arrays recPictureCb, and recPictureCr as outputs.

# Conclusion

In the contribution, a functionality to control the chroma deblocking filter process is proposed.

In Section 2, the impact to disable chroma process is reported as {0.0, -0.6, -0.3}/{0.0, -1.7, -1.6}/{0.0, -1.6, -1.5} for AI/RA/LB, respectively.

In Section 3, it is reportedly shown that the controlling deblocking filter improves the visual evaluation results.

In Section 4, the syntax and the semantics are proposed.

It is believed the proposal is beneficial for encoders which can control the chroma deblocking filter based on the content characteristics without significant impact on the low-level process. It is also emphasized that the proposal is independent from the coding mode so that it is applicable where the conventional (Ver1/RExt) coding mode is selected by the encoders.

# Reference

- JCTVC-R1000, “Meeting Report of 18th JCT-VC Meeting”

- JCTVC-R1015, “Common Test Conditions for Screen Content Coding”

- JCTVC-R1005, “HEVC Screen Content Coding Draft Text 1”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).