| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0047_r3 |
| --- | --- |

| Title: | Non-CE 6: Modifications of copy-left and copy-above modes in index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Jih-Sheng Tu, 2Chun-Lung Lin, Chao-Hsiung Hung, 3Ching-Chieh Lin, Yao-Jen Chang 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1sunrisejstu@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

In past meetings, several palette redundancy removal methods are provided. This proposal presents  redundancy removal methods for the palette coding; in certain cases, the index value of current pixel can be deduced impossible to have the same index value of its neighbouring pixels. Therefore the index coding bits can be saved. In this proposal, it also presents a copy-above mode modification method. The test results report that the proposed methods achieve 0.3% gain for 1080p text and graphics test sequences in the full frame intra BC test condition at All Intra lossy case.

# Introduction

In current design, palette coding provides three modes to code color index map : “Copy Left Run” mode (CL)、“Copy Above Run” mode (CA)、“Escape” mode (ES). In this proposal, a redundancy check of index coding of CL, and a modification process of CA, and a redundancy check of run coding of CL are applied.

## Redundancy Removal in Index Coding

In order to code an index, a redundancy check is applied as follows:

If the current pixel is coded as CL and its left neighbouring pixel is also coded as CL, then the index values of these two positions can not be the same, otherwise they would be merged into a longer run. Then, if the index values of the left neighbouring pixel and the above neighbouring pixel are different, also if the run value of the current pixel is smaller than 3, then it can be considered that the index values of the current pixel and its above neighbouring pixel would not be the same, otherwise the current pixel would be coded as CA.

## Modification of Copy Above mode

If the left neighbouring pixel of current pixel is coded as CL, and if the above neighbouring pixel of current pixel has the same index value with the left neighbouring pixel, it is impossible for the current pixel to be coded as CA. In this circumstance, a modification is proposed to set a pseudo index value to replace the above neighbouring pixel virtually when testing CA is valid or not.

## Redundancy Removal in Run Coding

In order to code a run, a redundancy check is applied as follows:

- If there are only two major colours in the current CU

- If the current pixel and its left neighbouring pixel are coded as CL

- If the index values of the left neighbouring pixel and the above neighbouring pixel are different

When those conditions are true, it can be considered that the run value of the current pixel must great than or equal to 3.

# The Proposed Methods

## Copy Left Method

Fig. 2-1. Processing condition of copy left method. A and C are all coded by CL. Thus C and A must be different.

Assuming the current, the left, and the above indexes are C, A, and B, as shown in Fig. 2-1. A and C are all coded by CL. Thus C must be different from A. The run value of coding C is RC. To code index C, following conditions is considered:

1. If C and B are different, then RC can be any value.

2. If C and B are the same, then RC must ≥ 3. Otherwise C will be coded by CA.

Thus, if the RC ≥ 3, there might be an indicator to signal whether C and B are the same or not.

Fig. 2-2. Flow chart of copy left method.

The flow chart of this method is shown in Fig. 2-2. Where A and C are all coded by CL.

1. If RC < 3, C and B must be different, otherwise C would be coded by CA.

Since C must not equal to A and B, the current index C may be coded with a shorter codeword. The coding syntax in this case is shown as Fig. 2-3.

Coding C: Copy-left + reduced index C + run

Fig. 2-3. Code index C in a shorter codeword.

2. If RC ≥ 3, C and B may be the same.

  - If C and B are the same, code index 0 instead of C and code an indicator after coding run RC. Fig. 2-4 illustrates this case; the indicator would be coded as 1 to signal the decoder that the current index should be recovered by the above index. The coding syntax in this case is shown as Fig. 2-5.

Fig. 2-4. An example of run value ≥ 3 and the current index is the same with the above index.

Fig. 2-5. Set indicator = 1 when the current index is identical to the above index.

  - If C and B aren’t the same :

    - When the codeword of the current index = 0, the indicator would be coded as 1. The coding syntax in this case is shown as Fig. 2-6.

Fig. 2-6. Set indicator = 0 when the current index isn’t identical to the above index and the current index is coded as 0.

    - In other cases, there is no need to code an indicator. The coding syntax in this case is shown as Fig. 2-3.

## Modification of Copy Above mode

Fig. 2-7. Processing condition of modified copy above mode method. The left and the current indexes are coded as CL.

In Fig. 2-7, since the left and the above indexes are the same, the current index C can’t be coded as CA. In this proposed method, the above index is replaced by a default value B virtually, which is different from A. If B is equal to C, the current index could be coded as CA.

## Coded run value reduction

Fig. 2-8. Processing condition of reduced run method. A and B are CL.

Fig. 2-9. Flow chart of reduced run method.

Palette coding skips coding color index if the CU contains only two major colors. For the CU containing two major colors, let the current, the left, and the above indexes are B, A, B, as shown in Fig. 2-8. Current index B and its left index A are all coded by CL. Thus the run value RB of B must ≥ 3. RB can be coded as RB–3 for saving coding bits, as shown in Fig. 2-9.

# Simulation Results

The proposed scheme is implemented on SCM2.0.The modification improves 0.3% gain in average for 1080p text and graphics test sequences. The experimental conditions are subject to the test conditions issued by JCT-VC in the 18th meeting [1]. Table 1 to Table 6 demonstrate the coding performance under full frame intra BC against SCM2.0 anchor. The encoding time may not be accurate.

Table 1. Proposed vs. Anchor  (AI-lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Proposed vs. Anchor  (RA-lossy)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | 0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 82% |  |  |
| Dec Time[%] | 94% |  |  |

Table 3. Proposed vs. Anchor  (LB-lossy)

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.1% |
| YUV, text & graphics with motion,720p | -0.2% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 92% |  |  |

Table 4. Proposed vs. Anchor  (AI-lossless)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 85% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 5. Proposed vs. Anchor  (RA-lossless)

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 87% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 6. Proposed vs. Anchor  (LB-lossless)

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusion

# This contribution proposes a modification to improve the coding of the run and the index of the palette mode coding. The proposed methods do not increase computing complexity and show additional coding gains. It is recommended to adopt these modifications.

# Reference

- H. Yu, R. Cohen, K. Rapaka, J. Xu, " Common Test Conditions for Screen Content Coding," Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1015, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

In section 2.1, there is a requirement for that proposed method. The requirement is that the left neighbouring pixel must be coded as CL. Here, a modification of the aforementioned requirement is proposed. In this modification the left neighbouring can be codec as CL or ES, the rest processes described in section 2.1 do not have to change. The experimental result of this modification is shown as Table 7.

Table 7. Proposed(supplemental) vs. Anchor  (AI-lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 65% |  |  |
| Dec Time[%] | 33% |  |  |

Runtime measurements are not accurate.