| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0048 |
| --- | --- |

| Title: | CE6 Test B2: Encoder modification of palette coding for escape pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang Chao-Hsiung Hung 2Chun-Lung Lin 3Ching-Chieh Lin Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | 1+886-3-5918774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This proposal reports the CE6 Test B2 results based on SCM 2.0 reference software. There are four encoding modes, including the mode with original size of major color table, the mode with the size minus one, the mode with the size minus two, and the mode with the size minus three. The best encoding mode is selected based on the rate-distortion cost. The major colors discarded from the original major color table will be represented as escape pixels. Compared with SCM 2.0 reference software under the conditions of AI-lossy coding and IBC full frame search, the BDrates are -0.3% and -0.2% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” respectively.

# Introduction

In palette coding [1], some of major colors may only represent few pixels in the CU. It could be a waste of bits to add the major color indexes for representing these few pixels. Therefore, JCTVC-R0075 proposes a method to remove some major colors which only represent few pixels. To further study the technical parts and evaluate its performance on top of SCM-2.0, the method is tested in CE6 Test B.2 [3].

This contribution presents the results of CE6 Test B.2. There are four encoding modes, including the mode with original size of major color table, the mode with the size minus one, the mode with the size minus two, and the mode with the size minus three. The best encoding mode is selected based on the rate-distortion cost. The major colors discarded from the original major color table will be represented as escape pixels.

# Proposed Algorithm

JCTVC-R0075 proposes to move a major color index to escape index if the pixel number related to the major color is ≤ 2. This fixed threshold method is not suitable for SCM 2.0 due to different palette table generation algorithm. In this proposal, an adaptive algorithm is proposed to move major colors to escape pixels by discarding some major colors, whose pixel numbers are fewest. For the palette table before reordering of palette prediction, each of the last three major colors is decided whether to be moved to escape pixels. The following shows the function of this method:

Assuming the original table size is M , four cases are selected by RD cost:

Mode 1: the last one major color is moved to escape pixels, where the table size is M-1.

Mode 2: the last two major colors are moved to escape pixels, where the table size is M-2.

Mode 3: the last three major colors are moved to escape pixels, where the table size is M-3.

Mode 4: No major colors are moved to escape pixels, where the table size is M.

The best encoding mode is selected based on the rate-distortion cost.

# Simulation Results

The proposed method is simulated based on SCM 2.0 reference software under the common test condition [3], where the search range of IBC is full frame. Table 1 shows the lossy results. Anchor is the results of SCM 2.0. The BD rates are -0.3% and -0.2% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” under All intra, lossy, cases.

Table 1. Lossy results.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.6% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.5% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | -0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -1.1% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

For the palette table before reordering of palette prediction, this proposal removes some major colors from the last three major colors. The pixels represented by the discarded major colors will be coded as escape pixels. Compared with SCM 2.0 reference software under the conditions of AI-lossy and full frame IBC search, the BDrates are -0.3% and -0.2% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” respectively.

# Reference

- P. Onno, X. Xiu, Y. W. Huang, and R. Joshi, “Suggested combined software and text for run-based palette mode,” 18th JCT-VC meeting, JCTVC-R0348, Sapporo, Japan, July 2014.

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, and C. H. Hung, “Non-SCCE3: Encoder modification of palette mode for escape pixels,” 18th JCT-VC meeting, JCTVC-R0075, Sapporo, Japan, June 2014.

- Y. W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, and Z. Ma, “Description of core experiment 6 (CE6): palette mode improvement,” 18th JCT-VC meeting, JCTVC-R1106, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

ITRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)