#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| slice_bi_sample_mode_enable_flag | ae(v) |
| … |  |
| … |  |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

7.4.7.1 General slice segment header semantics

slice_bi_sample_mode_enable_flag specifies whether bi-color mode can be used for intra mode coding. If slice_bi_sample_mode_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no bi-color mode is used in decoding of the current picture. Otherwise (slice_bi_sample_mode_enabled_flag is equal to 1), bi-color mode may be used in decoding of the current picture. When not present, the value of slice_bi_sample_mode_enabled_flag is inferred to be equal to 0.

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_bi_sample_mode_enable_flag ) |  |
| bi_sample_flag[ x0 ][ y0 ] | ae(v) |
| if(bi_sample_flag[ x0 ][ y0 ] ) { |  |
| bi_color_coding ( x0, y0, nCbS, nCbS ) |  |
| } else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } // bi_sample_flag[ x0 ][ y0 ] |  |
| } |  |

| bi_color_coding(x0, y0, CbWidth, CbHeight) { | Descriptor |
| --- | --- |
| bi_sample_idx[ x0 ][ y0 ] | ae(v) |
| scanPos = 0 |  |
| palette_size = 2 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| if (scanPos >= nCbS && previous_run_type_flag != COPY_ABOVE_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = INDEX_MODE |  |
| previous_run_type_flag = palette_run_type_flag[ xC ][ yC ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) { |  |
| adjustedIndexMax = palette_size – 1 |  |
| adjustedRefIndex = palette_size |  |
| } |  |
| if( scanPos > 0 && palette_run_type_flag[ xC-1 ][ yC ] = = INDEX_MODE && palette_escape_val_flag [ xC-1 ][ yC ] ! = 1) { |  |
| adjustedIndexMax -= 1 |  |
| adjustedRefIndex = paletteMap[xC-1][yC] |  |
| } |  |
| if (palette_run_type_flag[ xC-1 ][ yC ] = = COPY_ABOVE_MODE && palette_escape_val_flag [ xC ][ yC – 1 ] ! =1) { |  |
| adjustedIndexMax -= 1 |  |
| adjustedRefIndex = paletteMap[xC][yC-1] |  |
| } |  |
| if( adjustedIndexMax > 0) |  |
| palette_index | ae(v) |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_ABOVE_MODE ¦ ¦ ( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE && palette_index < palette_size ) ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| while ( runPos <= palette_run ) { |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) { |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| else |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC – 1 ] |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

bi_sample_flag [ x0 ][ y0 ] equal to 1 specifies that bi-color mode is enabled for the current coding unit. bi_sample_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not coded by bi-color mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When bi_sample_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

bi_sample_idx[ x0 ][ y0 ] specifies which bi-color candidate index from the bi-color candidate list is selected, where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

##### 8.x.x.x.x. Specification of bi-color prediciton process

Inputs to this process are:

–	a sample location ( xCurr, yCurr ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 2 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS − 1 and x = 0..nTbS  − 1, y = −1, are derived as follows:

–	The neighbouring location ( xNb, yNb ) is specified by:

( xNb, yNb ) = ( xCurr + x, yCurr + y )	(8-1)

The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) and the neighbouring luma location ( xNb, yNb ) as inputs, and the output is assigned to availableN.

Each sample p[ x ][ y ] is derived as follows:

- If the variable availableN is equal to FALSE, the sample p[ x ][ y ] is marked as "not available for intra prediction".

- Otherwise, the sample p[ x ][ y ] is derived as the sample at the location ( xNb, yNb).

availableFlagAn, availableFlagBn, availableFlagA1, availableFlagB1, availableFlagB0 are set equal to TRUE.

predSample An is set equal to p[ −1 ][ nTbS −1  ].

predSample Bn is set equal to p[ nTbS −1  ][ −1  ].

predSample A1 is set equal to p[ −1 ][ 0 ].

predSample B1 is set equal to p[ 0 ][ −1 ].

predSample B0 is set equal to p[ −1 ][ −1 ].

When one or more of the following conditions are true, availableFlagAn is set equal to FALSE:

- p[ −1 ][ nTbS −1  ] is marked as "not available for intra prediction".

When one or more of the following conditions are true, availableFlagBn is set equal to FALSE:

- p[ nTbS −1  ][ −1 ] is marked as "not available for intra prediction".

- availableFlagAn is equal to TRUE and predSample Bn is equal to predSample An.

When one or more of the following conditions are true, availableFlagB1 is set equal to FALSE:

- p[ 0 ][ −1 ] is marked as "not available for intra prediction".

- availableFlagAn is equal to TRUE and predSample B1 is equal to predSample An.

- availableFlagBn is equal to TRUE and predSample B1 is equal to predSample Bn.

When one or more of the following conditions are true, availableFlagA1 is set equal to FALSE:

- p[ −1 ][ 0 ] is marked as "not available for intra prediction".

- availableFlagAn is equal to TRUE and predSample A1 is equal to predSample An.

- availableFlagBn is equal to TRUE and predSample A1 is equal to predSample Bn.

- availableFlagB1 is equal to TRUE and predSample A1 is equal to predSample B1.

When one or more of the following conditions are true, availableFlag B0is set equal to FALSE:

- p[ −1 ][ −1 ] is marked as "not available for intra prediction".

- availableFlagAn is equal to TRUE and predSample B0is equal to predSample An.

- availableFlagBn is equal to TRUE and predSample B0is equal to predSample Bn.

- availableFlagB1 is equal to TRUE and predSample B0is equal to predSample B1.

- availableFlagA1 is equal to TRUE and predSample B0 is equal to predSample A1.

The sample candidate list, sampleCandList[i][j], is constructed as follows, where j = 0, 1:

i = 0
if( availableFlagAn or availableFlagBn)
sampleCandList[ i++ ][0] = An

sampleCandList[ i++ ][1] = Bn
if( availableFlagAn or availableFlagB1)
sampleCandList[ i++ ][0] = An

sampleCandList[ i++ ][1] = B1
if( availableFlagBn or availableFlagB1)
sampleCandList[ i++ ][0] = Bn

sampleCandList[ i++ ][1] = B1
if( availableFlagAn or availableFlagA1)
sampleCandList[ i++ ][0] = An

sampleCandList[ i++ ][1] = A1

if( availableFlagBn or availableFlagA1)
sampleCandList[ i++ ][0] = Bn

sampleCandList[ i++ ][1] = A1

if( availableFlagB1 or availableFlagA1)
sampleCandList[ i++ ][0] = B1

sampleCandList[ i++ ][1] = A1

if( availableFlagAn or availableFlagB0)
sampleCandList[ i++ ][0] = An

sampleCandList[ i++ ][1] = B0

if( availableFlagBn or availableFlagB0)
sampleCandList[ i++ ][0] = Bn

sampleCandList[ i++ ][1] = B0

if( availableFlagB1 or availableFlagB0)
sampleCandList[ i++ ][0] = B1

sampleCandList[ i++ ][1] = B0

if( availableFlagA1 or availableFlagB0)
sampleCandList[ i++ ][0] = A1

sampleCandList[ i++ ][1] = B0

numSampleMergeCand is set equal to i.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

predSamples[ x ][ y ] = sampleCandList[ single_sample_idx][palette_index[x][y]]

The values of the residual samples resSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

predSamples[ x ][ y ] = 0

The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location (xCurr, yCurr ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.