| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0050 |
| --- | --- |

| Title: | Non-CE8: Combination of CE8 Test A and CE8 Test B for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Ching-Chieh Lin, 3Chun-Lung Lin, Jih-Sheng Tu, and Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This contribution presents the results of combination of CE8 Test A and CE8 Test B. The combination scheme includes two different color modes, one is single color mode which uses one neighboring color to smooth area in the coding unit (CU), and the other is bi-color mode which uses a pair of neighboring colors to smooth area in the CU. For the latter mode, a bi-index map is established to represent each pixel in the CU by a binary index. Compared to the anchor SCM2.0, the combination scheme reportedly shows that 0.7%, 0.9%, 0.4% and 0.7% BD-rate savings under All-Intra Lossy-coding conditions are achieved for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

# Introduction

Because screen content video contains a lot of smooth area with similar sample values, the samples within the smooth area could share single sample value. Therefore, a single color intra mode is proposed in JCTVC-R0198 to reconstruct the coding unit (CU) with single sample value, and is further tested in CE8 Test B [4]. However, it may require two colors to fully represent the samples of the smooth area within the CU. Therefore, a bi-color intra mode is proposed in JCTVC-R0112 to reconstruct the CU with two diverse sample values, and is further tested in CE8 Test A [4].

This contribution presents the results of combination of CE8 Test A and CE8 Test B. The combination scheme includes two different color modes, one is single color mode which uses one neighboring color to smooth area in the CU, and the other is bi-color mode which uses a pair of neighboring colors to smooth area in the CU. For the latter mode, a bi-index map is established to represent each pixel in the CU by a binary index.

# The proposed combination scheme

Figure 1. One or two of the neighboring samples inside the grey region are selected to reconstruct the current coding unit.

A CU-level flag is signaled to indicate the enabling/disabling of the combination scheme. if the combination scheme is enabled, a flag is further signaled to indicate which color mode out of single color mode and bi-color mode is used. If the single color mode is selected, the most frequently occurred sample from the neighboring reference block is used to smooth area in the CU. If the bi-color mode is selected, the two most frequently occurred samples from the neighboring reference block are used to smooth area in the CU. The neighboring reference block is inside the above, left and above left CUs of the current CU. As shown in Figure 1, the grey region is the neighboring reference block, where the parameter H is the height of the grey area in above CU and the above-left CU, the parameter W is the width of the grey area in the left CU and the above-left CU, and W and H are set as 2 in this contribution. For the bi-color mode, a bi-index map is established to represent each pixel in the CU by a binary index. The copy left run mode and copy above run mode appeared in JCTVC-R0348 are applied in the combination scheme to code the bi-index map.

# Simulation results

The experiments were conducted on top of SCM-2.0 in the common test conditions [5]. The lossy coding results of the proposed combination scheme under three different conditions (AI, RA, and LB) are summarized in Table 1. As shown in the table, the combination scheme provides 0.3-0.9% BD-rate savings for different classes of screen content sequences under AI condition. The lossless coding results are shown in Table 2, where 0.x-0.x% bitrate savings are shown for different classes of screen content sequences under AI condition.

Table 1. Proposed combination scheme vs. SCM-2.0 (Lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.3% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | -0.4% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.2% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.2% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.5% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Proposed combination scheme vs. SCM-2.0 (Lossless)

# Conclusions

In this contribution, a combination scheme of CE8 Test A and CE8 Test B is presented to efficiently code the smooth area within a picture. The simulation results reportedly show that the combination scheme gains 0.7%, 0.9%, 0.4% and 0.7% BD-rate savings under All-Intra Lossy-coding conditions for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively. It is recommended to adopt the combination scheme into the screen content coding for its gain and low additional cost.

# References

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, and C. H. Hung, "Non-SCCE3: Bi-color intra mode for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0112, June 2014.

- P. Lai, S. Liu, Y.-W. Chen, and S. Lei, “Non-SCCE3 Test D.1 Modified single color mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0198, June 2014.

- R. Cohen, T.-D. Chuang, C.-C. Lin, and K. Rapaka, “Description of core experiment 8 (CE8): Uni- and bi-colour mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1108, Jul. 2014.

- P. Onno, X. Xiu, Y. W. Huang, and R. Joshi, “Suggested combined software and text for run-based palette mode,” 18th JCT-VC meeting, JCTVC-R0348, Sapporo, Japan, July 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” 18th JCT-VC meeting, JCTVC-R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).