#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| slice_combined_sample_mode_enable_flag | ae(v) |
| … |  |
| … |  |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

7.4.7.1 General slice segment header semantics

slice_combined_sample_mode_enable_flag specifies whether the combination scheme can be used for intra mode coding. If slice_combined_sample_mode_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no combined color modes are used in decoding of the current picture. Otherwise (slice_combined_sample_mode_enabled_flag is equal to 1), one of color modes may be used in decoding of the current picture. When not present, the value of slice_combined_sample_mode_enabled_flag is inferred to be equal to 0.

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_combined_sample_mode_enable_flag ) |  |
| combined_sample_flag[ x0 ][ y0 ] | ae(v) |
| if(combined_sample_flag[ x0 ][ y0 ] ) { |  |
| combined_color_coding ( x0, y0, nCbS, nCbS ) |  |
| } else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } // combined_sample_flag[ x0 ][ y0 ] |  |
| } |  |

| combined_color_coding(x0, y0, CbWidth, CbHeight) { | Descriptor |
| --- | --- |
| combined_sample_idx[ x0 ][ y0 ] | ae(v) |
| If(combined_sample_idx[ x0 ][ y0 ] >= 1) { |  |
| scanPos = 0 |  |
| palette_size = 2 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| if (scanPos >= nCbS && previous_run_type_flag != COPY_ABOVE_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = INDEX_MODE |  |
| previous_run_type_flag = palette_run_type_flag[ xC ][ yC ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) { |  |
| adjustedIndexMax = palette_size – 1 |  |
| adjustedRefIndex = palette_size |  |
| } |  |
| if( scanPos > 0 && palette_run_type_flag[ xC-1 ][ yC ] = = INDEX_MODE && palette_escape_val_flag [ xC-1 ][ yC ] ! = 1) { |  |
| adjustedIndexMax -= 1 |  |
| adjustedRefIndex = paletteMap[xC-1][yC] |  |
| } |  |
| if (palette_run_type_flag[ xC-1 ][ yC ] = = COPY_ABOVE_MODE && palette_escape_val_flag [ xC ][ yC – 1 ] ! =1) { |  |
| adjustedIndexMax -= 1 |  |
| adjustedRefIndex = paletteMap[xC][yC-1] |  |
| } |  |
| if( adjustedIndexMax > 0) |  |
| palette_index | ae(v) |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_ABOVE_MODE ¦ ¦ palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE && ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| while ( runPos <= palette_run ) { |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| else |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC – 1 ] |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

combined_sample_flag [ x0 ][ y0 ] equal to 1 specifies that multiple color mode is enabled for the current coding unit. combined_sample_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not coded by multiple color mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When combined_sample_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

combined_sample_idx[ x0 ][ y0 ] specifies which color mode out of multiple color modes is used to code the coding unit. If combined_sample_idx is M, the coding unit is coded by M- color mode. x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

##### 8.x.x.x.x. Specification of combined single color and bi-color prediciton process

Inputs to this process are:

–	a sample location ( xCurr, yCurr ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 – 1, and x = 0..nTbS * 2 − 1, y = −1

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 2 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS − 1 and x = 0..nTbS  − 1, y = −1, are derived as follows:

–	The neighbouring location ( xNb, yNb ) is specified by:

( xNb, yNb ) = ( xCurr + x, yCurr + y )	(8-1)

The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) and the neighbouring luma location ( xNb, yNb ) as inputs, and the output is assigned to availableN.

Each sample p[ x ][ y ] is derived as follows:

- If the variable availableN is equal to FALSE, the sample p[ x ][ y ] is marked as "not available for intra prediction".

- Otherwise, the sample p[ x ][ y ] is derived as the sample at the location ( xNb, yNb).

The neighbouring colors c[i] from neighbouring samples p[ x ][ y ] are derived as follows:

j = 0, 1, …, i:

If(p[ x ][ y ] != c[j])

c[i++] = p[x][y]

The numbers N[i] of neighbouring samples p[ x ][ y ] equal to c[i] are derived as follows with x = −1, y = −1..nTbS − 1 and x = 0..nTbS  − 1, y = −1:

N[i] = 0

If(p[ x ][ y ] == c[i])

N[i]++

The most frequently occurring neighboring colors c[i] in nTbS * 2 neighbouring samples p[ x ][ y ] are derived as follows:

j = 1, 2, … the number of  c[i]:

If(N[j] > N[0])

temp = c [0]

c[0] = c[j]

c[j] = temp

N[0] = N[j]

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

predSamples[ x ][ y ] = (combined_sample_idx[ x0 ][ y0 ]) ? c[palette_index[x][y]] : c[0]

The values of the residual samples resSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

resSamples[ x ][ y ] = predSamples[ x ][ y ]

The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location (xCurr, yCurr ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.