| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0051 |
| --- | --- |

| Title: | Non-CE8: Multi-color intra mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Ching-Chieh Lin, 3Chun-Lung Lin, Jih-Sheng Tu, and Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2JackLin@itri.org.tw 3Chunlung@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

JCTVC-S0050 proposes a combined single color and bi-color mode, which is a combination of two CE8 tests. This contribution proposes a multi-color intra mode, which is an extension of JCTVC-S0051. The multi-color intra mode includes single color mode, bi-color mode and the color modes with more colors. The colors of the multi-color intra mode are selected from the neighboring reference block. For the modes with the number of colors over one, an index map is established to represent each pixel in the CU by an index. Compared to the anchor SCM2.0, the proposed mode reportedly shows that 0.9%, 1.0%, 0.7% and 0.8% BD-rate savings under All-Intra Lossy-coding conditions are achieved for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

# Introduction

Because screen content video contains a lot of smooth area with similar sample values, the samples within the smooth area could share single sample value. Therefore, a single color intra mode is proposed in JCTVC-R0198 to reconstruct the coding unit (CU) with single sample value, and is further tested in CE8 Test B [4]. However, it may require two colors to fully represent the samples of the smooth area within the CU. Therefore, a bi-color intra mode is proposed in JCTVC-R0112 to reconstruct the CU with two diverse sample values, and is further tested in CE8 Test A [4].

This contribution proposes a multi-color intra mode which includes single color mode, bi-color mode and the color modes with more colors. The colors of the multi-color intra mode are selected from the neighboring reference block. For the modes with the number of colors over one, an index map is established to represent each pixel in the CU by an index. Note that the proposed mode is an extension of JCTVC-S0050.

# The proposed mode

Figure 1. One or over one neighboring samples inside the grey region are selected to reconstruct the current coding unit.

A CU-level flag is signaled to indicate the enabling/disabling of the multi-color intra mode, which includes the single color mode, bi-color mode, and the modes with more colors. If the multi-color intra mode is enabled, a flag is further signaled to indicate which color mode is used. If the single color mode is selected, the most frequently occurred sample from the neighboring reference block is used to smooth area in the CU. If the bi-color mode is selected, the two most frequently occurred samples from the neighboring reference block are used to smooth area in the CU. If the N color mode with N colors is selected, the N most frequently occurred samples from the neighboring reference block are used to smooth area in the CU, where N could be any size. The neighboring reference block is inside the above, left and above left CUs of the current CU. As shown in Figure 1, the grey region is the neighboring reference block, where the parameter H is the height of the grey area in above CU and the above-left CU, the parameter W is the width of the grey area in the left CU and the above-left CU, and W and H are set as 2 in this contribution. For the color mode with the number of colors over one, an index map is established to represent each pixel in the CU by an index. The copy left run mode and copy above run mode appeared in JCTVC-R0348 are applied in the combination scheme to code the index map.

# Simulation results

The experiments were conducted on top of SCM-2.0 in the common test conditions [5]. The lossy coding results of the proposed mode under three different conditions (AI, RA, and LB) are summarized in Table 1. As shown in the table, the proposed mode provides 0.3-1.0% BD-rate savings for different classes of screen content sequences under AI condition. The lossless coding results are shown in Table 2, where 0.x-0.x% bitrate savings are shown for different classes of screen content sequences under AI condition.

Table 1. Proposed combination scheme vs. SCM-2.0 (Lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.4% | -0.3% | -0.4% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.8% | -0.8% | -0.7% |
| YUV, mixed content, 1440p | -0.4% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.4% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.4% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | -0.3% | -0.5% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.4% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2. Proposed combination scheme vs. SCM-2.0 (Lossless)

# Conclusions

In this contribution, a multi-color intra mode, an extension of JCTVC-S0050, is presented to efficiently code the screen-content CU. The simulation results reportedly show that the combination scheme gains 0.9%, 1.0%, 0.7% and 0.8% BD-rate savings under All-Intra Lossy-coding conditions for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively. It is recommended to adopt the proposed mode into the screen content coding for its gain and low additional cost.

# References

- Y. J. Chang, C. L. Lin, C. C. Lin, J. S. Tu, and C. H. Hung, "Non-SCCE3: Bi-color intra mode for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0112, June 2014.

- P. Lai, S. Liu, Y.-W. Chen, and S. Lei, “Non-SCCE3 Test D.1 Modified single color mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0198, June 2014.

- R. Cohen, T.-D. Chuang, C.-C. Lin, and K. Rapaka, “Description of core experiment 8 (CE8): Uni- and bi-colour mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1108, Jul. 2014.

- P. Onno, X. Xiu, Y. W. Huang, and R. Joshi, “Suggested combined software and text for run-based palette mode,” 18th JCT-VC meeting, JCTVC-R0348, Sapporo, Japan, July 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” 18th JCT-VC meeting, JCTVC-R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).