| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0052 |
| --- | --- |

| Title: | Non-CE6: Escape coded pixel prediction using previous escape coded pixels for palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jiangli Ye , Jianqing Zhu 29F,Kerry Parkside,NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5811 threerabbit@cn.fujitsu.com zhujianqing@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution document, escape coded pixels are predicted using escape coded pixels those occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, there is a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 3.4% and 5.1% on TGM RGB and YUV (AI), 2.6% and 3.1% on TGM RGB and YUV (RA), 2.5% and 2.8% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128;

The other is use max index value to indicate escape mode. Reported test results show gain of 2.0% and 3.4% on TGM RGB and YUV (AI), 1.6% and 2.1% on TGM RGB and YUV (RA), 1.6% and 1.9% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128.

# Introduction

In the 18th JCT-VC meetings in Sapporo, a base software [1] containing palette mode was suggested. This contribution is an improvement on top of the base software.

In base software, for escape coded pixels in palette mode, truncated coding is used directly on pixel value, no prediction is applied. In this contribution, escape coded pixel is predicted using escape coded pixels those occurred in previous palette-coded CUs.

# Proposed method

## Escape coded pixel prediction using previous escape coded pixels

- Escape coded pixel prediction

If escape coded pixel exists, a CU level flag is used to indicate whether prediction is used for escape coded pixels in current CU.

If the prediction is used in current CU, a flag is used for each escape coded pixel (containing 3 components) to indicate whether it is predicted or not.

If the escape coded pixel is predicted, an index is coded to indicate which element in escape predictor is its prediction. Otherwise (it is not predicted), the escape pixel is coded same as in base software.

- Escape predictor construction

Firstly, the decoded escape pixels are stuffed to form a list of escape pixels (each escape pixel in the list is unique and in decoding order), maximum number of escape pixels(MAX_ESC_SIZE) in the list is 64;

After decoding of each palette-coded CU, the list of escape pixels is stuffed into escape predictor in similar way as stuffing process of palette predictor.

There is MAX_ESC_PRED_SIZE to indicate maximum size of escape predictor.

# Experimental results

The anchor is the base software of SCM2 [1]. Test condition is common conditions for screen content coding test in R1015[2]. In following test results, MAX_ESC_PRED_SIZE is 128.

## Result of escape mode using flag coding

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.4% | 3.2% | 2.0% | 4.7% |
| RGB, text & graphics with motion,720p | 1.2% | 0.9% | 0.1% | 2.1% |
| RGB, mixed content, 1440p | 0.5% | 0.5% | 0.4% | 0.7% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.1% | 5.2% | 2.5% | 9.2% |
| YUV, text & graphics with motion,720p | 1.2% | 0.9% | 0.2% | 2.4% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.2% | 0.7% |
| YUV, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.6% | 1.9% | 1.1% | 2.8% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.1% | 3.7% | 1.3% | 7.4% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.1% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.5% | 1.6% | 0.5% | 2.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.8% | 3.3% | 0.6% | 7.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

Table 2. Simulation results in lossy coding mode

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 106% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.2% | 0.8% | 0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.5% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

## Result of escape mode using index coding

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.0% | 1.9% | 1.2% | 2.9% |
| RGB, text & graphics with motion,720p | 0.9% | 0.7% | 0.1% | 1.9% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.4% | 3.5% | 2.0% | 6.0% |
| YUV, text & graphics with motion,720p | 1.0% | 0.7% | 0.1% | 2.1% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |  |
| Dec Time[%] | 107% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.1% | 0.7% | 1.8% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.1% | 2.5% | 1.1% | 4.8% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 109% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.0% | 0.5% | 1.7% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.9% | 2.2% | 0.6% | 4.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 105% |  |  |  |

Table 2. Simulation results in lossy coding mode

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 108% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | -0.2% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 110% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.5% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.2% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.2% | 0.4% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 109% |  |  |

# References

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 2 Encoder Description (SCM 2),” JCTVC-R1014, Sapporo, JP, July. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, July. 2014.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).