| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0056 |
| --- | --- |

| Title: | Non-CE2 : Slice-level Intra block copy enabling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Lim Jonghyun Ma Yongjo Ahn Donggyu Sim 507 Bima-gwan, 20 Gwagwun-ro Nowon-gu, Seoul, South Korea | Tel: Email: | +82-2-940-5470 {limwoong, mday519, yongjoahn, dgsim}@kw.ac.kr |
| Source: | KWU (Kwanwoon University) |  |  |

_____________________________

# Abstract

This contribution proposes to signal Intra block copy enabling flag in slice header. From the experimental results of the current SCM2.0, Intra block copy achieves high coding gain compared to the current HM14.0. However, the selection ratio of Intra block copy is quite low in case of inter coding pictures because the temporal correlation is high especially when the temporal distance between the current picture and the reference picture is close. In addition, for coding the mixed contents, Intra block copy selection ratio in natural texture region is also low but the CU-level Intra block copy enabling flag is signaled for every CU. Therefore, in this contribution, we propose slice-level Intra block copy enabling.

# Introduction

The SCC profile for screen content coding of HEVC contains Intra block copy which is one of the most outperforming tools for screen content video. It achieves high coding gain by enabling block matching in intra-picture and signaling a block vector in CU level. However, the SCC content includes not only the computer generated video but also the spatially mixed contents which contain the natural scene and computer generated one in a picture. In addition, we need to consider coding the temporally mixed contents. According to the current Intra block copy technique, it is enabled in sequence level which means that the enabling flag is signaled in SPS and the enabling flag for a CU is signaled for all the CUs in the sequence. Therefore, we propose slice-level Intra block copy enabling flag signaling to the current technology in this contribution.

# The slice-level Intra block copy enabling

In the proposed slice-level Intra block copy enabling, the slice-level Intra block copy enabling flag is signaled in slice header. Generally, Intra block copy outperforms for screen contents and it achieves high coding gain. For enabling Intra block copy, sequence-level Intra block copy enabling flag (intra_block_copy_enabled_flag) is signaled and if it is enabled, intra_bc_flag is signaled for all the CUs in the sequence according to the current draft text.

For the case of which Intra block copy does not outperform e.g. natural video content in the input video which consists of both the natural and computer generated video, natural region in mixed content, especially in case of multiple slice encoding and etc., we can skip the Intra block copy enabled flag for each CU. Intra block copy selection ratio under the CTC [1] encoding experimental results of the current SCM2.0 is shown in Table 1.

Table 1. Intra block copy selection ratio under CTC configurations

| Resolution | Sequence | Category | Intra | Intra BC |
| --- | --- | --- | --- | --- |
| 1920x1080 | sc_flyingGraphics_1920x1080_60_8bit | TGM | 33.90% | 66.10% |
|  | sc_desktop_1920x1080_60_8bit | TGM | 38.41% | 61.59% |
|  | sc_console_1920x1080_60_8bit | TGM | 33.41% | 66.59% |
|  | MissionControlClip3_1920x1080_60p_8b444 | M | 72.13% | 27.87% |
|  | EBURainFruits_1920x1080_50_10bit | CC | 98.86% | 1.14% |
|  | Kimono1_1920x1080_24_10bit | CC | 99.78% | 0.22% |
| 1280x720 | sc_web_browsing_1280x720_30_8bit | TGM | 52.81% | 47.19% |
|  | sc_map_1280x720_60_8bit | TGM | 87.32% | 12.68% |
|  | sc_programming_1280x720_60_8bit | TGM | 62.48% | 37.52% |
|  | sc_SlideShow_1280x720_20_8bit | TGM | 62.48% | 37.52% |
|  | sc_robot_1280x720_30_8bit | A | 95.81% | 4.19% |
| 2560x1440 | Basketball_Screen_2560x1440_60p_8b444 | M | 69.43% | 30.57% |
|  | MissionControlClip2_2560x1440_60p_8444 | M | 79.68% | 20.32% |

- AI configuration

| Resolution | Sequence | Category | I-Slice |  | B-Slice |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra | Intra BC | Inter | Intra | Intra BC |
| 1920x1080 | sc_flyingGraphics_1920x1080_60_8bit | TGM | 34.45% | 65.55% | 83.41% | 6.03% | 10.56% |
|  | sc_desktop_1920x1080_60_8bit | TGM | 38.80% | 61.20% | 99.15% | 0.31% | 0.54% |
|  | sc_console_1920x1080_60_8bit | TGM | 33.93% | 66.07% | 97.56% | 1.20% | 1.23% |
|  | MissionControlClip3_1920x1080_60p_8b444 | M | 70.53% | 29.47% | 99.46% | 0.36% | 0.18% |
|  | EBURainFruits_1920x1080_50_10bit | CC | 98.82% | 1.18% | 99.54% | 0.45% | 0.01% |
|  | Kimono1_1920x1080_24_10bit | CC | 99.77% | 0.23% | 90.77% | 9.21% | 0.02% |
| 1280x720 | sc_web_browsing_1280x720_30_8bit | TGM | 52.79% | 47.21% | 99.57% | 0.30% | 0.13% |
|  | sc_map_1280x720_60_8bit | TGM | 87.63% | 12.37% | 98.76% | 0.74% | 0.50% |
|  | sc_programming_1280x720_60_8bit | TGM | 60.93% | 39.07% | 97.02% | 2.42% | 0.55% |
|  | sc_SlideShow_1280x720_20_8bit | TGM | 94.30% | 5.70% | 97.03% | 2.80% | 0.16% |
|  | sc_robot_1280x720_30_8bit | A | 95.33% | 4.67% | 99.04% | 0.92% | 0.04% |
| 2560x1440 | Basketball_Screen_2560x1440_60p_8b444 | M | 66.20% | 33.80% | 99.47% | 0.51% | 0.02% |
|  | MissionControlClip2_2560x1440_60p_8444 | M | 77.15% | 22.85% | 98.05% | 1.37% | 0.57% |

- RA configuration

| Resolution | Sequence | Category | I-Slice |  | B-Slice |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra | Intra BC | Inter | Intra | Intra BC |
| 1920x1080 | sc_flyingGraphics_1920x1080_60_8bit | TGM | 31.84% | 68.16% | 81.15% | 8.65% | 10.20% |
|  | sc_desktop_1920x1080_60_8bit | TGM | 40.08% | 59.92% | 99.13% | 0.31% | 0.55% |
|  | sc_console_1920x1080_60_8bit | TGM | 36.96% | 63.04% | 97.64% | 1.12% | 1.23% |
|  | MissionControlClip3_1920x1080_60p_8b444 | M | 65.82% | 34.18% | 99.54% | 0.28% | 0.18% |
|  | EBURainFruits_1920x1080_50_10bit | CC | 98.85% | 1.15% | 99.84% | 0.15% | 0.00% |
|  | Kimono1_1920x1080_24_10bit | CC | 99.83% | 0.17% | 95.79% | 4.20% | 0.02% |
| 1280x720 | sc_web_browsing_1280x720_30_8bit | TGM | 63.55% | 36.45% | 99.17% | 0.58% | 0.25% |
|  | sc_map_1280x720_60_8bit | TGM | 85.36% | 14.64% | 99.31% | 0.52% | 0.17% |
|  | sc_programming_1280x720_60_8bit | TGM | 53.89% | 46.11% | 96.76% | 2.61% | 0.62% |
|  | sc_SlideShow_1280x720_20_8bit | TGM | 82.97% | 17.03% | 95.45% | 4.28% | 0.28% |
|  | sc_robot_1280x720_30_8bit | A | 95.47% | 4.53% | 98.90% | 1.05% | 0.04% |
| 2560x1440 | Basketball_Screen_2560x1440_60p_8b444 | M | 69.30% | 30.70% | 99.58% | 0.41% | 0.01% |
|  | MissionControlClip2_2560x1440_60p_8444 | M | 66.86% | 33.14% | 98.12% | 1.30% | 0.58% |

(c) LD configuration

The Intra block copy selection ratio shown in Table 1, simply, varies depending on the slice type. The selection ratio of Intra block copy is high in intra-slice and low in inter-slice and it depends on the temporal correlation rather than the spatial correlation. For this reason, we propose slice-level Intra block copy enabling flag indication.

In the current draft text [2], intra_block_copy_enabled_flag is signaled as

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| } |  |

and CU-level intra_bc_flag is decoded as

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } | ae(v) |

.

By using the proposed method, we can adaptively apply Intra block copy in slice-level with simple changes and allow to skip intra_bc_flag in CU-level as shown in Fig. 1. Figure 1 shows some possible applications using the proposed slice-level Intra block copy enabling.

(a)

(b)

(c)

Figure 1. Example applications using the proposed slice-level Intra block copy enabling

Figure 1 (a) shows an example application using the proposed method in case of multiple slice encoding. If there is a region to be expected to get higher gain using Intra block copy, it can be enabled for some specific slices. Figure 1 (b) shows another example application of the proposed method. If the encoding sequence is temporally mixed (screen content and natural video), Intra block copy can be adaptively applied for screen content pictures. Figure 1 (c) shows the other example. According to the coding structure or temporal correlation, Intra block copy can be applied adaptively. Based on the Table 1, selection ratio of Intra block copy in inter-slice is low. It reflects that, generally, inter-coding achieves higher coding gain than Intra block copy and temporal correlation is highly exploited.

To signal the slice-level Intra block copy enabling flag, we propose the text would be fixed as

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( intra_block_copy_enabled_flag ) |  |
| slice_intra_block_copy_enabled_flag | u(1) |
| … |  |
| } |  |

and CU-level intra_bc_flag can be decoded as

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if(slice_intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| } | ae(v) |

. With these simple changes of the current SCC profile, it can support encoding the various applications efficiently.

# Conclusion

In this contribution, we proposed slice-level Intra block copy enabling method for functionalities of screen content coding. By using the proposed slice-level Intra block copy enabled flag indication in slice header, we expect that the current screen content coding can support various applications and flexible coding.

# Patent rights declaration(s)

KWU (Kwangwoon University) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “JCT-VC R1015: Common Conditions for Screen Content Coding Tests,” 18th Meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Sapporo, Japan, Jul., 2014.

- R. Joshi, and J. Xu, “JCT-VC R1005: High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 1,” 18th Meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Sapporo, Japan, Jul., 2014.