| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0057 |
| --- | --- |

| Title: | CE1: Results of Test 1.3, Test 4.1 and Test 4.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports experimental results of CE1 Test 1.3, Test 4.1 and Test 4.3 related to the adaptive vector switching technique proposed in JCTVC-R0074. The proposed scheme introduces to select whether BV is predicted or not for each PU. The selection flag is signalled only when the BV component is non-zero. In Test 1.3, the proposed scheme (Method 1) and another version of the proposed scheme adding the modification of the vector sign coding process (Method 2) have been evaluated. It has been shown that the coding performance of Method 1 and Method 2 achieve up to 0.5% and up to 0.7% average coding gains relative to CE1 anchor, respectively. The coding performance of the combination of the proposed schemes and each of other BVD coding schemes (Test 1.1 and Test1.4) has also been confirmed. In Test 4.1, it has been reported that the combination of Method 1 + Test 1.1 and Method 2 + Test1.1 achieve up to 0.6% and up to 1.0% average coding performance improvement compared to CE1 anchor, respectively. Test 4.3 has also been reported that the average coding performance of the combination of Method 1 + Test 1.4 and Method 2 + Test 1.4 show up to 0.7% and up to 0.9 % gains relative to CE1 anchor, respectively.

# Introduction

In JCTVC-R0074 [1], the adaptive switching between differential coding and direct coding for intra block copy vectors is proposed. This scheme (Method 1) introduces one flag to specify whether the Block Vector (BV) is predicted or not. If the flag indicates not to predicted, BV is directly coded. Moreover, an additional version (Method 2) adding Method 1 to the modification of the coding process of vector sign flag is also proposed in JCTVC-R0074. In CE1 [2], the following tests related to the proposed scheme have been conducted.

- Test 1.3

  - Evaluation of Method 1 and Method 2

- Test 4.1

  - Evaluation of the combination of Test 1.3 and Test 1.1

- Test 4.3

  - Evaluation of the combination of Test 1.3 and Test 1.4

# Proposed schemes

In the proposed scheme, a PU-level flag is transmitted with a block vector (BV) in intra block copy to indicate whether the vector is predicted or not. Although BV is coded separately divided into horizontal and vertical components, the proposed scheme uses a single flag which indicates if both horizontal and vertical components are predicted or not. The flag is signalled only when the BV is non-zero vector. The proposed scheme has two different versions called as “Method 1” and “Method 2”.

## Method 1

In this method, bv_pred_flag is added to the current specification as shown in Fig. 1. When bv_pred_flag is equal to 1, BV is predicted and its prediction error is coded, while the flag is equal to 0, the block vector is encoded directly. bv_pred_flag is present when neither horizontal nor vertical components of BV are equal to 0, since the two components of BV cannot be both 0. If bv_pred_flag is not present, bv_pred_flag is inferred to be set to 1.

Fig. 1 Decoding process of BV in Method 1

## Method 2

In Method 2, bv_pred_flag is coded after coding process of absolute values of BV (or BVD) and the sign flag coding of horizontal and vertical components are modified as shown in Fig. 2. This modification can remove the redundancy of coding process of the sign flag when bv_pred_flag indicates to 0 due to the fact that the horizontal and vertical components of BV cannot be both positive values. When one component is equal to 0, it is not necessary to encode the sign flag of the other component since it has always negative value. When the two components are both non-zero, we introduce a new flag named bv_sign_flag to indicate whether the horizontal and vertical components are both negative or not. When this flag is equal to 1, the two components are both negative, while this flag is equal to 0, there are two possibilities: (i) the horizontal component is negative and the vertical component is positive, and (ii) the horizontal component is positive and the vertical component is negative. So we additionally encode bvd_sign_flag[ 0 ] to distinguish these two cases when bv_sign_flag is equal to 0.

Fig. 2 Decoding process of BV in Method 2

# Experimental results

In this section, CE1 results related to the proposed scheme are presented. The tested methods are implemented on top of SCM-2.0. According to CE1 description [2], SCC common test conditions are be used in all experiments [3]. The coding performance is compared to the SCM-2.0 for lossy and lossless configuration in terms of BD-rates and rate savings, respectively. Moreover, this section also reports the performance of the combination of the proposed scheme and each of BVD coding schemes in Test 1.1 and Test 1.4, respectively.

## Results of Test 1.3

### Method 1

Table 1 Lossy results of Method 1 on top of SCM-2.0 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | 0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | -0.5% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2 Lossless results of Method 1 on top of SCM-2.0 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

### Method 2

Table 3 Lossy results of Method 2 on top of SCM-2.0 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.6% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.4% |
| YUV, text & graphics with motion,720p | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | 0.2% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.4% | -0.8% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 4 Lossless results of Method 2 on top of SCM-2.0 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.3% | 0.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.4% | 0.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.2% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.1% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

## Results of Test 4.1

### Method 1

Table 5 Lossy results of Method 1 on top of Test 1.1 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.4% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.4% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

Table 6 Lossless results of Method 1 on top of Test 1.1 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.2% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.2% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.2% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

### Method 2

Table 7 Lossy results of Method 2 on top of Test 1.1 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.5% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.4% | -0.2% | -0.4% |
| YUV, mixed content, 1080p | -0.5% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.4% | -0.5% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.4% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.4% |
| RGB, mixed content, 1080p | -0.1% | 0.4% | 0.3% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.2% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 8 Lossless results of Method 2 on top of Test 1.1 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.3% | 0.8% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.4% | 0.8% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.3% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.2% | 0.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

## Results of Test 4.3

### Method 1

Table 9 Lossy results of method 1 on top of Test 1.4 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.6% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | -0.4% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.2% | -0.5% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.4% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.3% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.5% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | 0.0% | 0.3% | -0.5% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 10 Lossless results of method 1 on top of Test 1.4 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.2% | 0.7% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.3% | 0.7% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.1% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

### Method 2

Table 11 Lossy results of method 2 on top of Test 1.4 (Anchor: SCM-2.0)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.1% | -1.0% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -0.6% |
| YUV, mixed content, 1440p | -0.4% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.5% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | -0.3% | -0.5% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.2% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.7% | 0.3% |
| RGB, Animation, 720p | -0.1% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.2% |
| YUV, text & graphics with motion,720p | -0.2% | 0.2% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 12 Lossless results of method 2 on top of Test 1.4 (Anchor: SCM-2.0)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.4% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.4% | 0.9% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.5% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.2% | 0.7% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.2% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.1% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

# Conclusions

This contribution reported CE1 results related to adaptive switching between differential coding and direct coding for BV proposed in JCTVC-R0074. In Test 1.3, the coding performance of the proposed scheme (Method 1) and the modified scheme changing the coding process of sign flags (Method 2) have been evaluated. The coding results indicate that Method 1 and Method 2 achieve up to 0.5% and up to 0.7% average coding gains relative to CE1 anchor for lossy case, respectively. In lossless case, the average bitrate savings of Method 1 and Method 2 indicates up to 0.3% and 0.5% compared to CE1 anchor, respectively. Moreover, Test 4.1 has been reported that the combination of Method 1 + Test 1.1 and Method 2 + Test1.1 achieve up to 0.6% and up to 1.0% average coding performance improvement compared to CE1 anchor in lossy case, respectively. In Test 4.3, it has also been confirmed that the average coding performance of the combination of Method 1 + Test 1.4 and Method 2 + Test 1.4 show up to 0.7% and up to 0.9 % improvement relative to CE1 anchor in lossy case, respectively. We recommend to adopt one of the proposed methods into the new SCC draft text.

# References

- K. Miyazawa, A Minezawa and S. Sekiguchi, “Non-SCCE1: Adaptive switching between differential and direct coding for intra block copy vectors,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0074, 18th Meeting: Sapporo, JP, June 2014.

- C. Pang and X. Xu, “Description of Core Experiment 1 (CE1): Vector entropy coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1101, 18th Meeting: Sapporo, JP, June 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1015, 18th Meeting: Sapporo, JP, June 2014.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

# CE1 Test 1.3 Method 1

The text changes for Method 1 based on JCTVC-R1005-v2 are marked in yellow.

# 7	Syntax and semantics

7.3.8.10	Block vector difference syntax

| bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_bvd_greater0_flag[ 0 ] ¦ ¦ abs_bvd_greater0_flag[ 1 ] ) |  |
| bv_pred_flag | ae(v) |
| if( abs_bvd_greater0_flag[ 0 ] ) { |  |
| abs_bvd_minus1[ 0 ] | ae(v) |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ 1 ] ) { |  |
| abs_bvd_minus1[ 1 ] | ae(v) |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

7.4.9.7	Prediction unit semantics

abs_bvd_greater0_flag[ compIdx ] equal to 1 specifies that the absolute value of the compIdx-th block vector component difference is greater than 0. abs_bvd_greater0_flag[ compIdx ] equal to 0 specifies that the absolute value of the compIdx-th block vector component difference is equal to 0.

bv_pred_flag specifies whether the block vector is predicted or not.

When bc_bv_pred_flag is not present, it is inferred to be equal to 1.

abs_bvd_minus1[ compIdx ] plus 1 specifies the absolute value of the compIdx-th block vector component difference.

When abs_bvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

bvd_sign_flag[ compIdx ] specifies the sign of the compIdx-th block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the compIdx-th block vector component difference has a positive value.

- Otherwise, the compIdx-th block vector component difference has a negative value.

When bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference BvdIntra[ compIdx ] for compIdx in the range of 0 to 1, inclusive, is derived as follows:

BvdIntra[ compIdx ] = abs_bvd_greater0_flag[ compIdx ] *	(7-XX)
	( abs_bvd_minus1[ compIdx ] + 1 ) * ( 1 − 2 * bvd_sign_flag[ compIdx ] )

# 9	Parsing Process

9.3.3.1	General

| Table 9-13 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| bvd_coding ( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | bv_pred_flag | FL | cMax = 1 |
|  | abs_bvd_minus1[ ] | EG3 | - |
|  | bvd_sign_flag[ ] | FL | cMax = 1 |

# CE1 Test 1.3 Method 2

The text changes for Method 2 based on JCTVC-R1005-v2 are marked in yellow.

# 7	Syntax and semantics

7.3.8.10	Block vector difference syntax

| bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_bvd_greater0_flag[ 0 ] ) { |  |
| abs_bvd_minus1[ 0 ] | ae(v) |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ 1 ] ) { |  |
| abs_bvd_minus1[ 1 ] | ae(v) |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ 0 ] ¦ ¦ abs_bvd_greater0_flag[ 1 ] ) |  |
| bv_pred_flag | ae(v) |
| if( bv_pred_flag ) { |  |
| if( abs_bvd_greater0_flag[ 0 ] ) |  |
| bvd_sign_flag[ 0 ] | ae(v) |
| if( abs_bvd_greater0_flag[ 1 ] ) |  |
| bvd_sign_flag[ 1 ] | ae(v) |
| else if( abs_bvd_greater0_flag[ 0 ] && abs_bvd_greater0_flag[ 1 ] ) { |  |
| bv_sign_flag | ae(v) |
| if( !bv_sign_flag ) |  |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| } |  |

7.4.9.7	Prediction unit semantics

abs_bvd_greater0_flag[ compIdx ] equal to 1 specifies that the absolute value of the compIdx-th block vector component difference is greater than 0. abs_bvd_greater0_flag[ compIdx ] equal to 0 specifies that the absolute value of the compIdx-th block vector component difference is equal to 0.

bv_pred_flag specifies whether the block vector is predicted or not.

When bc_bv_pred_flag is not present, it is inferred to be equal to 1.

bv_sign_flag specifies the sign of a block vector as follows:

- If bv_sign_flag is equal to 1, bvd_sign_flag[ 0 ] and bvd_sign_flag[ 1 ] is set to 1.

- Otherwise, bv_sign_flag is equal to 0, bv_pred_flag is equal to 0 and bvd_sign_flag[ 0 ] is equal to 0, bvd_sign_flag[ 0 ] is set to 1.

- Otherwise, bv_sign_flag is equal to 0, bv_pred_flag is equal to 0 and bvd_sign_flag[ 0 ] is equal to 1, bvd_sign_flag[ 0 ] is set to 0.

When bv_sign_flag is not present, it is inferred to be equal to 0.

abs_bvd_minus1[ compIdx ] plus 1 specifies the absolute value of the compIdx-th block vector component difference.

When abs_bvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

bvd_sign_flag[ compIdx ] specifies the sign of the compIdx-th block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the compIdx-th block vector component difference has a positive value.

- Otherwise, the compIdx-th block vector component difference has a negative value.

When bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference BvdIntra[ compIdx ] for compIdx in the range of 0 to 1, inclusive, is derived as follows:

BvdIntra[ compIdx ] = abs_bvd_greater0_flag[ compIdx ] *	(7-XX)
	( abs_bvd_minus1[ compIdx ] + 1 ) * ( 1 − 2 * bvd_sign_flag[ compIdx ] )

# 9	Parsing Process

9.3.3.1	General

| Table 9-14 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| bvd_coding ( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | bv_pred_flag | FL | cMax = 1 |
|  | abs_bvd_minus1[ ] | EG3 | - |
|  | bvd_sign_flag[ ] | FL | cMax = 1 |
|  | bv_sign_flag | FL | cMax = 1 |