| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0060 |
| --- | --- |

| Title: | Non-CE6: Modified unpredicted palette entries coding in palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zheng Wang Jianqing Zhu 29F,Kerry Parkside,NO.1155 Fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5810 cheng@cn.fujitsu.com zhujianqing@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

Modification of unpredicted palette entries coding is presented. A flag is used to indicate whether difference between current palette entry and previous one or original value is coded. The process of encoding and decoding is proposed. Reported test results show gain of 0.5% and 0.7% on TGM RGB and YUV (AI), 0.1% and 0.2% on TGM RGB and YUV (RA), 0.1% and 0.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor.

# Introduction

In the 18th JCT-VC meetings in Sapporo, a base software [1] containing palette mode was suggested. This contribution is a modification on the base software.

In base software, palette entries are ordered by frequency. And fixed bit length coding is used on original pixel value of palette entry. In this contribution, palette is reordered by the pixel value of the first component. And difference value is coded for some palette entries.

# Proposed method

## Modified palette entries coding

- sort the palette depend on the value of the first color component

The palette entries are sorted in ascending order depend on the pixel value of the first color component. For YUV image, the first component is Y component. And for RGB image, it is R component. If the pixel values of the first color component of some palette entries are same, these entries are still sorted by frequency.

- reorder the palette

Scan the palette and divide entries into predicted group and non-predicted group. Set the predicted palette entries at the top of palette table. In this contribution, only focus on non-predicted palette entries.

- code the non-predicted palette entries as follows way

- The first component (It means Y of YUV image or R of RGB image)

For the first entry, use 8bits to code its original value. For others, check the difference between current entry and previous one. If the difference is less than 15, use 4bits to code the difference. Otherwise, use 8bits to code the original value. One flag is needed to indicate the coded value is original pixel value or the difference.

- Others components (It means U/V of YUV image or G/B of RGB image)

For the first entry, use 8bits to code its original value. For others, check the absolute value of difference between current entry and previous one. If the difference is less than 7, use 3bits to code the absolute value. Otherwise, use 8bits to code the original value. Two flags are needed. The one is mode flag. It indicates the follow coded value is original pixel value or the absolute value of difference. Another is sign flag. It indicates the sign of the difference.

# Experimental results

The anchor is the base software of SCM2 [1]. Test condition is common conditions for screen content coding test in R1015[2].

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.3% | 0.7% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.7% | 0.8% | 0.3% | 1.1% |
| YUV, text & graphics with motion,720p | 0.4% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 73% |  |  |  |
| Dec Time[%] | 88% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 0.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 83% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.1% | 0.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 84% |  |  |  |

# References

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen Content Coding Test Model 2 Encoder Description (SCM 2),” JCTVC-R1014, Sapporo, JP, July. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, JP, July. 2014.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).