#### Palette coding syntax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_transpose_flag | ae(v) |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_share_flag[ x0 ][ y0 ] ) { |  |
| palette_size = previousPaletteSize |  |
| for( n = 0; n < palette_size; n++ ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][ n ] |  |
| } else { |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize; i++ ) |  |
| previous_palette_entry_flag[ i ] = 0 |  |
| palette_last_group = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize && !palette_last_group && numPredPreviousPalette < max_palette_size; i++ ) { |  |
| lastPossibleGroupFlag = ( i + 4 >= previousPaletteStuffingSize ) |  |
| lastIdx = min( i + 4, previousPaletteStuffingSize ) − 1 |  |
| if( i > 3 && !lastPossibleGroupFlag ) |  |
| palette_all_zeros_in_group | ae(v) |
| else |  |
| palette_all_zeros_in_group = 0 |  |
| if( palette_all_zeros_in_group ) |  |
| i += 4 |  |
| else { |  |
| numOnesInGroup = 0 |  |
| for( idx = i; idx <= lastIdx && numPredPreviousPalette < max_palette_size; idx++ ) { |  |
| if ( idx = = lastIdx && numOnesInGroup = = 0 ) |  |
| previous_palette_entry_flag[ idx ] = 1 |  |
| else |  |
| previous_palette_entry_flag[ idx ] | ae(v) |
| if ( previous_palette_entry_flag[ idx ] ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ idx ] |  |
| numPredPreviousPalette++ |  |
| numOnesInGroup++ |  |
| } |  |
| } |  |
| if( !palette_all_zeros_in_group && !lastPossibleGroupFlag && numPredPreviousPalette < max_palette_size ) |  |
| palette_last_group | ae(v) |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ){ |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| if(i! = 0) |  |
| palette_entries_enc_mode | ae(v) |
| if(palette_entries_enc_mode==0) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| Else{ |  |
| if(cIdx != 0) |  |
| entries_dif_sign | ae(v) |
| palette_entries_dif[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| } |  |
| } |  |
| palette_size = numPredPreviousPalette + num_signalled_palette_entries |  |
| } |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) |  |
| indexMax = palette_size |  |
| else |  |
| indexMax = palette_size − 1 |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xC_prev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| yC_prev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE ) |  |
| palette_mode[ xC ][ yC ] | ae(v) |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| adjustedIndexMax = indexMax |  |
| adjustedRefIndex = indexMax + 1 |  |
| } |  |
| if( scanPos > 0 && palette_mode[xC_prev][yC_prev] ! = ESCAPE ) { |  |
| if( palette_mode[xC_prev][yC_prev] = = INDEX ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = paletteMap[ xC_prev ][ yC_prev ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[ xC_prev ][ yC_prev ] = = COPY_ABOVE && palette_mode[ xC ][ yC − 1 ] ! = ESCAPE ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = paletteMap[ xC ][ yC − 1 ] |  |
| } |  |
| } |  |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| if( adjustedIndexMax > 0 ) |  |
| palette_index | ae(v) |
| if( palette_index > = adjustedRefIndex ) |  |
| palette_index++ |  |
| if( palette_index = = palette_size ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| palette_mode[ xC ][ yC ] = ESCAPE |  |
| scanPos++ |  |
| } |  |
| } |  |
| if( palette_mode[xC][yC] ! = ESCAPE ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| runMode = palette_mode[ xC ][ yC ] |  |
| while ( runPos < = palette_run ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_mode[ xC ][ yC ] = = INDEX ) { |  |
| palette_mode[ xC ][ yC ] = INDEX |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } else { |  |
| palette_mode[ xC ][ yC ] = COPY_ABOVE |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ y − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| previousPaletteSize = palette_size |  |
| current_size = palette_size |  |
| for( i = 0; i < palette_size; i++ ) |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| tempPaletteEntries[ cIdx ][ i ] = palette_entries[ cIdx ][ i ] |  |
| for( i = 0; i < previousPaletteStuffingSize && current_size < max_palette_predictor_size; i++ ) |  |
| if( previous_palette_entry_flag[ i ] = = 0 ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| tempPaletteEntries[ cIdx ][ current_size ] = previousPaletteEntries[ cIdx ][ i ] |  |
| current_size++ |  |
| } |  |
| previousPaletteStuffingSize = current_size |  |
| previousPaletteEntries = tempPaletteEntries |  |
| } |  |

#### Palette coding semantics

palette_entries_enc_mode equal to 0 specifies that code value is the original palette entry. palette_entries_enc_mode equal to 1 specifies that code value is the difference between current palette entry and previous palette entry. When palette_entries_enc_mode is not present, it is inferred to be equal to 0.

palette_entries_dif[ cIdx ][ i ] specifies the absolute defference value between i-th palette entry and (i-1)-th palette entry in the palette for the color component cIdx.

entries_dif_sign specifies the sign of entries difference. When entries_dif_sign is not present, it is inferred to be equal to 0.

#### Decoding process of unpredicted palette entries

Inputs to this process are:

- a variable cIdx specifying the colour component of the current block

- palette_entries_enc_mode

- palette_entries[ cIdx ][ i ]

- entries_dif_sign

- palette_entries_dif[ cIdx ][ i ]

Output of this process are pltSample[ cIdx ][ i ], specifying a unpredicted palette entry value for the palette block.

The pltSample [ cIdx ][ i ] is derived as follows:

- If cIdx is equal to 0, the following applies:

  - If i is equal to 0

pltSample[ cIdx ][ i ] = palette_entries[ cIdx ][ numPredPreviousPalette + i ]

- Others ( i is not equal to 0), the following applies:

  - If the flag palette_entries_enc_mode is equal to 0

pltSample[ cIdx ][ i ] = palette_entries[ cIdx ][ numPredPreviousPalette + i ]

-  If the flag palette_entries_enc_mode is equal to 1

pltSample[ cIdx ][ i ] = pltSample[ cIdx ][ i-1 ] + palette_entries_dif[ cIdx ][ numPredPreviousPalette + i ]

- Others (cIdx is not equal to 0), the following applies:

  - If i is equal to 0

pltSample[ cIdx ][ i ] = palette_entries[ cIdx ][ numPredPreviousPalette + i ]

- If i is not equal to 0, the following applies:

  - If the flag palette_entries_enc_mode is equal to 0

pltSample[ cIdx ][ i ] = palette_entries[ cIdx ][ numPredPreviousPalette + i ]

-  If the flag palette_entries_enc_mode is equal to 1

    - If the flag entries_dif_sign is equal to 0

pltSample[ cIdx ][ i ] = pltSample[ cIdx ][ i-1 ] + palette_entries_dif[ cIdx ][ numPredPreviousPalette + i ]

    - If the flag entries_dif_sign is equal to 1

pltSample[ cIdx ][ i ] = pltSample[ cIdx ][ i-1 ] - palette_entries_dif[ cIdx ][ numPredPreviousPalette + i ]