| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0062 |
| --- | --- |

| Title: | CE6: Results of Test A.3 on restricted run coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results of CE6 Test A.3 which is related to the conditional use of the palette_run syntax element. The proposed method limits the amount of palette_run syntax elements decoded for the INDEX mode of the current palette design in the Screen Content Coding extension. When the colour index is larger than a threshold, the palette_run is never signalled. It is reported that the results of this modification brings an average BDR gain of -0.2% for AI, -0.1 % for RA and -0.2% for LDB for the lossy case.

# Introduction

The palette mode implementation basis of the SCM draft specifications [1] uses 3 modes to code the palette index: the “INDEX” mode, the “COPY_ABOVE” mode and the “ESCAPE” pixel mode. For the INDEX mode, a palette index is transmitted followed by a run value, the “palette_run” syntax element which indicates the number of following palette indexes with the same value. For the “COPY_ABOVE” mode, the “palette_run” is transmitted to indicate that, for the following positions (including the current one), the palette index is equal to the palette index of the row above.  For Escape pixel mode, no “palette-run” is transmitted.

# Proposed method

In the proposed method initially presented [2] and [3], the principle consists in limiting the amount of palette_run syntax element decoded for the INDEX mode of the current palette implementation. As shown in [2], the palette_run has different values for COPY_ABOVE mode and INDEX  mode and the palette_run is closely related to the palette index encoded for the INDEX mode. The principle of the method is based on the fact that the run coding is not efficient for large index value.

Consequently, the method consists in indicating the maximum palette index value (palette_limit_run). Thus, for all palette index values superior to this palette_limit_run, the run is not coded. The syntax element palette_limit_run is decoded just after the palette_entries for each Palette CU. It is coded with a unary max code with the maximum value equal to the palette_size.

On encoder side, the palette_limit_run is determined a posteriori. It means that, during the selection between the Run mode and the Copy above mode, the rates of using or not the proposed method are estimated. At the end of this process, the best palette_limit_run value is determined and the syntax of the current palette CU is updated.

# Experimental results

Table 1 gives the results of the proposed method compared to SCM2.0 under the common tests conditions [4].

Table 1: Lossy results of the proposed method compared to SCM2.0.

Table 2: Lossless results of the proposed method compared to SCM2.0.

# WD text

See attached document WD_CE6_Test_A.3.docx

# Conclusion

This contribution gives the results of a modification of the run coding for the INDEX mode. It consists in indicating an index value from which the run coding is not allowed. Coding efficiency shows a slight BDR gain of -0.2% for AI, -0.1 % for RA and -0.2% for LDB for the lossy case.

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 1”, JCTVC-R1005, 18th JCT-VC meeting, Sapporo, Japan, June/July 2014

- G. Laroche, T. Poirier, C. Gisquet, P. Onno, “AHG10: Run coding for palette mode,” JCTVC-Q0066, 17th JCT-VC meeting, March 2014, Valencia, Spain.

- G. Laroche, T. Poirier, C. Gisquet, P. Onno , “SCCE3: Test B.7 - Run coding”, JCTVC-R0085, 18th JCT-VC meeting, Sapporo, Japan, June/July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”. JCTVC-R1015, 18th JCT-VC meeting ,Sapporo, Japan, June/July 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).