| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0063 |
| --- | --- |

| Title: | CE6 Test C.1: transition prediction method in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports the results of CE6 test C.1 on a new index prediction method based on transitions, corresponding to the JCT-VC proposal R0084 presented during the 18th meeting in Sapporo. It is proposed in the present contribution to store the following index value for the last occurrence of a given index value, and conditionally use it as the inferred index value in the new prediction mode. It is reported that the use of such a prediction method provides gains, compared to CE6 palette anchor, of -0.3% (AI) / -0.2% (RA) / -0.2% (LDB) with IBC search on the full frame.

# Introduction / Problem Statement / …

SCM2.0 proposes 2 palette index prediction methods that can be seen in the following figure:

Figure 1 : palette index prediction modes

In the above example, we have represented arrows for these 2 prediction modes:

- The orange arrow copies from the left, i.e. represents a repetition of the same value;

- The white arrow copies from the above, i.e. is a repetition of the line above.

In the above example, one can easily deduce that the above mode will be selected for the first case, while the left mode will be used for the second one.

These modes are well suited for strictly vertical or horizontal features. However, there are instances where palette-like content exhibits different behavior. In particular, one such content can be observed below:

Figure 2 : example of specific palette content

This example represents text rendered with antialiasing, which is critical to user reading comfort. In such an example, one can clearly see the existence of so-called transitions. One can easily notice the repetitiveness of these transitions: the red, orange and blue arrows show repetitive transitions across the above figure.

# Proposal

The proposal is further detailed in modifications to the specifications in a document attached to this contribution.

## Concept

We propose adding a new prediction mode, called here transition mode or pattern mode, which takes into account above transitions. The idea is, for each palette index signaled in the copy left mode, to record the index value that immediately preceded it. Then, the new prediction mode will use these stored index values, depending on the value of the pixel on the left.

The above can be represented in the following figure:

Figure 3 : transition index prediction mode

Classical prediction modes are identical here, while we have presented the new possible modes with black arrows. In these cases, when encoding the orange index, the green one on the left is an indication of a potential transition. Now, having recorded the index following green index on previous occurrences, it is possible infer the orange index from this recorded index. The above example is a simplified version, but Figure 2 easily shows how that can apply.

It is interesting not to store the transitions occurring when the copy-up mode is used, as they actual provide no new information and may actually erase an updated transition.

In addition, it may happen that the index coded by the copy left prediction method is the same as the last one: indeed, the copy-up run may end because a new index occurs, while the run of the value is actually longer. Such is the case in Figure 3 where blue and red levels are present on the above line, but the current line contains a run of blue levels. As a consequence, the copy-left mode actually provides no new information, as it does not correspond to a transition.

## Modified encoder algorithm

The modified algorithm for coding the index can be represented as follows:

Figure 4 : encoder algorithm

The pattern encoding is only used when the PLT_RUN_LEFT prediction mode has been selected. When the number of elements in the palette is less than 3 or the pattern level is the same as the one above (if it exists), pattern encoding is disabled, and the regular index processing and coding is used. When it is enabled, a flag indicating whether the pattern index matches the current index is written. If it matches, then the index value is implicit, otherwise, the index is processed, knowing it cannot be the pattern index.

As usual, the pattern table is updated only if the pattern is not constant.

## Entropy coding

Transition mode is not used when the palette size is lower than 3 and the transition index is the same as above (in which case transition is often redundant). In addition, the syntax can be rather seen as an additional flag with its own context only present after the prediction mode flag:

| Codeword | Mode |
| --- | --- |
| 00 | Copy left |
| 01 | Copy transition |
| 1 | Copy above |

Table 1 : prediction mode codewords

Of course, when the transition prediction mode is off, the additional flag does not exist.

## Improved version

An important factor was neglected in the implementation in SCM2.0: scan patterns. Indeed, new, non-raster-scan patterns are now used, e.g.:

Figure 5: a new scan order

It can be seen that on odd lines, the transition is the opposite of what was encountered on the previous line. Basically, transition on odd lines may not be correctly used, and may actually reset the correct transition for even lines.

# Results

## Initial version

The results were generated using the CTC for Screen Content Coding [1].

### Lossy coding

### Lossless results

## Bug-fixed version

The palette mode adopted makes uses of specific scan orders, as described in section 2.4. Once they are properly accounted for, we have (results to be updated once RA and LDB scenarios are in):

### Lossy results

### Lossless results

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” JCTVC-R1015, 18th JCT-VC meeting, July 2014, Sapporo, Japan.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).