| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0064 |
| --- | --- |

| Title: | Non-CE6: Last run flag for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is about a modification of the run coding in the current design of the palette coding mode. The SCM2.0 design of the run coding is modified in order to insert a flag for signaling if the current run is the last run for the processed CU. It is reported that the results of this modification brings an average BDR gain of -0.3% for AI, -0.2 % for RA and -0.2% for LDB for the lossy case on Screen Content sequences.

# Problem statement

In SCM2.0 the binarization of the palette_run is performed according to the table. Value of the palette_run 0, 1 and 2 are respectively signaled by the following codewords:”0”, “10”, “110”. When the value exceed 2, the codeword is built by using a prefix starting by “111” and the suffix is built by using a truncated Rice code (TR) with a Rice parameter of 3.

Table 1: Binarization for palette_run

| palette_run | Codeword |
| --- | --- |
| 0 | '0' |
| 1 | '10' |
| 2 | '110' |
| >2 | Prefix = '111', Suffix = TR code , cRiceParam = 3 |

The main advantage of this binarization is to associate very short codewords for same value of the palette_run. However, when the palette_run value is high, this binarization process leads to a quite high number of bits and thus is not very efficient in terms of coding efficiency.

# Proposed method

In the proposed method, we suggest adding an additional flag to signal if the current run is the last run for the current CU as depicted by the grey blocks of Figure 1. The binarization of the 3 first values of the palette_run remains unchanged.

Figure 1: New binarization process of palette_run syntax element.

The proposed method can be summarized by the table which consists in adding a codeword for the last run and change the prefix value for palette_run value > 2 compared to the design of SCM2.0 in Table 2.

Table 2: New binarization for palette_run

| palette_run | Codeword |
| --- | --- |
| 0 | '0' |
| 1 | '10' |
| 2 | '110' |
| last run (up to the end) | '1111' |
| >2 (not up to the end) | Prefix = '1110', Suffix = TR code , cRiceParam = 3 |

# Experimental results

Table 3 gives the results of the proposed method compared to SCM2.0 under the common tests conditions [2].

Table 3: Lossy results of the proposed method compared to SCM2.0.

Table 4: Lossless results of the proposed method compared to SCM2.0.

# WD text

The change in the SCC Working Draft corresponds to change Table 9-46 (Table 1 in this document) by the new binarization presented in Table 2.

# Conclusion

This contribution gives the results of a slight modification of the binarization process for the run_palette syntax element. It consists in adding a flag  indicating the final run of the palette mode for a current CU. Coding efficiency shows a noticeable BDR gain of -0.3% for AI, -0.2 % for RA and -0.2% for LDB for the lossy case. Gains are also observed for the lossless case.

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 1”, JCTVC-R1005, 18th JCT-VC meeting, Sapporo, Japan, June/July 2014

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”. JCTVC-R1015, 18th JCT-VC meeting ,Sapporo, Japan, June/July 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).