| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0065 |
| --- | --- |

| Title: | Non-CE2: Intra Block Copy encoder improvements for SCM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | G. Laroche, C. Gisquet, T. Poirier, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution is proposing several encoder improvements related to the Intra Block Copy mode in the SCM2.0 without introducing any normative changes. It is asserted that these proposed encoders changes provide a BDR gain of 2.2% over SCM2.0 while decreasing the encoding runtime to 81% for Intra configuration.

# Introduction

During the 18th meeting held in July 2014 in Sapporo, it was noticed that further coding efficiency improvements can be obtained by modifying the Intra block copy encoder selection only.   In particular, encoder improvements of 1% for 10% additional runtimes have not been adopted for IBC selection.  The current proposal asserts that various improvements to IBC search can be achieved by using better tradeoffs than this. The proposal asserts furthermore that many intra-frame copy-based methods should be reviewed by separating the normative changes from the encoder only-changes.

# Proposed search improvements

## Fast evaluations

### Rationale

Like in Inter scenarios, it is very likely that the image can actually be segmented in non-square areas. As a consequence, we can expect from the BV candidates that:

- They can be a good match;

- They are a very good match when they already provide the best block predictors;

- Some of the speed-up algorithms for inter can be applied too.

### Fast evaluation

When the best CU mode has no residual, no intra mode is evaluated. This could happen also for IBC, except IBC is not yet ran at this point, and its runtime is generally too big to help at this point. Therefore, it is proposed to only test a few positions, and use the encoding result for this to decide whether to skip intra evaluation (including IBC with actual search and palette). We name this search fast IBC search in the following, as opposed to IBC “full” search.

Furthermore, like for inter, if the best mode is IBC and the CU has no residual, the sub-CUs evaluations can also be skipped.

Overall, it was observed that the coding gain was impacted in negligible ways by these choices.

### Fast evaluation and SAD measurement

As a consequence, the SAD measurement here can serve 2 purposes:

- Set the bar for the following evaluation (2Nx2N and other PUs when applicable);

- Stop fast evaluation rapidly: indeed, the candidates may fall within the threshold where IBC full search is not evaluated, but the normal intra evaluation would be beneficial (e.g. case of gradients or strongly horizontal or vertical features). This can be checked using said SAD, stopping the rate evaluation which requires costly RQT depth evaluation.

However, for all these fast evaluations to work best, it is very beneficial to use as many potential good positions here, as the added evaluation time is often completely offset by the saving on IBC full search.

## New positions evaluated

This provides by large the biggest coding efficiency and runtime gains in this proposal.

### In relation to CE2

Various CE2 proposals have an end result of introducing new BV predictors, thereby evaluating additional positions besides the ones reached by the current search algorithm. By adapting this in a non-normative way, it is possible to both improve encoding efficiency and runtime.

Therefore, the spatial neighboring as the temporally collocated BVs are considered, as well as the BVs of the CU pointed at by those BVs

### Caching system

As many of those BVs may be identical, it is beneficial to keep only the unique ones to have a limited cache size (set to 64) and reduce the redundant evaluations. Furthermore, it is possible to keep the best candidates during the IBC search for a CU and reuse them for the sub-PUs and sub-CUs. This is achieved by keeping more chroma refinement candidates after normal or hash-based searches, and adding them to the cache.

## Better distribution of the search time across PUs

With the cache of good candidates, it was observed the amount of work was inefficiently distributed. The following changes were added to improve further coding efficiency and runtime gains:

- 32x32 CUs are now also evaluated, providing a runtime gain;

- The Nx2N PUs provide the most gains, while 2NxN can be compensated in various ways: 2D search is no longer performed (except due to the BVs in the cache) for 2NxN, while Nx2N 1D search has been increased to the full width and height;

- While the IBC PU evaluations are conditioned to a high enough intra cost, it is actually possible to recoup a bit of the coding gain efficiency loss of this by performing, in that case, only the fast evaluation for NxN.

This optimization is very preliminary: in particular, no attempt at changing the thresholds in the current algorithms was made.

# Results

The results were generated using the CTC for Screen Content Coding Error: Reference source not found.

## Lossy coding

Table 1 gives the BDR gain for lossy configurations compared to SCM2.0 with the common test conditions. In average for all sequences, the proposed encoder selection gives a -2.2%, -1.3% and -0.8% BDR gain for respectively AI, RA and LDB configuration. The gain reaches 5.8% for RGB text and graphics 1080p. In addition to this coding gain the encoding run time is decreased especially for Intra configuration with an encoding run time equal to 81%. This encoding run time is especially reduced for sequences where the IBC mode is especially efficient. For all configurations the encoding run time is more reduced for 1080p text and graphics with 70% of encoding run time.

Please note that the decoding run time is not accurate.

Table 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | Enc Time[%] |
| RGB, text & graphics with motion, 1080p | -5.8% | -5.9% | -5.8% | 70.2% |
| RGB, text & graphics with motion,720p | -2.4% | -2.4% | -2.4% | 80.9% |
| RGB, mixed content, 1440p | -2.1% | -2.0% | -2.1% | 83.2% |
| RGB, mixed content, 1080p | -3.0% | -3.0% | -2.8% | 83.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 95.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 95.8% |
| YUV, text & graphics with motion, 1080p | -2.9% | -3.5% | -3.5% | 70.0% |
| YUV, text & graphics with motion,720p | -1.8% | -2.0% | -2.0% | 76.5% |
| YUV, mixed content, 1440p | -1.7% | -1.8% | -1.9% | 79.6% |
| YUV, mixed content, 1080p | -2.5% | -2.4% | -2.8% | 79.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% | 92.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 92.5% |
| Enc Time[%] | 81% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | G/Y | B/U | R/V | Enc Time[%] |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.8% | -2.8% | 95.2% |
| RGB, text & graphics with motion,720p | -1.5% | -1.5% | -1.5% | 97.2% |
| RGB, mixed content, 1440p | -1.3% | -1.1% | -1.2% | 97.5% |
| RGB, mixed content, 1080p | -2.0% | -2.0% | -1.8% | 98.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 99.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 99.3% |
| YUV, text & graphics with motion, 1080p | -1.9% | -2.0% | -2.0% | 95.6% |
| YUV, text & graphics with motion,720p | -1.2% | -1.3% | -1.1% | 97.2% |
| YUV, mixed content, 1440p | -0.8% | -0.7% | -0.9% | 96.9% |
| YUV, mixed content, 1080p | -1.6% | -1.5% | -2.0% | 98.9% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 98.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% | 99.1% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 106% |  |  |  |
|  | Low delay B |  |  |  |
|  | G/Y | B/U | R/V | Enc Time[%] |
| RGB, text & graphics with motion, 1080p | -2.0% | -1.9% | -1.9% | 96.6% |
| RGB, text & graphics with motion,720p | -0.7% | -0.6% | -0.6% | 98.8% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% | 98.3% |
| RGB, mixed content, 1080p | -0.9% | -0.1% | -0.2% | 98.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 98.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 98.6% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.4% | 98.2% |
| YUV, text & graphics with motion,720p | -0.5% | -0.1% | -0.6% | 98.3% |
| YUV, mixed content, 1440p | -0.5% | 0.0% | -0.1% | 99.3% |
| YUV, mixed content, 1080p | -0.4% | 0.5% | -0.2% | 98.4% |
| YUV, Animation, 720p | 0.0% | -0.4% | 0.1% | 100.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 100.2% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

## Lossless coding

# Conclusion

Several improvements are proposed on the IBC search, with large improvements in both encoding time and coding efficiency. It is suggested, first, to adopt those changes, second, to review any intra copy method, whether IBC or string-based, by separating improvements coming from the normative changes themselves and the encoder algorithm, irrespective of the CE.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).