| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0068 |
| --- | --- |

| Title: | AHG14: On IBC memory reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr. |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is related to the Intra block copy. An IBC PU predictor comes from reconstructed non-filtered blocks. This increases the memory needed for some implementations, an example being the case where loop filtered blocks need to be stored in addition to the reconstructed blocks because of IBC. To avoid this additional storage, this contribution proposes to signal the CTB which are available for IBC prediction. For these CTBs, the DBF and SAO are disabled. The proposed method gives 0.7% losses for Intra configuration compared to the current SCM2.0.

# Introduction

For the Intra Block Copy method, the PU predictors come from reconstructed data of the current frame and as all other Intra modes this data is non-loop filtered.

In the HEVC specification, the DBF and SAO loop filtering are defined at picture level. But these filterings can be fully parallelized at CU level if all neighboring data are available. So for some implementations, the DBF and SAO are proceeded on the fly and/or in parallel to the other processes of block reconstruction. To keep this ability to apply the SAO and DBF at lower level in parallel, when Intra Block Copy is enabled, it is needed to store both loop filtered data and non-filtered data which increases the memory needed by one frame for the worst case because a BV in the last CU can point at the very beginning of the image.

The alternative is to delay DBF and SAO until the frame has been reconstructed, but this then requires saving all DBF and SAO syntax for delayed application.

In this contribution, the available CTBs for Intra Block Copy predictor are signaled in the bitstream. When a CTB is signaled as available for IBC, it is not loop filtered.

# Proposed method

At the beginning of each CTB, a flag, available_for_IBC_flag, is inserted in the bitstream. This flag indicates that the CTB can be used as reference for IBC prediction. Moreover when this flag is true, the deblocking filter and SAO are disabled.

In addition, the current CTB and the previous one are always considered as available for IBC prediction.

As a consequence, the memory increase for the implementation processing DBF and SAO on the fly is only 2 CTBs. Figure 1 illustrates the proposed method.

Figure 1 Example of the proposed method.

This method was implemented on top of SCM2.0 where DBF and SAO are processed at picture level.  In our implementation, before using DBF and SAO, the encoder sets the value of the flag available_for_IBC_flag to one if at least one block vector points to this CTB and if the IBC PU is not in the current CTB or following CTB.

# Experimental results

Table 1 gives the results of the proposed method. For Intra configuration, the impact is small with an average of 0.7% losses. For Inter configurations, the losses are larger especially for YUV content. But these losses could be compensated with an appropriate encoder selection.

Table 1 Lossy results of the proposed method.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p | 0.3% | -1.7% |  | -1.5% |  |
| RGB, text & graphics with motion,720p | 0.6% | -0.6% |  | -0.6% |  |
| RGB, mixed content, 1440p | 0.6% | 0.1% |  | -0.1% |  |
| RGB, mixed content, 1080p | 1.1% | 0.2% |  | 0.4% |  |
| RGB, Animation, 720p | 0.2% | 0.4% |  | 0.2% |  |
| RGB, camera captured, 1080p | 0.4% | 0.2% |  | 0.6% |  |
| YUV, text & graphics with motion, 1080p | 0.7% | -1.8% |  | -1.6% |  |
| YUV, text & graphics with motion,720p | 1.0% | 0.6% |  | 0.2% |  |
| YUV, mixed content, 1440p | 1.0% | 1.1% |  | 0.9% |  |
| YUV, mixed content, 1080p | 1.2% | -0.1% |  | 0.3% |  |
| YUV, Animation, 720p | 0.4% | 0.9% |  | 0.5% |  |
| YUV, camera captured, 1080p | 0.6% | 0.7% |  | 0.7% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 91% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U | R/V |  |
| RGB, text & graphics with motion, 1080p | 2.0% |  | -0.7% | -0.2% |  |
| RGB, text & graphics with motion,720p | 1.6% |  | 0.0% | -0.1% |  |
| RGB, mixed content, 1440p | 1.7% |  | 0.6% | 0.3% |  |
| RGB, mixed content, 1080p | 2.0% |  | 0.6% | 0.6% |  |
| RGB, Animation, 720p | 0.3% |  | 0.2% | 0.3% |  |
| RGB, camera captured, 1080p | 0.6% |  | 0.4% | 0.5% |  |
| YUV, text & graphics with motion, 1080p | 2.5% |  | -0.8% | -0.6% |  |
| YUV, text & graphics with motion,720p | 2.7% |  | 1.3% | 0.6% |  |
| YUV, mixed content, 1440p | 3.4% |  | 1.9% | 1.9% |  |
| YUV, mixed content, 1080p | 3.2% |  | 0.4% | 0.7% |  |
| YUV, Animation, 720p | 0.8% |  | 0.8% | 0.9% |  |
| YUV, camera captured, 1080p | 0.9% |  | 1.0% | 0.9% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U | R/V |  |
| RGB, text & graphics with motion, 1080p | 3.1% |  | 1.1% | 1.6% |  |
| RGB, text & graphics with motion,720p | 2.4% |  | 1.5% | 1.5% |  |
| RGB, mixed content, 1440p | 2.3% |  | 1.3% | 1.2% |  |
| RGB, mixed content, 1080p | 3.0% |  | 2.0% | 1.9% |  |
| RGB, Animation, 720p | 0.3% |  | 0.5% | 0.4% |  |
| RGB, camera captured, 1080p | 0.5% |  | 0.3% | 0.4% |  |
| YUV, text & graphics with motion, 1080p | 2.9% |  | 0.7% | 1.2% |  |
| YUV, text & graphics with motion,720p | 3.7% |  | 2.7% | 2.2% |  |
| YUV, mixed content, 1440p | 4.5% |  | 2.9% | 2.8% |  |
| YUV, mixed content, 1080p | 5.1% |  | 2.0% | 2.5% |  |
| YUV, Animation, 720p | 1.0% |  | 1.1% | 1.1% |  |
| YUV, camera captured, 1080p | 0.7% |  | 0.6% | 0.7% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |

# Conclusion

In order to solve the memory increase due to the use of non-loop filtered block as reference for Intra block copy, this contribution proposes to signal the available CTB for IBC, and to disable the deblocking filter and SAO for these available CTBs. The proposed method reduces the memory impact for IBC to only 2 CTBs. For Intra configuration, the average loss is 0.7% compared to the SCM2.0.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).