| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0069 |
| --- | --- |

| Title: | On adaptive colour transform and Inter modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr. |
| Source: | Canon |  |  |

_____________________________

# Abstract

The adaptive colour transform was adopted during the previous meeting. This contribution proposes to enable the colour transform for Inter modes at sequence parameters set instead of enabling it at CU level. A BDR average of 0% and -0.1% compared to SCM2.0 for respectively RA and LDB configurations is reported with an encoding run time of 95% and 96%.

# Introduction

The adaptive colour transform was adopted during the previous meeting. The method consists in applying a colorspace transform on the residual at CU level. For each CU containing a residual, the cu_residual_act_flag is decoded to know if the color transform is applied or not. But it seems that this CU level flag for Inter modes is not mandatory for coding efficiency improvement.

# Proposed modification

In this contribution, we propose to enable residual color transform for Inter modes at sequence level instead of at CU level. A flag inter_residual_colour_transform_enabled_flag is transmitted in the SPS when the residual_adaptive_colour_transform_enabled_flag is set equal to 1. The inter_residual_colour_transform_flag is set equal to 1 for RGB sequences and to 0 for YUV sequences. When this flag is set equal to true, the residual color transform is systematically applied on Inter CU residual and the cu_residual_act_flag doesn’t need to be decoded.

# Experimental results

Table 1 gives the BDR gain for lossy configurations compared to SCM2.0 with the common test conditions. In average for all sequences, the proposed encoder selection gives a 0%, -0.1% BDR gain for respectively RA and LDB configuration. For Chroma component a loss is observed. The encoding run time is decreased with an encoding run time equal to 95% and 96%. This encoding run time is especially reduced for sequences text and graphics 1080p sequences.

Please note that the decoding run time is not accurate.

Table 1 Lossy results of the proposed simplification compared to SCM2.0

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | Enc Time[%] |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.5% | 89.5% |
| RGB, text & graphics with motion,720p | 0.7% | 1.2% | 0.6% | 95.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 96.2% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | 0.1% | 97.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 96.1% |
| RGB, camera captured, 1080p | -0.5% | 0.9% | -0.4% | 96.3% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 92.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.2% | 96.4% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | 0.1% | 97.2% |
| YUV, mixed content, 1080p | -0.1% | 0.0% | 0.0% | 97.7% |
| YUV, Animation, 720p | -0.1% | 1.3% | 0.8% | 96.4% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.0% | 96.9% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low delay B |  |  |  |
|  | G/Y | B/U | R/V | Enc Time[%] |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.6% | 92.3% |
| RGB, text & graphics with motion,720p | 0.8% | 1.6% | 1.1% | 95.3% |
| RGB, mixed content, 1440p | -0.1% | 0.3% | 0.2% | 97.2% |
| RGB, mixed content, 1080p | 0.0% | 0.5% | 0.5% | 96.8% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.5% | 96.7% |
| RGB, camera captured, 1080p | -2.3% | 3.0% | -1.2% | 96.9% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% | 94.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.4% | 0.0% | 95.9% |
| YUV, mixed content, 1440p | 0.0% | 0.4% | 0.1% | 98.5% |
| YUV, mixed content, 1080p | -0.1% | 0.1% | 0.2% | 96.7% |
| YUV, Animation, 720p | 0.1% | 3.2% | 2.3% | 97.3% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.2% | 97.5% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

# Conclusion

This contribution proposes to remove the cu_residual_act_flag for Inter modes, in order to simplify the decoding process and to reduce the encoding run time. The average BDR is 0% for an encoder run time of 95% for Inter configurations. We propose to adopt this simplification of the residual color transform.

# Draft text modification

##### 7.4.3.2.3 Sequence parameter set scc extension

intra_block_copy_enabled_flag equal to 1 specifies that intra block copying may be invoked in the decoding process for intra prediction. intra_block_copy_enabled_flag equal to 0 specifies that intra block copying is not applied. When not present, the value of intra_block_copy_enabled_flag is inferred to be equal to 0.

palette_mode_enabled_flag equal to 1 specifies that the palette mode may be used for intra blocks. palette_mode_enabled_flag equal to 0 specifies that the palette mode is not applied. When not present, the value of palette_mode_enabled_flag is inferred to be equal to 0.

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0.

inter_residual_colour_transform_enabled_flag equal to 1 specifies that the adaptive colour transform is applied to the residual in the decoding process for Inter modes. inter_residual_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual for Inter modes. When not present, the value of inter_residual_colour_transform_enabled_flag is inferred to be equal to 0.

#### Coding unit syntax

| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag | ae(v) |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

#### Coding quadtree semantics

cu_residual_act_flag equal to 1 specifies that adaptive colour transform is applied to the residual samples of the current coding unit. cu_residual_act_flag equal to 0 specifies that adaptive colour transform is not applied to the residual samples of the current coding unit. When this flag is equal to 1 and the coding unit is intra coded, chroma modes of all prediction units within the current coding unit (i.e., intra_chroma_pred_mode) shall be equal to 4.

When cu_residual_act_flag is not present and CuPredMode[ x0 ][ y0 ]  is MODE_INTER cu_residual_act_flag is set equal to inter_residual_colour_transform_enabled_flag, otherwise it is inferred to be equal to 0.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).