| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0070 |
| --- | --- |

| Title: | AHG14: On IBC constraint for Wavefront Parallel Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr. |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is related to the Intra Block Copy method and the Wavefront Parallel Processing. It is proposed to constraint the IBC search range for Wavefront Parallel Processing to avoid IBC predictor pointing to a non-reconstructed block. The constraint rule consists in limiting the IBC search range to the potential reconstructed blocks when WPP is enabled. The proposed rule gives in average 0.6%, 0.3%, 0.1% BDR losses for respectively AI, RA and LDB configurations compared to SCM2.0 when the Wavefront Parallel Processing is enabled.

# Introduction

The wavefront parallel processing of HEVC offers a parallelisation at CTB line, as depicted in 
Figure 1. When the WPP is enabled, the delay between 2 CTB lines should be at least of one CTB. But by considering that a CTB should access the upper right CTB for the reconstruction, the minimum delay between CTB should be 2 CTBs as depicted in the following figure.

Figure 1 Example of wavefront parallel decoding with 4 threads (JCTVC-F274).

In the current IBC design, the block vector can point to the full reconstructed blocks of the current frame. When IBC is used with wavefront parallel processing, an IBC predictor can come from a non-reconstructed block as depicted in Figure 2. This slows down for the worst case the wavefront decoder implementation when IBC is used.

Figure 2 Example of reconstructed block with WPP and possible area for IBC block predictor.

# Proposed restriction

To solve the issue described in the previous section, we propose to limit the area pointed by the block vector when the WPP are enabled (entropy_coding_sync_enabled_flag  set equal to 1). The proposed search area is represented in green in Figure 3

Figure 3 Proposed IBC search area when WPP is enabled.

If we consider (CurrCTB_X, CurrCTB_Y) the coordinates of the current CTB, a CTB (RefCTB_X, RefCTB_Y) is considered available in the search area if it respects the following conditions:

- RefCTB_Y ≤ CurrCTB_Y and RefCTB_X-CurrCTB_X<-2*( RefCTB_Y-CurrCTB_Y)

# Experimental results

Table 1 gives the results of the proposed restriction of Figure 3 compared to the SCM2.0 when the Wavefront is enabled. In average 0.6%, 0.3%, 0.1% BDR losses for respectively AI, RA and LDB configurations are obtained with this restriction.

Please note that for experiment we fixed a bug for WPP at decoder side already fixed for HMRExt but not for SCM2.0.

Table 1Lossy results of the proposed restriction when WPP is enabled for both test and Anchor.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 1.3% | 1.2% | 1.3% |
| RGB, text & graphics with motion,720p | 1.4% | 1.4% | 1.4% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.4% | 1.3% | 1.3% |
| YUV, text & graphics with motion,720p | 1.6% | 1.5% | 1.4% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.4% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.5% |
| RGB, text & graphics with motion,720p | 0.9% | 0.8% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.6% |
| YUV, text & graphics with motion,720p | 1.0% | 1.0% | 1.0% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.1% | 0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.2% | 0.2% |
| YUV, text & graphics with motion,720p | 0.4% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.2% |
| YUV, mixed content, 1080p | 0.3% | 0.7% | 0.4% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |

# Conclusion

In this contribution, it is proposed to restrict the search range of the Intra block copy method when the Wavefront Parallel Processing is enabled. The aim is to prevent a block vector from pointing to a non-reconstructed block of another thread. The proposed restriction gives in average 0.6%, 0.3%, 0.1% BDR losses for respectively AI, RA and LDB configurations.

According to the small losses in coding efficiency, we propose to add this restriction in the HEVC Screen Content Coding Draft text.

# Proposed Draft text

### Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array [Ed. some callees still call this a single vector] of block vectors bvIntra.

The variables nCbS, nPbSw, and nPbSh are derived as follows:

nCbS = 1  <<  log2CbSize

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- The variable LastBvIntra[ 0 ][ compIdx ] and LastBvIntra[ 1 ][ compIdx ]specifies the last two block vector predictor. If this process is invoked for the first time for the current coding tree unit, LastBvIntra[ compIdx ] is derived as follows:

LastBvIntra[ 0 ][ 0 ] = −2* nCbS; LastBvIntra[ 0 ][ 1 ] = 0

LastBvIntra[ 1 ][ 0 ] = −nCbS; LastBvIntra[ 1 ][ 1 ] = 0

Depending upon the number of times this process has been invoked for the current coding tree unit, subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbSw, the luma prediction block height nPbSh, the last block vectors LastBvIntra, and the partition index blkIdx as inputs, and the block vector predictor BvpIntra[ xPb ][ yPb ] as the output, and bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to BvdIntra[ xPb ][ yPb ][ compIdx ] + BvpIntra[ xPb ][ yPb ][ compIdx ] [Ed. (GJS): Needs further formatting cleanup.]

When bvIntra[ xPb ][ yPb ][ 0 ] is not equal to LastBvIntra[ 0 ][ 0 ] or bvIntra[ xPb ][ yPb ][ 1 ] is not equal to LastBvIntra[ 0 ][ 1 ], the value of LastBvIntra[ 1 ][ compIdx ] is updated to be LastBvIntra[ 0 ][ compIdx ], and the value of LastBvIntra[ 0 ][ compIdx ] is updated to be bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignment is made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output is set equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output is set equal to TRUE.

–	One or both of the following conditions shall be true: [Ed. (GJS): Clarify that this is a bitstream constraint (if that is the correct interpretation).]

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw is less than or equal to 0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh is less than or equal to 0

–	when entropy_coding_sync_enabled_flag  is set equal to 1, it is a requirement of bitstream conformance that all of the following conditions are true:

– (bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh  – 1) /CtbSizeY is less or equal to 0

– (bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw – 1)/CtbSizeX is less than

-2*(bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh  – 1)/CtbSizeY

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).