| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0072 |
| --- | --- |

| Title: | CE6: Cross-check of Test CE6 Test A.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-S0039 about the “Binarization for run coding in palette mode”. The verification work was performed by using the software provided by the proponent for the CE6 Test A.6. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. As a result, the BD rate results reported during this crosscheck match to those provided in the contribution JCTVC-S0039.

# Analysis of the proposal

For CE6 A6, 2 configurations have been tested. The first one is related to a new binarization of the run coding for palette mode and the second one is the combination of Test A6 and A5 config 1.

CE6 Test A6 contains several modifications of the run coding:

- Removal of greater than 1 flag and greater than 2 flag.

- Replacement of the rice Golomb code by the exponential Golomb code for the remaining run.

- Add CABAC contexts for remaining run.

- Limit the value of the run according to the maximum possible value. Use of truncated Golomb code.

This last modification is probably the most complex modification because it needs to change the exponential Golomb code for each run decoding.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the CTC for SCC extension Error: Reference source not found. The results match to those provided by the proponent

The4 Excel Sheets of the cross-check results are attached to this document.

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” JCTVC-R1015, 18th JCT-VC meeting, July 2014, Sapporo, Japan.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).