| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0073 |
| --- | --- |

| Title: | CE6: Cross-check of Test C.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-S0078 about “CE6 Test C.2: Transition copy mode”. The verification work was performed by using the software provided by the proponent for the CE6 Test C.2. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. As a result, the BD rate results reported during this crosscheck match those provided in the contribution JCTVC- S0078.

# Analysis of the proposal

The proposal uses a method similar to the transition method that was first presented in January 2014 [1]. The notable elements of this proposal are:

- The mode is only activated for palette sizes of at least 2;

- For every index in the palette, a table records the association with another level;

  - The levels are called respectively pilot and TC indexes

- Because of the alternate scan orders used in palette mode, there are 2 tables, for respectively odd and even lines;

- The method has a table propagation mechanism that finds and updates each association from the palette of the previous CU to the palette of the current CU;

- The method exploits redundancy of the TC index in 2 ways:

  - If the TC index is equal to the adjusted Reference Index adjustedRefIndex, it is modified to be different, i.e. if adjustedRefIndex = 0 then TCindex = 1 otherwise TCindex = 0;

  - Explicit coding of the index means that the index is neither TCindex nor adjustedRefIndex, therefore this can be further exploited to reduce the actual indexes to code.

- The palette run type has been modified to indicate the new prediction method, adding a context to code the second bit of the codeword if need be.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the CTC for SCC extension [2]. Table 1 to Table 4 show the results of the proposed scheme compared to SCM2.0 anchors for lossy and lossless configurations.

### Lossy coding

### Lossless results

# References

- C. Gisquet, G. Laroche, P. Onno, “Non-RCE4: Transition copy mode for Palette mode,” JCTVC-P0115, 16th JCT-VC meeting, January 2014, San José, USA

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” JCTVC-R1015, 18th JCT-VC meeting, July 2014, Sapporo, Japan.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).