| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0078 |
| --- | --- |

| Title: | CE6 Test C.2: Transition copy mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Chen Sun +886-3-5670766 ext. 24572 {yc.sun, peter.chuang, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a run coding mode: transition copy. Experimental results show that compared with SCM-2.0, the BD-rates are -1.2%, -0.4%, and -0.4% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Introduction

In the palette mode in SCM-2.0 [1], the colour indices in a CU are run-coded. There are two run modes: index run and copy-above run. In the index run mode, an index would be signalled and the following indices in the run are set equal to the signalled index. In the copy-above run, the following indices in the run are set equal to the corresponding indices from the above line. Note that the above description is only correct for horizontal traverse scan, but the behaviour for vertical traverse scan can be easily derived by analogy.

This contribution proposes a run mode: transition copy (TC) run. In the TC run mode, a decoder records the adjacent colour index patterns and uses it to predict to-be-coded indices. This proposal is a part of CE6 [2] and is recorded as CE6 Test C.2. The detail is introduced in the Section 2.

# Proposed method

Before the detailed algorithm is described, please note that the following description is only for horizontal traverse scan, but the behaviour for vertical traverse scan can be easily derived by analogy.

A transition copy (TC) run is considered as an additional run mode in colour index run coding. An example of the TC colour indices is shown in Fig. 1. The current pixel, C, has a pilot colour index equal to 1. The shaded squares with numbers labeled are coded pixels. From the causal area of coded pixels, it can be found that when a colour index equal to 1 is coded with a left-to-right direction, the colour index to the right of the colour index equal to 1 is likely to be 2. This transition pattern (i.e., 1=>2) is recorded in the TC table to enhance colour index prediction efficiency.

Fig. 1.  An example of deriving the color index, where each square denotes a sample, C denotes the current sample, and the numbers in the squares denote the coded color indices.

In each CU, at the first sample a TC table is reset when the current CU is the first CU of the slice or inherits the TC table of the previous CU when the current CU is a non-first CU of the slice. We call this inheriting property “TC table propagation.” The TC table is maintained and updated as the pixels in the CU are coded. As shown in Table 1, for each to-be-coded non-first-column pixel, the input of the TC table is called a pilot colour index, Idxpilot, and is set to the colour index of the left or right pixel depending on the scan direction, and the outputs of the TC table is the TC colour index. For a pilot colour index, Idxpilot, the TC table records two TC colour indices, TCT[Idxpilot][0] and TCT[Idxpilot][1]. TCT[Idxpilot][0] is likely to occur at a pixel position to the right of an occurrence of the pilot colour index according to the causal area of the CU, and TCT[Idxpilot][1] is likely to occur at a pixel position to the left of an occurrence of the pilot colour index. A pixel on the Nth row will choose TCT[Idxpilot][Nth%2] as the TC predictor.

Table 1.  Illustration of a transition copy table (TCT)

The encoder and decoder will further refine the TC predictor. If the TC predictor is equal to the reference index (adjustedRefIndex[0] in the SCC text specification) or is greater than or equal to the palette size, the refined predictor is the smallest index not equal to the reference index. If the previous run is a copy above run, the previous index is not equal to the above index, and the TC predictor is equal to above predictor, the refined predictor is the previous index; otherwise, the refined predictor is equal to the TC predictor.

The refined predictor will be used to predict the current colour index. If the current colour index is coded in index run mode and is not the same as the pilot colour index, the encoder and decoder will replace TCT[Idxpilot][Nth%2] with the current colour index and replace TCT[the current colour index][(Nth +1)%2] with Idxpilot. Note that, because the refined TC predictor is always different from the reference index (adjustedRefIndex[0]), a second reference index (adjustedRefIndex[1]) is recorded and set equal to the refined TC predictor. In this way, if a pixel is coded by an index run mode, more redundancy can be removed to represent the index value (i.e., the index value of the index run mode cannot be the same as either the first reference index or the second reference index). In addition to more redundancy removal, the decoder does not need to maintain a TC table in the parsing stage.

There are three run modes: index run, copy run, and TC run. Therefore, palette_mode in SCM-2.0 is proposed to be modified as Table 2.

Table 2.  Proposed palette_mode syntax modification.

|  | palette_mode in SCM-2.0 | palette_mode in proposed syntax |
| --- | --- | --- |
| INDEX | 0 | 0 |
| COPY_ABOVE | 1 | 10 |
| INDEX_TC | N/A | 11 |

# Experimental results

The proposed method is implemented on top of SCM-2.0. The tests were conducted under the common test conditions in CE6 [3].

Table 3 shows the results of the proposed method using SCM-2.0 as the anchor. The encoding and decoding times are accurate. Experiments show that, compared with SCM-2.0, 1.2%, 0.4%, and 0.4% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

# Conclusion

In this contribution, a run coding mode, transition copy, is proposed for palette mode and implemented on top of SCM-2.0. Experimental results show that compared with SCM-2.0, the BD-rates are -1.2%, -0.4%, and -0.4% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-2.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0/.

- Y.-W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 6 (CE6): Palette Mode Improvement,” JCTVC-R1106, July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, July 2014.

- F. Zou, W. Pu, M. Karczewicz, R. Joshi, V. Sergin, “CE6: Test C.3 Copy previous row mode for palette coding”, JCTVC-S0174, 19th Meeting: Strasbourg, Oct. 2014.

Table 3.  Results of the proposed method; anchor: SCM-2.0.