#### Palette mode syntax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_transpose_flag | ae(v) |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_share_flag[ x0 ][ y0 ] ) { |  |
| palette_size = previousPaletteSize |  |
| for( n = 0; n < palette_size; n++ ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][ n ] |  |
| } else { |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize; i++ ) |  |
| previous_palette_entry_flag[ i ] = 0 |  |
| palette_last_group = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize && !palette_last_group && numPredPreviousPalette < max_palette_size; i++ ) { [Ed. YY: max_palette_size probably needs to be signalled] |  |
| lastPossibleGroupFlag = ( i + 4 >= previousPaletteStuffingSize ) |  |
| lastIdx = min( i + 4, previousPaletteStuffingSize ) − 1 |  |
| if( i > 3 && !lastPossibleGroupFlag ) |  |
| palette_all_zeros_in_group | ae(v) |
| else |  |
| palette_all_zeros_in_group = 0 |  |
| if( palette_all_zeros_in_group ) |  |
| i += 4 |  |
| else { |  |
| numOnesInGroup = 0 |  |
| for( idx = i; idx <= lastIdx && numPredPreviousPalette < max_palette_size; idx++ ) { |  |
| if ( idx = = lastIdx && numOnesInGroup = = 0 ) |  |
| previous_palette_entry_flag[ idx ] = 1 |  |
| else |  |
| previous_palette_entry_flag[ idx ] | ae(v) |
| if ( previous_palette_entry_flag[ idx ] ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ idx ] |  |
| numPredPreviousPalette++ |  |
| numOnesInGroup++ |  |
| } |  |
| } |  |
| if( !palette_all_zeros_in_group && !lastPossibleGroupFlag && numPredPreviousPalette < max_palette_size ) |  |
| palette_last_group | ae(v) |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + num_signalled_palette_entries |  |
| } |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) |  |
| indexMax = palette_size |  |
| else |  |
| indexMax = palette_size − 1 |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xC_prev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| yC_prev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE ) |  |
| palette_mode[ xC ][ yC ] | ae(v) |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| adjustedIndexMax = indexMax |  |
| adjustedRefIndex[0] = indexMax + 1 |  |
| adjustedRefIndex[1] = indexMax + 1 |  |
| } |  |
| if( scanPos > 0 && palette_mode[xC_prev][yC_prev] ! = ESCAPE ) { |  |
| if( palette_mode[xC_prev][yC_prev] = = INDEX ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex[0] = paletteMap[ xC_prev ][ yC_prev ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[ xC_prev ][ yC_prev ] = = COPY_ABOVE && palette_mode[ xC ][ yC − 1 ] ! = ESCAPE ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex[0] = paletteMap[ xC ][ yC − 1 ] |  |
| } |  |
| } |  |
| if(indexMax > 2){ |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex[1] = tran_idx |  |
| if(adjustedRefIndex[1]> adjustedRefIndex[0]) |  |
| (adjustedRefIndex[0], adjustedRefIndex[1]) = Swap(adjustedRefIndex[0], adjustedRefIndex[1] ) |  |
| } |  |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE && palette_mode[ xC ][ yC ] ! = INDEX_TC ) { |  |
| if( adjustedIndexMax > 0 ) |  |
| palette_index | ae(v) |
| if( palette_index > = adjustedRefIndex[0] ) |  |
| palette_index++ |  |
| if( palette_index > = adjustedRefIndex[1]) |  |
| palette_index++ |  |
| if( palette_index = = palette_size ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| palette_mode[ xC ][ yC ] = ESCAPE |  |
| scanPos++ |  |
| } |  |
| } |  |
| if( palette_mode[xC][yC] ! = ESCAPE ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| runMode = palette_mode[ xC ][ yC ] |  |
| while ( runPos < = palette_run ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_mode[ xC ][ yC ] = = INDEX ) { |  |
| palette_mode[ xC ][ yC ] = INDEX |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } else if( palette_mode[ xC ][ yC ] = = INDEX_TC ) { |  |
| palette_mode[ xC ][ yC ] = INDEX_TC |  |
| paletteMap[ xC ][ yC ] = tran_idx |  |
| } else { |  |
| palette_mode[ xC ][ yC ] = COPY_ABOVE |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ y − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| previousPaletteSize = palette_size |  |
| current_size = palette_size |  |
| for( i = 0; i < palette_size; i++ ) |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| tempPaletteEntries[ cIdx ][ i ] = palette_entries[ cIdx ][ i ] |  |
| for( i = 0; i < previousPaletteStuffingSize && current_size < max_palette_predictor_size; i++ ) |  |
| if( previous_palette_entry_flag[ i ] = = 0 ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| tempPaletteEntries[ cIdx ][ current_size ] = previousPaletteEntries[ cIdx ][ i ] |  |
| current_size++ |  |
| } |  |
| previousPaletteStuffingSize = current_size |  |
| previousPaletteEntries = tempPaletteEntries |  |
| } |  |

[Ed. (YK): There are some issues with this syntax table. For example, syntax elements should not have math (e.g., increment) or assignment operations. The variables should not contain underscores]

[Ed. (YY): consider moving some of the following from syntax table to semantics or decoding process. Also review the naming style below to make sure to properly distinguish between syntax element style naming from local/global variable style naming]

[Ed. (YY): substring matching problems exist and should be resolved later]

[Ed. (GS): Enumeration type codes should contain underscores (e.g., COPY_ABOVE seems OK, but not ESCAPE or INDEX).]

##### 8.4.5.2.8 Decoding process for palette mode

Inputs to this process are:

–	a location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nCbS specifying the size of the current block,

–	An array palette_mode[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1 and  y = 0..nCbS − 1, specifying the palette mode for each sample in the current block

–	The array paletteMap[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette index for each sample in the current block for which the palette mode is not equal to ESCAPE, and [Ed. (GJS): Check "is not" versus "is not equal to" (generally).] [Ed. (GJS): Check/fix use of "the palette mode".]

–	The array palette_escape_val[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1 and  y = 0..nCbS − 1, specifying the quantized escape value for each sample in the current block for which the palette mode is equal to ESCAPE.

–	The array palette_mode[ xCb + x ][ yCb + y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette run mode.

Output of this process array recSamples[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying a reconstructed sample values for the palette block.

The decoding process for transition index table propagation as specified in subclause 8.4.5.2.X is invoked assuming that input, prev_TC_index, is equal to TC_index of previous CU.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y.

–	Otherwise, if cIdx is equal to 1, recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C.

–	Otherwise (cIdx is equal to 2), recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C.

The (nCbS x nCbS) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is derived as follows:

For x = 0..nCbS − 1,  y = 0..nCbS − 1, recSample[ cIdx ][ yCb + y ][ xCb + x ] is set as follows:

–	If palette_mode[ xCb + x ][ yCb + y ] is equal to INDEX_TC, the decoding process for transition index table propagation as specified in subclause 8.4.5.2.X is invoked. The output, tran_idx, is assigned to paletteMap[ xCb + x ][ yCb + y ].

–	If palette_mode[ xCb + x ][ yCb + y ] is equal to INDEX, the decoding process for transition index table updating as specified in subclause 8.4.5.2.X is invoked.

For x = 0..nCbS − 1,  y = 0..nCbS − 1, recSample[ cIdx ][ yCb + y ][ xCb + x ] is set as follows:

–	If palette_mode[ xCb + x ][ yCb + y ] is not equal to ESCAPE, the following applies:

–	If palette_transpose_flag is true, the following applies:

recSample[ cIdx ][ yCb + y ][ xCb + x ] = palette_entries[ cIdx ][ paletteMap[ xCb + x ][ yCb + y ] ],

–	Otherwise (palette_transpose_flag is false), the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = palette_entries[ cIdx ][ paletteMap[ xCb + x ][ yCb + y ] ],

–	Otherwise, if cu_transquant_bypass_flag is true, the following applies:

–	Of palette_transpose_flag is true, the following applies:

recSample[ cIdx ][ yCb + y ][ xCb + x ] = palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ],

–	Otherwise (palette_transpose_flag is false), the following applies:

recSample[ cIdx ][ xCb + x ][ yCb + y ] = palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ],

–	Otherwise (palette_mode[ xCb + x ][ yCb + y ] is equal to ESCAPE and cu_transquant_bypass_flag is false), the following ordered steps apply: [Ed. (GJS): Indentation problem.]

- The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked assuming that the current block is the first block in the slice, availableA is equal to FALSE, availableB is equal to FALSE, and CuQpDeltaVal is equal to 0.

- The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0, qP is set to Qp′Y

–	Otherwise, if cIdx is equal to 1, qP is set to Qp′Cb

–	Otherwise (cIdx is equal to 2), qP is set to Qp′Cr.

- The variable bdShift is derived as follows:

bitShift = ( ( cIdx  = =  0 ) ? BitDepthY : BitDepthC)  + Log2( nTbS ) − 5	(8-XX)

[Ed. (GJS): Correct equation numbering.]

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies

–	If palette_transpose_flag is true, the following applies [Ed. (GJS): Check use of "true" versus "TRUE" and "is" versus "is equal to" (generally).]

recSamples[ xCb + x ][ yCb + y ] = Clip3( 0, 1 << bdShift − 1, ( ( palette_escape_val[ cIdx ][ yCb + y ][ xCb + x ]* 16 * levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift ),

–	Otherwise (palette_transpose_flag is false), the following applies

recSamples[ xCb + x ][ yCb + y ] = Clip3( 0, 1 << bdShift − 1, ( ( palette_escape_val[ cIdx ][ xCb + x ][ yCb + y ]* 16 * levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift ) [Ed. (GJS): Indentation problem.]

[Ed. (GJS): Reformulate as sentences or add equation numbers.]

##### 8.4.5.2.X Decoding process for transition index table propagation

Inputs to this process are:

–	The array prev_TC_index[pilot_index][candidate_index], with pilot_index = 0..31,  candidate_index = 0 and 1, specifying transition index table of previous CU.

Output of this process

–	The array TC_table[pilot_index][candidate_index], with pilot_index = 0..31,  candidate_index = 0 and 1, specifying transition index table of current CU.

For the derivation of the transition index table TC_index[ pilot ][ cand ] with pilot = 0..31,  cand = 0 and 1, the following applies:

–	If the current CU is the first CU of the slice, TC_index[pilot][cand] = 0, with pilot = 0..31,  cand = 0 and 1.

–	Otherwise, the following applies, with pilot = 0..31,  cand = 0 and 1:

The color map table color_map[ c ] with c = 0…31 is derived as follows:

for( idx = i; idx  <=  32; idx++ )

{

color_map[idx] = -1;

if ( previous_palette_entry_flag[ idx ] )

{color_map[idx] = numPredPreviousPalette}

numPredPreviousPalette++

}

For the derivation of the transition index table TC_index[ pilot ][ cand ] , the following applies with pilot = 0..31,  cand = 0 and 1:

TC_index[color_map[pilot]][cand] = 0.

And, the following applies, with pilot = 0..31:

–	If 	color_map[pilot] is not equal to -1, TC_index[color_map[pilot]][cand] = prev_TC_index[pilot][cand],
 , cand = 0 and 1.

And, the following applies, with pilot = 0..31,  cand = 0 and 1:

–	If 	color_map[TC_index[pilot][cand]] is equal to -1, TC_index[pilot][cand] = 0.

–	Otherwise,  TC_index[pilot][cand] = color_map[TC_index[pilot][cand]].

##### 8.4.5.2.X Decoding process for transition index derivation

Inputs to this process are:

–	a location ( x, y ) specifying the current sample relative to the top-left sample of the current block,

–	a location ( x_prev , y_prev  ) specifying the previous scaned sample relative to the top-left sample of the current block,

–	a variable indexMax specifying the maximal index value of the current block,

–	a variable adjustedRefIndex[0] specifying the redundant index value of the location ( x, y ),

–	a variable palette_index specifying the decoded index value of the location ( x, y ),

–	a variable nCbS specifying the size of the current block,

–	The array paletteMap[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette index for each sample in the current block for which the palette mode is not equal to ESCAPE, and [Ed. (GJS): Check "is not" versus "is not equal to" (generally).]

–	The array palette_mode[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette run mode.

–	The array TC_table[pilot_index][candidate_index], with pilot_index = 0..31,  candidate_index = 0 and 1, specifying transition index table of current CU.

Output of this process tran_idx, specifying a prediction index value of transition copy mode.

Depending on the palette_mode[ xCb][ yCb], the following assignments are made:

–	If palette_mode[ x ][ y ] is equal to INDEX or INDEX_TC,

tran_idx_tmp = palette_mode[paletteMap[ x ][ y ]][ y%2 ].

Depending on the adjustedRefIndex[0], the following assignments are made:

–	If adjustedRefIndex[0] is equal to 0, tran_idx_refine = 1

–	Otherwise, tran_idx_refine = 0

Depending on the value of tran_idx_tmp, indexMax, adjustedRefIndex[0], palette_mode[ xC_prev ][ yC_prev ], and paletteMap[xCb][ yCb- 1], the following assignments are made:

–	If tran_idx_tmp is larger than indexMax,

tran_idx = tran_idx_refine.

–	Otherwise, if tran_idx_tmp is equal to adjustedRefIndex[0],

tran_idx = tran_idx_refine.

–	Otherwise, if palette_mode[ xC_prev ][ yC_prev ] is equal to COPY_ABOVE,  paletteMap[xCb][ yCb- 1] is not equal to paletteMap[ xC_prev][ yC_prev], and tran_idx_tmp is equal to paletteMap[xCb][ yCb- 1],

tran_idx = paletteMap[ xC_prev][ yC_prev].

–	Otherwise,

tran_idx = tran_idx_tmp.

##### 8.4.5.2.X Decoding process for transition index table updating

Inputs to this process are:

–	a location ( x, y ) specifying the current sample relative to the top-left sample of the current block,

–	a location ( x_prev , y_prev  ) specifying the previous scaned sample relative to the top-left sample of the current block,

–	a variable palette_index specifying the decoded index value of the location ( x, y ),

–	a variable nCbS specifying the size of the current block,

–	The array paletteMap[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette index for each sample in the current block for which the palette mode is not equal to ESCAPE, and [Ed. (GJS): Check "is not" versus "is not equal to" (generally).]

–	The array palette_mode[ x ][ y ], with x = 0..nCbS − 1,  y = 0..nCbS − 1, specifying the palette run mode.

–	The array TC_table[pilot_index][candidate_index], with pilot_index = 0..31,  candidate_index = 0 and 1, specifying transition index table of current CU.

Depending on the palette_mode[  x ][ y ] and x, the following assignments are made:

–	If x % nCbS is not equal to 0,

palette_mode[paletteMap[ x_prev][ y_prev]][ y%2 ] = palette_index, and

palette_mode[palette_index][ (y+1)%2 ] = paletteMap[ x_prev][ y_prev].

### Binarization process

#### 9.3.3.1 General

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element.

specifies the type of binarization process associated with each syntax element and corresponding inputs.

The specification of the truncated Rice (TR) binarization process, the k-th order Exp-Golomb (EGk) binarization process, limited k-th order Exp-Golomb (EGk) binarization process the fixed-length (FL) binarization process, and truncated binarization process are given in subclauses Error: Reference source not found through Error: Reference source not found respectively. Other binarizations are specified in subclauses Error: Reference source not found through Error: Reference source not found. [Ed. (GJS): Should that last cross-reference be to subclause Error: Reference source not found?]

Table 9-1 – Syntax elements and associated binarizations

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_share_flag | FL | cMax = 1 |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx = = 0 ? ( (1<<BitDepthY) − 1 ) : ( (1<<BitDepthC) − 1 ) |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | palette_escape_val | Error: Reference source not found | cIdx, qP |
|  | escape_val_present_flag | FL | cMax = 1 |
|  | palette_mode_flag | 9.3.3.X |  |
|  | palette_index | TB | cMax = adjustedIndexMax |
|  | palette_run | Error: Reference source not found | - |
|  | palette_all_zeros_in_group | FL | cMax = 1 |
|  | palette_last_group | FL | cMax = 1 |

#### 9.3.3.X Binarization process for palette_mode_flag

Input to this process is a request for a binarization for the syntax element palette_mode_flag.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element palette_mode_flag is specified in Table 9-X.

Table 9-X – Binarization for palette_mode_flag

| Value of palette_mode_flag | indexMax > 2 | indexMax <= 2 |
| --- | --- | --- |
|  | Bin string | Bin string |
| COPY_ABOVE | 0 | 0 |
| INDEX | 10 | 1 |
| INDEX_TC | 11 |  |

Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| end_of_subset_one_bit | terminate | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | bypass | na | na | na |
| sao_eo_class_chroma | bypass | bypass | bypass | na | na | na |
| split_cu_flag[ ][ ] | 0,1,2 (subclause 9.3.4.2.2) | na | na | na | na | na |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause 9.3.4.2.2) | na | na | na | na | na |
| intra_bc_flag | 0 | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| palette_mode_flag | 0 | na | na | na | Na | Na |
| palette_share_flag | 0 | na | na | na | Na | na |
| previous_palette_entry_flag | bypass | na | na | na | na | Na |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_transpose_flag | 0, 1 (subclause 9.3.4.2.2) | na | na | na | na | Na |
| palette_escape_val | bypass | bypass | bypass | bypass | bypass | bypass |
| escape_val_present_flag | bypass | na | na | na | na | na |
| palette_run_type_flag | 0, 1 (subclause 9.3.4.2.2X) | 2, 3 (subclause 9.3.4.2.2X) | Na | na | Na | Na |

##### Derivation process of ctxInc using left and above syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) and the variable availableA specifying the availability of the coding block located directly above the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

The assignment of ctxInc for the syntax elements split_cu_flag[ x0 ][ y0 ] and cu_skip_flag[ x0 ][ y0 ] is specified in Table 9-3.

Table 9-3 – Specification of ctxInc using left and above syntax elements

| Syntax element |  | condL | condA | ctxInc |
| --- | --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] |  | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] |  | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_transpose_flag [ x0 ][ y0 ] |  | palette_transpose_flag [ xNbL ][ yNbL ] | na | ( condL && availableL ) |
| palette_run_type_flag [ x0 ][ y0 ] | binIdx = 0 | na | palette_run_type_flag [ xNbA ][ yNbA ] | ( condA && availableA) |
|  | binIdx = 1 | na | palette_run_type_flag [ xNbA ][ yNbA ] | 2 + ( condA == COPY_ABOVE && availableA) |