| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0079 |
| --- | --- |

| Title: | CE6-related: Cross-CU palette colour index prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun1, Jungsun Kim2, Tzu-Der Chuang1, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, jungsun.kim, peter.chuang, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In JCTVC-P0093, it was proposed for palette mode to use the nearest row of the above CU and the nearest column of the left CU to predict colour indexes of the current CU. In this proposal, the method is ported to SCM-2.0 and evaluated again. Experiments show that, compared with SCM-2.0, X.X%, X.X%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

# Problem statement

In SCM-2.0, the palette indices can be predicted from the above decoded indices (or the left decoded indices when palette_transpose_flag is on). For first row pixels in the CU, the above prediction sources exceed the CU boundary. In the current scheme, the above-prediction run is disabled for first row pixels.

JCTVC-P0093 [1] proposed to use pixels in the nearest above row and the nearest left row column in neighboring CUs to generate index prediction sources for the boundary pixels. In this proposal, we port this method to SCM-2.0 and evaluate the performance again.

# Proposed method

We propose to use pixels of the nearest above row (or the nearest left column) in neighboring CUs to generate index prediction source. We call the pixels of the nearest above row (or the nearest left column) neighboring CU pixels (NCPs). For example, Fig. 1 shows the NCPs and their pixel values. The NCPs’ three color component values are indicted by (num1, num2, num3).

Fig. 1.  An example of NCPs.

During reconstruction the colour indices of the current CU, the decoder transforms each NCP value into to a colour index that minimizes the pixel value difference between the NCP value and the pixel value converted from the colour index by the palette table of the current CU. Assume there are N colours in the palette table, e.g. Colour0, Colour1, … ColourN-1, where a colour, Colourj, has three sample values, {Colourj[0], Colourj[1], Colourj[2]}, and a palette index equal to j. For an NCP, NCPi, its transformed index, Idxi is defined as:

Idxi=argmin0≤n<Nk=02NCPi[k]−Colourn[k]				(1)

Fig. 2 shows an example of the palette of the current CU. The decoder uses the palette to transform the NCP values into colour indices.

Fig. 2.  An example of transformed indices of NCPs.

Based on the proposed scheme, the proposed copy-above run mode at CU boundary is shown in Fig. 3. To reconstruct a colour index of the first row of the current CU, if the index is encoded by a copy-above run mode, the decoder reconstruct the index from the corresponding NCP index; otherwise (index run mode), the decoder reconstructs the index from a parsed index and a reference index (adjustedRefIndex). For an index of the first row, if the previous run is copy-above run, the reference index is the above NCP index.

Fig. 3.  Proposed copy-above run mode at CU boundary.

# Experimental results

The proposed method is implemented on top of SCM-2.0. The tests were conducted under the common test conditions in CE6. Table 1 shows the results of the proposed method using SCM-2.0 as the anchor. The encoding and decoding times are accurate. Experiments show that, compared with SCM-2.0, X.X%, X.X%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

Table 1.  Results of the proposed method; anchor: SCM 2.0

# Simplified methods

In the proposed method, the decoder will transform the NCPs to the colour indices. The operations are parallelizable and can be easily done without additional off-chip memory access. The results show that the increasing decoding time is around X%, which is insignificant.

Although we think the complexity increase is acceptable, four simplified methods are provided to show different gain-complexity tradeoffs.

## Infer NCP indices as 0

In JCTVC-P0108[2] and JCTVC-Sxxxx [3], the NCP indices are inferred as 0, and the decoder does not need to transform NCP values to colour indices. We evaluate this method and compare it with the proposed method.

## Reduce searching colours

In this simplification, we search at most eight colours. Eq. (1) is modified as:

Idxi=argmin0≤n<min(N,8)k=02NCPi[k]−Colourn[k]				(2)

In SCM-2.0, the maximal palette size is 31. Therefore, the worst case decoding complexity increase is roughly reduced by 75% (from searching 31 colours to 8 colours).

## Reduce searching colour components

In this simplification, we only use the first colour component to search the colour index. Eq. (1) is modified as:

Idxi=argmin0≤n<NNCPi[0]−Colourn[0]				(3)

Therefore, the decoding complexity increase is reduced by 66% (from searching 3 components to 1 component).

## Directly copy pixel value

In this simplification, we propose to directly use a NCP value to predict a current pixel instead of the transformed index of the NCP. In other words, the transform process, Eq. (1), can be totally removed. Fig. 4 shows an example. To reconstruct the colour indices of the first row of the current CU, if the indices are encoded by a copy-above run, the decoder reconstructs them from the NCP values instead of the transformed indices of NCPs. Note that the decoder does not know the transformed indices of NCPs. If the reference index for an index run mode is from an NCP, the redundant index removal will be disabled.

Fig.4.  Simplified method: copy pixel value.

Table 2 shows the results of the proposed method and the four simplified methods using SCM-2.0 as the anchor. The encoding amd decoding times are accurate.

Table 2.  Results of the proposed method and the four simplified method; anchor: SCM-2.0 [Results to be added]

# Conclusion

In this proposal, the proposed method in JCTVC-P0093 is evaluated on top of SCM-2.0. It is proposed to use the nearest row of the above CU (or the nearest column of the left CU) to predict colour indices of the CU. Compared with SCM-2.0, it is reported to achieve X.X%, X.X%, and 1.0% BD-rate savings for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, “Non-RCE4: Cross-CU major color index prediction,” JCTVC-P0093, Jan. 2014.

- X. Guo, Y. Lu, S. Li, “RCE4 Test1: Major-color-based screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0108, Jan. 2014.

- CE6 Test C.3