| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0082 |
| --- | --- |

| Title: | CE9: Result of Test A.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Robert Cohen Xingyu Zhang |  |  |
| Source: | Microsoft Corp. Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document reports the experimental results of test A.2 in CE9. The intra boundary filter, which is enabled for the first color component, is also enabled for the second and third color components. The experimental results show the proposed modification achieves 1.5% bit-rate saving on average for RGB, Mixed Content, 1440p, AI lossy coding.

# Introduction

The intra boundary filter is enabled for the first component using DC, Horizontal, and Vertical prediction. This document proposes to also enable boundary filter for the second and third component in 444 chroma sampling formats.

# Solution

Detailed text modification is provided in Section 6.

# Experimental results

The HM-15.0+RExt-8.0+SCM-2.0 default coding configuration is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results for lossy and lossless coding are shown in Table 1 and 2, respectively. Detailed experimental results could be found in the attached spreadsheets.

Table 1. Experimental results for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.8% | -0.4% | -0.5% |
| RGB, mixed content, 1440p | -1.5% | -0.9% | -1.0% |
| RGB, mixed content, 1080p | -1.3% | -0.9% | -0.8% |
| RGB, Animation, 720p | -0.5% | -0.3% | -0.4% |
| RGB, camera captured, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -1.0% | -0.7% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.2% | -0.3% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -1.2% | -0.4% | -0.7% |
| RGB, mixed content, 1080p | -0.8% | 0.1% | 0.5% |
| RGB, Animation, 720p | -0.2% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | -0.2% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Experimental results for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R0314, “On intra chroma boundary filtering,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R1109, “Description of Core Experiment 9 (CE9): IBF/CCP interdependency,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

One high level flag of intra_chroma_boundary_filter_enable_flag is added at the SPS or PPS level. When this flag is not present, it is inferred as 0.

##### 8.4.5.2.5	Specification of intra prediction mode INTRA_DC

……

- Depending on the value of the colour component index cIdx, the following applies:

- If cIdx is equal to 0 or intra_chroma_boundary_filter_enable_flag is equal to 1, and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-1)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-2)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-3)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-4)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-5)

##### 8.4.5.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 or intra_chroma_boundary_filter_enable_flag is equal to 1, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-6)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 or intra_chroma_boundary_filter_enable_flag is equal to 1, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-7)