| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0084 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 8 and HEVC Screen Content Coding Extensions Test Model 2 with AVC High 4:4:4 Predictive profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan | Email: | libin@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Main 4:4:4 12 Test Model 8 (using 8, 10, and 12 bit configurations), HEVC SCC Test Model 1 (SCM 2) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, and JCTVC-R0101 – updating the results by using the latest available reference software (JM-18.6, HM-15.0+RExt-8.0, HM-15.0+RExt-8.0+SCM-2.0), profile and test model designs, and the RExt and SCC test sequences. The overall results indicate that for RExt CTC sequences, RExt Main 4:4:4 12 Profile saves 25%, 33%, and 38% bit rate for AI-MT, RA-MT and LB-MT relative to AVC High 4:4:4 Predictive Profile. For SCC CTC sequences, RExt Main 4:4:4 12 Profile provides an even larger benefit. The overall results also indicate that for screen content sequences, the HEVC SCC Test Model 2 also improves quite substantially over HEVC SCC Test Model 1. For example, for RGB text and graphics with motion (TGM) 1080p sequences, the HEVC SCC Test Model 2 saves 33%, 23%, and 18% bits for AI, RA and LB lossy coding over HEVC SCC Test Model 1. Relative to what was reported most recently in JCTVC-R0101, the bit rate savings for SCC sequences mainly comes from the adoption of new coding tools, especially residual adaptive color transform and palette mode.

# Performance Comparison

We use HM-15.0+RExt-8.0, HM-15.0+RExt-8.0+SCM-2.0 and JM-18.6 in the tests. The JM software was configured with the same coding structure and referencing structure as HM using the “HM-like” configurations. High 4:4:4 Predictive profile is used in JM. Both the RExt CTC [11] and SCC CTC [12] test conditions (e.g. internal bit depth, intra period, QP setting, etc.) are followed in the comparison.

# Experimental result

It should be noted the encoding/decoding times, especially decoding times, are not so reliable – since the experiments were performed using a computing "cluster".

The results are shown in Tables 1–5 below. For the lossy coding results, the numbers are BD-Rate, for which a negative number means the codec listed on right provides better compression performance. For lossless coding result, the numbers are bit saving percentages, for which a positive number means the codec listed on the right is better.

Table 1 – JM18.6 VS. HM-15.0+RExt-8.0 for RExt CTC sequences

Table 2 – JM18.6 VS HM-15.0+RExt-8.0+SCM-2.0 for SCC CTC sequences, lossy coding

Table 3 – JM18.6 VS HM-15.0+RExt-8.0+SCM-2.0 for SCC CTC sequences, lossless coding

Table 4 – HM-14.0+RExt-7.0+SCM-1.0 VS HM-15.0+RExt-8.0+SCM-2.0 for SCC CTC sequences, lossy coding

Table 5 – HM-14.0+RExt-7.0+SCM-1.0 VS HM-15.0+RExt-8.0+SCM-2.0 for SCC CTC sequences, lossless coding

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-N1005, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

- JCTVC-O0184, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 23 Oct. – 1 Nov. 2013.

- JCTVC-P0213, “Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-R0101, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 7 and HEVC Screen Content Coding Extensions Test Model 1 with AVC High 4:4:4 Predictive profile,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-P1006, “Common test conditions and software reference configurations for HEVC range extensions,” San José, US, 9–17 Jan. 2014.

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.