| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0085_r1 |
| --- | --- |

| Title: | Adaptive motion vector resolution for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan You Zhou Bruce Lin |  |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes and reports performance measurements for disabling fractional pixel motion compensation in high-level syntax, as previously proposed in JCTVC-P0277, JCTVC-Q0155 and JCTVC-R0106. For many screen contents, the spatial displacements are of full pixel and the encoder can be notified accordingly by the application system. Not to consider fractional-pel displacement not only save bits on motion vectors, it also reduces the complexity of both the encoder and decoder. The proposed modification does not change the syntax or parsing process. A method to determine the MV resolution in SCM-2.0 encoder is also developed. The modification is shown to improve coding efficiency as much as 7.9% for some screen content. For RGB text and graphics with motion (TGM) 1080p sequences, 2.7% and 4.0% bit saving on average is obtained for RA and LB lossy coding. The experimental results show that most coding gain could be obtained without increasing the encoding running time.

# Introduction

The idea is based on our observation that when encoding video that is captured from a computer desktop, most of the motion vectors in the encoded bitstream point have integer values; very few of them are at fractional positions. While encoders normally interpret motion vectors in bitstreams as ¼ pixel offsets, we can often save bit rate by encoding these motion vectors for screen coding applications as 1 pixel offsets, reducing the precision of the motion vectors by a factor of four, and thereby reducing the number of bits needed to code them. Considering that fractional motion vector values are still very useful for camera-captured content, we propose to enable encoders to select the motion vector precision in a flexible way: when the video source is from a captured screen and there is no fractional pixel motion, we transmit the motion vector in units of 1 pixel, and for camera-content video we transmit motion vectors with ¼-pixel fractional precision.

# Solution

The encoder signals a flag at the slice header level to indicate whether integer-precision MV or quarter-pel precision MVs are used for the current slice. The encoder can then make decisions for the MV precision on a slice-by-slice basis. As tested, each slice is an entire picture. When integer-precision MV is used, the decoder only needs interpret the decoded MV as an integer MV when performing the motion compensation process – it does not need to change the MV prediction process. This is slightly different than previously proposed. Here, we propose to change only the interpretation of the MV values – not their prediction or parsing processes. This eliminates the need to round MV predictors from quarter-pel precision to integer-pel precision. As proposed here, the decoder performs the ordinary process for MV prediction and delta MV decoding, regardless of whether the derived value represents a quarter-pel MV or an integer MV. (The decoder also ignores which MV precision was used for the reference pictures when performing temporal MV prediction – it simply performs operations on values that may either represent quarter-sample units or whole-integer units.)

Encoder method to determine the MV resolution: To decide the MV precision of one picture, the encoder performs the following check with the help of hash. For every non-overlapped 8x8 block in a picture, the encoder checks whether it can find a matching block by hash in the first reference picture in list 0. The blocks are classified into the following categories.

- C: number of blocks matching with collocated block.

- S: number of blocks not matching with collocated block but belong to smooth region. For smooth region, it means every column has a single pixel value or every row has a single pixel value.

- M: number of blocks not belonging to C or S but can find a matching block by hash value.

T is the total number of blocks in one picture. CSMRate = (C+S+M)/T, MRate = M/T. AverageCSMRate is the average CSMRate of current picture and the previous 31 pictures. AverageMRate is the average MRage of the current picture and the previous 31 pictures.

The MV resolution is determined as:

- If CSMRate < 0.8, use quarter-pel MV.

- Otherwise, if C == T, use integer-pel MV.

- Otherwise, if AverageCSMRate < 0.95, use quarter-pel MV.

- Otherwise, if M > (T−C−S)/3, use integer-pel MV.

- Otherwise, if CSMRate > 0.99 and MRate > 0.01, use integer-pel MV.

- Otherwise, if AverageCSMRate + AverageMRate > 1.01, use integer-pel MV.

- Otherwise, use quarter-pel MV.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results for lossy and lossless coding (single-pass encoding) are shown in Table 1 and 2, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding (single-pass encoding)

Table 2 Experimental results for lossless coding (single-pass encoding)

Results of multi-pass encoding. Picture-based multi-pass encoding is used to select integer-pel MV or quarter-pel MV. The encoder selects the one with smaller picture RD cost as the better choice of the current picture. The lossy and lossless coding results are shown in table 3 and 4, respectively.

Table 3 Experimental results for lossy coding (multi-pass encoding)

Table 4 Experimental results for lossless coding (multi-pass encoding)

From table 3 and 4, we can know that one-pass encoding can achieve most of the gain brought by adaptive MV resolution.

Results of non-normative method. Picture-based multi-pass encoding is used. In one pass, only integer-pel MV is allowed and in the other pass, quarter-pel MV is allowed. The encoder selects the one with smaller picture RD cost as the better choice of the current picture. The lossy and lossless coding results are shown in table 5 and 6, respectively.

Table 5 Experimental results for lossy coding (non-normative method)

Table 6 Experimental results for lossless coding (non-normative method)

From table 5 and 6, we can know that non-normative method does not bring as large gain as normative method.

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-P0277, “Motion Vector Resolution Control for Screen Content Coding,” San José, US, 9–17 Jan. 2014.

- JCTVC-Q0155, “RCE1: Subtest 1 - Motion Vector Resolution Control,” Valencia, ES, 27 March – 4 April 2014.

- JCTVC-R0106, “Adaptive motion vector resolution for screen content,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

## Text for integer-pel precision MV and 1/4-pel precision MV adaptation

- 7.3.2.2.3 Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| adaptive_mv_resolution_enabled_flag | u(1) |
| } |  |

- 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if (adaptive_mv_resolution_enabled_flag) |  |
| use_integer_mv_flag | u(1) |
| } |  |
| …… |  |

- 7.4.3.2.3	Sequence parameter set screen extensions semantics

adaptive_mv_resolution_enabled_flag equal to 1 specifies that use_integer_mv_flag may be signaled at slice header. adaptive_mv_resolution_enabled_flag equal to 0 specifies that use_integer_mv_flag is not signaled at slice header.  When not present, the value of adaptive_mv_resolution_enabled_flag is inferred to be equal to 0.

- 7.4.7.1	General slice segment header semantics

use_integer_mv_flag equal to 1 specified that the motion vector precision is integer in the decoding process of current slice. use_integer_mv_flag equal to 0 specified that the motion vector precision is qual-pixel in the decoding process of current slice. When not present, the value of use_integer_mv_flag is inferred to be equal to 0.

- 8.5.3.3	Decoding process for inter prediction samples

- 8.5.3.3.1	General

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	the luma motion vectors mvL0 and mvL1,

–	when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

When use_integer_mv_flag is equal to 1, mvLX and mvCLX (where X equals to 0 or 1) are modified as mvLX <<= 2, mvCLX <<= 2.

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr, and predSamplesL1Cr be (nPbW / SubWidthC)x(nPbH / SubHeightC) arrays of predicted chroma sample values.

……

## Text for integer-pel precision MV, 1/2-pel precision  and 1/4-pel precision MV adaptation

- 7.3.2.2.3 Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| adaptive_mv_resolution_enabled_flag | u(1) |
| } |  |

- 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if (adaptive_mv_resolution_enabled_flag) |  |
| mv_resolution_idc | ue(v) |
| } |  |
| …… |  |

- 7.4.3.2.3	Sequence parameter set screen extensions semantics

adaptive_mv_resolution_enabled_flag equal to 1 specifies that mv_resolution_idc may be signaled at slice header. adaptive_mv_resolution_enabled_flag equal to 0 specifies that mv_resolution_idc is not signaled at slice header.  When not present, the value of adaptive_mv_resolution_enabled_flag is inferred to be equal to 0.

- 7.4.7.1	General slice segment header semantics

mv_resolution_idc equal to 0 specified that the motion vector precision is 1/4-pel in the decoding process of current slice. mv_resolution_idc equal to 1 specified that the motion vector precision is 1/2-pel in the decoding process of current slice. mv_resolution_idc equal to 2 specified that the motion vector precision is integer-pel in the decoding process of current slice. When not present, the value of mv_resolution_idc is inferred to be equal to 0.

- 8.5.3.3	Decoding process for inter prediction samples

- 8.5.3.3.1	General

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	the luma motion vectors mvL0 and mvL1,

–	when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

If use_integer_ mv_resolution_idc to 1, mvLX and mvCLX (where X equals to 0 or 1) are modified as mvLX <<= 1, mvCLX <<= 1.

Otherwise, if use_integer_ mv_resolution_idc to 2, mvLX and mvCLX (where X equals to 0 or 1) are modified as mvLX <<= 2, mvCLX <<= 2.

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr, and predSamplesL1Cr be (nPbW / SubWidthC)x(nPbH / SubHeightC) arrays of predicted chroma sample values.

……