| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0086 |
| --- | --- |

| Title: | On residual adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary Sullivan |  |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes several aspects to fix and modify the current design of residual adaptive color transform, including: 1) condition of encoding ACT flag; (2) the interaction on bit depth of ACT and CCP; (3) QP modification process for ACT; (4) unification of lossy conversion and lossless conversion; (5) residual_adaptive_colour_transform_enabled_flag signaling; (6) ACT for different bit depth.

# Introduction

Residual adaptive color transform brings significant coding efficiency improvement, especially for RGB sequences, and it has been adopted into screen content coding extension [2]. This document proposes several aspects to improve the design of ACT.

# Solution

## Condition of encoding ACT flag

In the current design, the ACT flag is signaled if the following conditions are satisfied.

- The current CU has non-zero rqt_root_cbf (has residue).

- The current CU is coded in inter mode, intra BC mode, or intra mode with intra_chroma_pred_mode equal to DM.

But in the current software and the working draft, when the current CU is intra CU, only the intra_chroma_pred_mode of the first PU is checked. It is not proper for if there are multiple PUs in the current CU. This document proposes to check the intra_chroma_pred_mode of all the PUs in the current CU to determine encoding ACT flag or not.

## Interaction on bit depth of ACT and CCP

In the current design, the CCP process take the bit depth of the first color component and the bit depth of the second and the third color component into consideration. However, when lossless ACT is enabled, the bit depth of the second and the third components will be increased by 1. This document proposes to take the bit depth increase into consideration in CCP design.

## QP modification process for ACT

In the current design, when ACT is used, the three components of Y, Cg, and Co will use a QP offset of −5, −5, and −3, respectively. In the decoding process, the qP will be used in a modulus operation (e.g. levelScale[ qP%6 ] in 8-279). For some small QP values, qP will be negative numbers. However, the modulus operation is not defined for negative numbers (5.2). This document proposes to fix the problem. A simple way to fix the problem is adding a max(qP, 0) operation after the modification of qP. This way will have no impact on the coding efficiency of CTC. The other way is instead of using negative QP offset, using a positive QP offset instead, e.g., using 1, 1, and 3 as the QP offset together with increasing the bit depth by 1. This method will be discussed in details in the next part.

## Unification of lossy conversion and lossless conversion

In the current design, lossy coding and lossless coding use a different color transform method. In lossless mode, the conversion is reversible. However, using reversible conversion does not harm the coding efficiency of lossy coding. To unify the design, this document proposes to use the reversible design for both lossy coding and lossless coding.

Furthermore, when using reversible conversion, the bit depth of the second and the third components will be increased by 1. Thus, the QP offset of the second and the third components are set to 1 and 3, respectively.

To solve the negative QP problem together, this document also proposes to increase bit depth by 1 for the first component when lossy coding is used. Thus, the QP offset for the first component is 1.

## residual_adaptive_colour_transform_enabled_flag signaling

In the current design, ACT flag is signaled at SPS level. However, another residual modification flag, CCP flag is signaled at PPS level. This document proposes to harmonize the design, e.g., signaling ACT flag at PPS level.

If the ACT flag is signaled at SPS level, it should be conditionally decoded based on chroma format.

The impact on the coding efficiency under CTC is negligible.

## ACT for different bit depth

In the current design, when ACT is enabled, the three components are assumed to have the same bit depth. However, if the bit depth of the three components are different, normalization is proposed to be added during the ACT process.

For lossy coding in the current design, the forward conversion is

Y = ((G<<1) + R + B + 2)>>2

Cg = ((G<<1) − R − B + 2)>>2

Co = ((R − B)<<1 + 2)>>2

The backward conversion is

G = Y + Cg

B = Y – Cg − Co

R = Y – Cg + Co

This document proposes to consider the normalization into the conversion.

BitDepthMax = Max(BitDepthY, BitDepthC)

For lossy coding, the forward conversion is

G <<= (BitDepthMax − BitDepthY)

B <<= (BitDepthMax – BitDepthC)

R <<= (BitDepthMax – BitDepthC)

Y = ((G<<1) + R + B + 2)>>2

Cg = ((G<<1) − R − B + 2)>>2

Co = ((R − B)<<1 + 2)>>2

The backward conversion is

G = Y + Cg

B = Y – Cg − Co

R = Y – Cg + Co

OY = BitDepthMax − BitDepthY == 0 ? 0 : 1<<(BitDepthMax − BitDepthY – 1)

OC = BitDepthMax – BitDepthC == 0 ? 0 : 1<<(BitDepthMax – BitDepthC – 1)

G = (G+OY)>>(BitDepthMax − BitDepthY)

B = (B+OC)>>(BitDepthMax – BitDepthC)

R = (R+OC)>>(BitDepthMax – BitDepthC)

Detailed modification are provided in Section 6.

Another solution is simply disable ACT is different bit depth is used for different color components.

There is no coding efficiency impact under CTC.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables. More detailed experimental results could be found in the attached spreadsheets.

## Condition of encoding ACT flag

Table 1 Experimental results for lossy coding

Table 2 Experimental results for lossless coding

## Bit depth interaction of ACT and CCP

Table 3 Experimental results for lossless coding

## QP modification process for ACT

No performance change under CTC is adding max operation in QP modification process. The results of using positive QP offset will be provided in the next part.

## Unification of lossy conversion and lossless conversion

There is no performance changing for lossless coding.

Table 4 Experimental results for lossy coding

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R0147, “SCCE5 Test 3.2.1: In-loop color-space transform,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

## Condition of encoding ACT flag

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| …… |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ (PartMode== PART_2Nx2N && intra_chroma_pred_mode[ x0 ][ y0 ] = = 4) ¦¦ (intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 && intra_chroma_pred_mode[ x0+nCbS/2 ][ y0 ] = = 4 && intra_chroma_pred_mode[ x0 ][ y0+nCbS/2 ] = = 4 && intra_chroma_pred_mode[ x0+nCbS/2 ][ y0+nCbS/2 ] = = 4) ) ) |  |
| cu_residual_act_flag |  |
| …… |  |

## Bit depth interaction of ACT and CCP

### 8.6.6 Residual modification process for transform blocks using cross-component prediction

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

Output of this process is the modified (nTbS)x(nTbS) array r of residual samples.

If cu_transquant_bypass_flag[xTbY][yTbY] is equal to 0 or cu_residual_act_flag[xTbY][yTbY] is equal to 0, the (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-1)
			( ( rY[ x ][ y ]  <<  BitDepthC )  >>  BitDepthY ) )  >>  3

Otherwise (cu_transquant_bypass_flag[xTbY][yTbY] is equal to 1 and cu_residual_act_flag[xTbY][yTbY] is equal to 1), the (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-2)
			( ( rY[ x ][ y ]  <<  (BitDepthC+1) )  >>  BitDepthY ) )  >>  3

## QP modification process for ACT

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = max(0, Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0) )	(8-3)

–	Otherwise, if cIdx is equal to 1,

qP = max(0, Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0) )	(8-4)

–	Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0) )	(8-5)

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-6)

bdShift = Max( 20 − bitDepth, extended_precision_processing_flag ? 11 : 0 )	(8-7)

tsShift = ( extended_precision_processing_flag ? Min( 5, bdShift − 2 ) : 5 ) + Log2( nTbS )	(8-8)

## Unification of lossy conversion and lossless conversion

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5+1 : 0)	(8-9)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5+1 : 0)	(8-10)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3+3 : 0)	(8-11)

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-12)

bdShift = Max( 20 − bitDepth, extended_precision_processing_flag ? 11 : 0 )	(8-13)

tsShift = ( extended_precision_processing_flag ? Min( 5, bdShift − 2 ) : 5 ) + Log2( nTbS )	(8-14)

### 8.6.8	Residual modification process for transform blocks using adaptive colour transform

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a variable blkSize specifying the block size,

–	an (blkSize)x(blkSize) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	an modified (blkSize)x(blkSize) array rY of luma residual samples,

–	an modified (blkSize)x(blkSize) array rCb of chroma residual samples,

–	an modified (blkSize)x(blkSize) array rCr of chroma residual samples.

The (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr are modified as follows:

- If cu_transquant_bypass_flag is equal to 1, tThe (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

If cu_transquant_bypass_flag is equal to 0, rY[ x ][ y ] = (rY[ x ][ y ]+1)>>1

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )
rY[ x ][ y ] = tmp + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − (  rCr[ x ][ y ]  >>  1 )
rCr[ x ][ y ] = rCb[ x ][ y ] + rCr[ x ][ y ]

- Otherwise (cu_transquant_bypass_flag is equal to 0), the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ] 
rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − rCr[ x ][ y ]
rCr[ x ][ y ] = tmp + rCr[ x ][ y ]

## residual_adaptive_colour_transform_enabled_flag signaling

### Move residual_adaptive_colour_transform_enabled_flag to PPS

##### 7.3.2.2.3	Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| } |  |

##### 7.3.2.3.1	General picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| pps_extension_present_flag | u(1) |
| if( pps_extension_present_flag ) { |  |
| pps_range_extensions_flag | u(1) |
| pps_scc_extensions_flag | u(1) |
| pps_extension_76bits | u(76) |
| } |  |
| if( pps_range_extensions_flag ) |  |
| pps_range_extensions( ) |  |
| if( pps_scc_extensions_flag ) |  |
| pps_scc_extensions( ) |  |
| if( pps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

##### 7.3.2.3.3	Picture parameter set scc extensions syntax

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| } |  |

##### 7.4.3.2.3	Sequence parameter set scc extension

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0.

##### 7.4.3.3.3	Picture parameter set scc extensions semantics

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0. When ChromaArrayType is not equal to 3, it is a requirement of bitstream conformance that the value of residual_adaptive_colour_transform_enabled_flag shall be equal to 0.

### Conditionally encoding of residual_adaptive_colour_transform_enabled_flag

##### 7.3.2.2.3	Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| if( chroma_format_idc = = 3 ) |  |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| } |  |

## ACT for different bit depth

### Scaling process for transform coefficients

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is the (nTbS)x(nTbS) array d of scaled transform coefficients with elements d[ x ][ y ].

The variables log2TransformRange, bdShift, coeffMin and coeffMax are derived as follows:

–	If cu_residual_act_flag[xTbY][yTbY] is equal to 1,

log2TransformRange = extended_precision_processing_flag ? Max( 15, Max(BitDepthY, BitDepthC) + 6 ) : 15	(8-15)

bdShift = Max(BitDepthY, BitDepthC) + Log2( nTbS ) + 10 − log2TransformRange	(8-16)

coeffMin = Min(CoeffMinY, CoeffMinC)		(8-17)

coeffMax = Max(CoeffMaxY, CoeffMaxC)		(8-18)

–	Otherwise, if cIdx is equal to 0,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthY + 6 ) : 15	(8-19)

bdShift = BitDepthY + Log2( nTbS ) + 10 − log2TransformRange	(8-20)

coeffMin = CoeffMinY		(8-21)

coeffMax = CoeffMaxY		(8-22)

–	Otherwise,

log2TransformRange = extended_precision_processing_flag ? Max( 15, BitDepthC + 6 ) : 15	(8-23)

bdShift = BitDepthC + Log2( nTbS ) + 10 − log2TransformRange	(8-24)

coeffMin = CoeffMinC		(8-25)

coeffMax = CoeffMaxC		(8-26)

### Residual modification process for transform blocks using cross-component prediction

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

Output of this process is the modified (nTbS)x(nTbS) array r of residual samples.

If cu_residual_act_flag [xTbY][yTbY] is equal to 1 and cu_transquant_bypass_flag[xTbY][yTbY] is equal to 0, the (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-27)
			 rY[ x ][ y ] )  >>  3

Otherwise if cu_residual_act_flag [xTbY][yTbY] is equal to 1 and cu_transquant_bypass_flag[xTbY][yTbY] is equal to 1, the (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-28)
			( rY[ x ][ y ]  <<  1 ) )  >>  3

Otherwise (cu_residual_act_flag [xTbY][yTbY] is equal to 0), the (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-29)
			( ( rY[ x ][ y ]  <<  (BitDepthC) )  >>  BitDepthY ) )  >>  3

The (nTbS)x(nTbS) array of residual samples r with  x = 0..nTbS − 1, y = 0..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-30)
			( ( rY[ x ][ y ]  <<  BitDepthC )  >>  BitDepthY ) )  >>  3

### 8.6.8	Residual modification process for transform blocks using adaptive colour transform

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a variable blkSize specifying the block size,

–	an (blkSize)x(blkSize) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	an modified (blkSize)x(blkSize) array rY of luma residual samples,

–	an modified (blkSize)x(blkSize) array rCb of chroma residual samples,

–	an modified (blkSize)x(blkSize) array rCr of chroma residual samples.

The (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr are modified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )
rY[ x ][ y ] = tmp + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − (  rCr[ x ][ y ]  >>  1 )
rCr[ x ][ y ] = rCb[ x ][ y ] + rCr[ x ][ y ]
BitDepthMax = Max(BitDepthY, BitDepthC)
rY[ x ][ y ] = (rY[ x ][ y ])>>(BitDepthMax − BitDepthY)
rCb[ x ][ y ] = (rCb[ x ][ y ])>>(BitDepthMax – BitDepthC)
rCr[ x ][ y ]  = (rCr[ x ][ y ] )>>(BitDepthMax – BitDepthC)

- Otherwise (cu_transquant_bypass_flag is equal to 0), the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

tmp = rY[ x ][ y ] − rCb[ x ][ y ] 
rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − rCr[ x ][ y ]
rCr[ x ][ y ] = tmp + rCr[ x ][ y ]
bitDepthMax = Max(bitDepth
BitDepthMax = Max(BitDepthY, BitDepthC)
OY = BitDepthMax − BitDepthY == 0 ? 0 : 1<<(BitDepthMax − BitDepthY – 1)
OC = BitDepthMax – BitDepthC == 0 ? 0 : 1<<(BitDepthMax – BitDepthC – 1)
rY[ x ][ y ] = (rY[ x ][ y ]+OY)>>(BitDepthMax − BitDepthY)
rCb[ x ][ y ] = (rCb[ x ][ y ]+OC)>>(BitDepthMax – BitDepthC)
rCr[ x ][ y ]  = (rCr[ x ][ y ] +OC)>>(BitDepthMax – BitDepthC)