| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0087_r1 |
| --- | --- |

| Title: | Non-CE2: on block vector predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes improvements on block vector predictor. First, this document proposes to test BVPs under full RDO for Intra BC mode (non-normative modification). The experimental results show that it brings 2.6% bit saving for RGB TGM 1080p AI lossy coding. Second, this document proposes to modify the BVP construction process. The experimental results shows about 3.8% bit saving for RGB TGM AI lossy coding.

# Introduction

This document proposes improvements on block vector predictor. First, this document proposes to test BVPs under full RDO for Intra BC mode (non-normative change). The experimental results show that it brings 2.6% bit saving for RGB TGM 1080p AI lossy coding. Second, this document proposes to modify the BVP construction process. The experimental results shows about 3.8% bit saving for RGB TGM AI lossy coding.

# Solution

## Test BVP under full RDO

This document proposes to test BVPs under full RDO for Intra BC mode. The modification is non-normative. To control the additional encoding complexity brought by adding new full RDOs, this document also proposes to only test non-padding BVPs under full RDO.

## Modifications on BVP construction process

In the current design, the BVP comes from left block and the above block, together the two last BVs. In the proposed method, the BVP construction process checks the similar position as the merge candidate construction process. The two last BVs are also used in the BVP construction process. The number of BVPs is still two and there is no syntax changes introduced by the proposed method.

The BVPs are constructed in the following order:

- Left

- Above

- Above right

- Left bottom

- Above left

- MvLast[0]

- MvLast[1]

- (−2*CUWidth, 0)

- (−CUWidth, 0)

For (1) to (8), if there already exists a same BVP, it is not added into the BVP list. For (1) to (7), if the BVP is not valid for the current PU (e.g., the BVP may result in predicting from unconstructed region), it is also not added into the BVP list.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables. More detailed experimental results could be found in the attached spreadsheets.

## Test BVP in RDO process

Table 1 Experimental results for lossy coding

Table 2 Experimental results for lossless coding

## BVP modifications

Table 3 Experimental results for lossy coding

Table 4 Experimental results for lossless coding

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

### 8.4.4	Derivation process for block vector components in intra block copying prediction mode

……

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- The variable LastBvIntra[ 0 ][ compIdx ] and LastBvIntra[ 1 ][ compIdx ]specifies the last two block vector predictor. If this process is invoked for the first time for the current coding tree unit, LastBvIntra[ compIdx ] is derived as follows:

LastBvIntra[ 0 ][ 0 ] = −2* nCbS; LastBvIntra[ 0 ][ 1 ] = 0

LastBvIntra[ 1 ][ 0 ] = −nCbS; LastBvIntra[ 1 ][ 1 ] = 0

Depending upon the number of times this process has been invoked for the current coding tree unit, subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbSw, the luma prediction block height nPbSh, the last block vectors LastBvIntra, and the partition index blkIdx as inputs, and the block vector predictor BvpIntra[ xPb ][ yPb ] as the output, and bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to BvdIntra[ xPb ][ yPb ][ compIdx ] + BvpIntra[ xPb ][ yPb ][ compIdx ] [Ed. (GJS): Needs further formatting cleanup.]

When bvIntra[ xPb ][ yPb ][ 0 ] is not equal to LastBvIntra[ 0 ][ 0 ] or bvIntra[ xPb ][ yPb ][ 1 ] is not equal to LastBvIntra[ 0 ][ 1 ], the value of LastBvIntra[ 1 ][ compIdx ] is updated to be LastBvIntra[ 0 ][ compIdx ], and the value of LastBvIntra[ 0 ][ compIdx ] is updated to be bvIntra[ xPb ][ yPb ][ compIdx ].

…

#### 8.4.4.1	Derivation process for intra block copy block vector prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- the reference index of the current prediction unit partition refIdxL2

- the last block vector LastBvIntra

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is the block vector prediction BvpIntra[ xPb ][ yPb ].

The block vector predictor BvpIntra is derived in the following ordered steps:

numBVP is set to 0.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1 , yPb + nPbH − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW − 1,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb + nPbW,  yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1,  yPb + nPbH ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the neighbouring block location ( xPb − 1, yPb − 1 ) and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN as output. If availableFlagN is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to bvIntraN and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != bvIntraN, bvpIntraList[1] is set to bvIntraN and numBVP is set to 2.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ LastBvIntra[0][0]+nPbW-1 , yPb+ LastBvIntra[0][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to LastBvIntra[0] and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != LastBvIntra[0], bvpIntraList[1] is set to LastBvIntra[0] and numBVP is set to 2.

- The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ LastBvIntra[1][0]+nPbW-1 , yPb+ LastBvIntra[1][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to LastBvIntra[1] and numBVP is set to 1.

  - Otherwise, if numBVP is equal to 1, if bvpIntraList[0] != LastBvIntra[1], bvpIntraList[1] is set to LastBvIntra[1] and numBVP is set to 2.

- If numBVP is less than 2, the following steps apply:

  - If numBVP is equal to 0, bvpIntraList[0] is set to (−2*nCbS, 0) and bvpIntraList[1] is set to (nCbS, 0),

  - Otherwise, if numBVP is equal to 1 and bvpIntraList[0] != (−2*nCbS, 0), bvpIntraList[1] is set to (−2*nCbS, 0),

  - Otherwise (numBVP is equal to 1 and bvpIntraList[0] == (−2*nCbS, 0)), bvpIntraList[1] is set to (−nCbS, 0).

- The derivation process for block vector predictor candidates from neighbouring prediction unit partitions in subclause Error: Reference source not found is invoked with the luma coding block location ( xCb, yCb ), the coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, and the partition index partIdx as inputs, and the availability flags availableFlagN and the block vectors bvIntraN, with N being replaced by A or B, as output.

- The variables bvpIntraVirtual[ i ][ j ] (with i and j being equal to 0 or 1) specify two virtual block vector predictors, and are derived as follows:

bvpIntraVirtual[ 0 ][ 0 ] = LastBvIntra[ 0 ][ 0 ], bvpIntraVirtual[ 0 ][ 1 ] = LastBvIntra[ 0 ][ 1 ];

bvpIntraVirtual[ 1 ][ 0 ] = LastBvIntra[ 1 ][ 0 ], bvpIntraVirtual[ 1 ][ 1 ] = LastBvIntra[ 1 ][ 1 ];

[Ed. (GJS): Correct equation formatting (e.g., adding equation numbers).]

- The block vector predictor candidate list, bvpIntraList, is constructed as follows:

i = 0	 [Ed. (GJS): Correct equation formatting (e.g., adding equation numbers).]
if(availableFlagA )
		bvpIntraList[ i++ ] = bvIntraA
if( availableFlagB  &&  bvIntraA  !=  bvIntraB )
		bvpIntraList[ i++ ] = bvIntraB
for( j=0; j < 2  &&  i < 2; j++)
		if( ( j = =1 | | bvpIntraList[ 0 ]  !=  bvpIntraVirtual[ j ] )
			bvpIntraList[ i++ ] = bvpIntraVirtual[ j ]

[Ed. (GJS): Check/fix usage of bvpIntraVirtual with two indices versus with one index and the assumption of being a two-dimensional vector. What do we do elsewhere for similar uses?]

- The block vector bvIntra[ xPb ][ yPb ] is derived as follows.

for (i =0; i< 2; i++)
	BvpIntra[ xPb ][ yPb ][ i ]= bvpIntraList[ i ]

[Ed. (GJS): Correct equation formatting (e.g., adding equation numbers). Is this supposed to say bvIntra or BvpIntra? Check/fix substring problem.]

#### 8.4.4.2	Derivation process for intra block copy block vector prediction candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

- a luma location ( xN, yN ) specifying the neighbouring sample to obtain the block vector predictor relative to the top-left luma sample of the current picture,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A or B):

- the block vectors bvIntraN of the neighbouring prediction units.

- the availability flags availableFlagN of the neighbouring prediction units.

The variables bvIntraNA[ compIdx ] specify the left neighbouring block vector predictor with compIdx being 0 or 1. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. The variable availableFlagN specifies the availability flags of the left and above neighbouring blocks, with N being equal to A or B. bvIntraN[ compIdx ] and availableFlagN are derived as follows:

bvIntraN[ compIdx ] is set equal to 0 for compIdx being equal to 0 and 1 and N being equal to A and B;

availableFlagN is set equal to FALSE for N being equal to A and B.

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb − 1 , yPb + nPbH − 1 ) ( xN , yN ), and the partition index partIdx as inputs, if the output is equal to TRUE, the following step applies:

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb+ bvIntra[ xN ][ yN ][0]+nPbW-1 , yPb+ bvIntra[ xN ][ yN ][1]+nPbH-1 ), and the partition index partIdx as inputs, if the output is equal to TRUE, availableFlagNA is set equal to TRUE, and bvIntraNA is set equal to bvIntra[ xPb − 1 ][ yPb + nPbH − 1 ] bvIntra[ xN ][ yN ]

[Ed. (GJS): Check usage of TRUE vs. true (generally).]

[Ed. (GJS): Check "is set to" versus "is set equal to" (generall).]

The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the current luma coding block size nCbS, the luma prediction block location ( xPb, yPb ), the luma prediction block width nPbW, the luma prediction block height nPbH, the luma location ( xPb − nPbW − 1 , yPb − 1 ), and the partition index partIdx as inputs. If the output is equal to TRUE, and (yPb/CtbSizeY) is equal to ( ( yPb − 1 ) / CtbSizeY ), availableFlagB is set equal to TRUE, and bvIntraB is set equal to bvIntra[ xPb − nPbW − 1 ][ yPb − 1 ]