| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0088 |
| --- | --- |

| Title: | On WPP with palette mode and intra BC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document discusses the interaction between WPP and palette mode and Intra BC mode. (1) This document proposes to synchronize previous palette information similar as CABAC status when WPP is used together with palette mode. The experimental results show that when synchronizing the previous palette information with above CTU row, 0.5% and 0.7% bit saving is achieved for lossy AI text & graphics with motion (TGM) 1080p RGB and YUV, respectively. (2) This document proposes to add bitstream conformance to limit the pixels used for Intra BC prediction when Intra BC is used together with WPP.

# Introduction

Palette mode brings significant coding efficiency improvement for screen content sequences, and it has been adopted into screen content coding extension [2]. In the current design, the previous palette information is reset at the beginning of decoding a new CTU row when WPP is enabled. This document proposes to synchronize previous palette information with above CTU row in the similar way as CABAC synchronization when WPP is enabled.

In the current design, there is no constraint on the pixels used for Intra BC prediction when Intra BC is used together with WPP. This document also proposes to limit the pixels used for Intra BC prediction.

# Solution

## WPP with palette mode

This document proposes to synchronize previous palette information with above CTU row in the similar way as CABAC synchronization when WPP is enabled. In the proposed method, previous palette information is treated as part of the entropy status, which will be synchronized together with CABAC status. The detailed text modifications can be found in Section 6.

## WPP with Intra BC

When WPP is used, not all the top-right region are available for Intra BC prediction. As shown in Fig. 1, when decoding CTU 20, CTU 12, 03, and 04 may not be reconstructed yet, thus, cannot be used for prediction. This document proposes to add bitstream conformance to limit the pixels used for Intra BC prediction. The detailed text modification can be found in Section 6.

Fig. 1 WPP processing

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. WPP is enabled for both anchor and test. Intra BC search range is set to 4 CTUs in the test. It should be noted that there is a bug in SCM-2.0 when WPP is enabled, as reported in ticket #1317. A temporal fix is applied in the tests.

The summarized experimental results of lossy coding and lossless coding are shown in table 1 and 2, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

Table 2 Experimental results for lossless coding

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R0348, “Suggested combined software and text for run-based palette mode,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v2.

## WPP with palette mode

#### 7.4.9.6	Palette mode semantics

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag[ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The variable previousPaletteSize is derived as follows:

- If the current CU is the first CU with palette_mode_flag[ x0 ][ y0 ] equal to 1 in a slice or tile, the value of previousPaletteSize is set equal to 0.

- Otherwise if entropy_coding_sync_enabled_flag is equal to 1 and the current CU is the first CU with palette_mode_flag[ x0 ][ y0 ] equal to 1 in a CTB row, the value of previousPaletteSize is set equal to 0.

- Otherwise, the value of previousPaletteSize is set equal to the size of the palette of the previous CU for which the palette_mode_flag[ x0 ][ y0 ] equal to 1.

### 9.3.1	General

……

The storage process for context variables is applied as follows:

  - When ending the parsing of the coding tree unit syntax in subclause Error: Reference source not found, entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 1 or both CtbAddrInRs is greater than 1 and TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 2 ] ], the storage process for context variables and Rice parameter initialization states, previousPaletteSize and previousPaletteEntries as specified in subclause  is invoked with TableStateIdxWpp, TableMpsValWpp, and TableStatCoeffWpp, TablePaletteSizeWpp, TablePaletteEntriesWpp as outputs.

  - When ending the parsing of the general slice segment data syntax in subclause Error: Reference source not found, dependent_slice_segments_enabled_flag is equal to 1 and end_of_slice_segment_flag is equal to 1, the storage process for context variables and Rice parameter initialization states, previousPaletteSize and previousPaletteEntries as specified in subclause  is invoked with TableStateIdxDs, TableMpsValDs, and TableStatCoeffDs, TablePaletteSizeDs, TablePaletteEntriesDs as outputs.

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables and the initialized Rice parameter initialization states StatCoeff, previousPaletteSize and previousPaletteEntries.

The context variables of the arithmetic decoding engine are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the following applies:

  - The initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set to 0.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 0 or TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 1 ] ], the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Error: Reference source not found) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-1)

  - The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables and Rice parameter initialization states as specified in subclause  is invoked with TableStateIdxWpp, TableMpsValWpp, and TableStatCoeffWpp, TablePaletteSizeWpp, TablePaletteEntriesWpp as inputs.

  - Otherwise, the following applies:

  - The initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set to 0.

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables and Rice parameter initialization states as specified in subclause  is invoked with TableStateIdxDs, TableMpsValDs, and TableStatCoeffDs, TablePaletteSizeDs, TablePaletteEntriesDs as inputs.

  - Otherwise, the following applies:

  - The initialization process for context variables is invoked as specified in subclause Error: Reference source not found.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set to 0.

#### 9.3.2.3	Storage process for context variables and Rice parameter initialization states

Inputs to this process are:

–	The CABAC context variables indexed by ctxTable and ctxIdx.

–	The Rice parameter initialization states indexed by k.

–	The previous palette size information

–	The previous palette entries information indexed by cIdx and i.

Outputs of this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the initialization process of context variables and Rice parameter initialization states that are assigned to all syntax elements in subclauses Error: Reference source not found through Error: Reference source not found, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variables tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the initialization process of context variables and Rice parameter initialization states.

–	The variables tablePreviousPaletteSizeSync containing the values of the variables previousPaletteSize used in the initialization process of previous palette size.

–	The variables tablePreviousPaletteEntriesSync containing the values of the variables previousPaletteEntries used in the initialization process of previous palette entries.

For each context variable, the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMps.

For each Rice parameter initialization state k, each entry of the table tableStatCoeffSync is initialized to the corresponding value of StatCoeff[ k ].

For prevous palette size information, tablePreviousPaletteSizeSync is initialized to previousPaletteSize.

For each entry of the table tablePreviousPaletteEntriesSync is initialized to the corresponding value of previousPaletteEntries[cIdx][i]

#### 9.3.2.4	Synchronization process for context variables and Rice parameter initialization states

Inputs to this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the storage process of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found through Error: Reference source not found, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variable tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the storage process of context variables and Rice parameter initialization states.

–	tablePreviousPaletteSizeSync containing the values of the variables previousPaletteSize used in the storage process of previous palette size.

–	tablePreviousPaletteEntriesSync containing the values of the variables previousPaletteEntries used in the storage process of previous palette entries.

Outputs of this process are:

–	The initialized CABAC context variables indexed by ctxTable and ctxIdx.

–	The initialized Rice parameter initialization states StatCoeff indexed by k.

–	The initialized previous palette size previousPaletteSize.

–	The initialized previous palette entries previousPaletteEntries indexed by cIdx and i.

For each context variable, the corresponding context variables pStateIdx and valMps are initialized to the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync.

For each Rice parameter initialization state, each variable StatCoeff[ k ] is initialized to the corresponding entry of table tableStatCoeffSync.

For prevous palette size information, previousPaletteSize is initialized to tablePreviousPaletteSizeSync.

For each entry of previousPaletteEntries[cIdx][i] is initialized to the corresponding value of the table tablePreviousPaletteEntriesSync.

## WPP with Intra BC

### 8.4.4	Derivation process for block vector components in intra block copying prediction mode

……

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output is set equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output is set equal to TRUE.

–	One or both of the following conditions shall be true: [Ed. (GJS): Clarify that this is a bitstream constraint (if that is the correct interpretation).]

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw is less than or equal to 0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh is less than or equal to 0

–	When entropy_coding_sync_enabled_flag is equal to 1, it is a requirement of bitstream conformance that (xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1)/CtbSizeY – xCurr/CtbSizeY <= yCurr/CtbSizeY – (yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 )/CtbSizeY