| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0089 |
| --- | --- |

| Title: | Improvement for hash based inter search |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes non-normative improvement for hash based ME. With the proposed method, the encoding time is reduced by 7% for lossy RA coding and lossy LB coding, without significant impact on the coding efficiency.

# Introduction

This document proposes non-normative improvement for hash based ME. In the current design, homogeneous blocks are excluded from the hash table to avoid one hash value corresponding to too many identical blocks. However, excluding them from hash table also excludes such blocks from benefiting from hash based early termination algorithms. This document proposes to add selective homogeneous blocks into the hash table to further speedup the encoding process. Only about five lines of code need to be added.

# Solution

Selective homogeneous blocks are added into the hash table. The blocks aligned with CUs are always added into the hash table, even if they are homogeneous blocks.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results of lossy coding and lossless coding are shown in table 1 and 2, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding

Table 2 Experimental results for lossless coding

# References

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R0348, “Suggested combined software and text for run-based palette mode,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).