| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0094-v3 |
| --- | --- |

| Title: | QP offset for Adaptive Colour Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add an additional set of QP offset parameters that are applied for coding units that uses the Adaptive Colour Transform.

The contribution claims that additional flexibility and improved visual quality can be achieved through the introduction of three new QP offset syntax elements in the PPS and three new QP offset syntax elements in the slice header. The new syntax elements are proposed to be applied when Adaptive Colour Transform is used, i.e. when the cu_residual_act_flag is equal to 1. The contribution claims that it would be beneficial to be able to separately select which QP to use for Green, Blue, Red, Luma, Chroma Orange and Chroma Green depending on the content of the video and the desired compressed result.

The contribution further proposes to move the fixed QP offset that is applied to blocks that are color transformed (to compensate for that the transform is not normalized) so that it is included in the normal QP derivation. The contribution claims that this change ensures that the QP value stays in the allowed range.

# Introduction and problem description

HEVC version 1 includes the possibility to signal offsets to be used for the quantization parameter (QP) of each of the Chroma components relative to the QP used for Luma. This functionality is inherited to the HEVC Screen Content Coding Draft Text 1.

When the SCC draft is used with GBR data, the L sample array is used for holding Green data, the Cb sample array is used for holding Blue data and the Cr sample array is used for holding Red data. For example, in order to adjust the reconstructed quality of the Blue data, an encoder can signal values such that pps_cb_qp_offset + slice_cb_qp_offset is not equal to zero.

Adaptive Colour Transform (ACT) was adopted at the 18th JCT-VC meeting to HEVC Screen Content Coding Draft Text 1. ACT enables the use of an additional color transform that is applied on the residual sample arrays. For CUs to which ACT is applied, the resSamplesL sample array is used for holding Luma data, the resSamplesCb sample array is used for holding Chroma Green and resSamplesCr is used for holding Chroma Orange data.

However, for CUs to which ACT is applied, the QP is derived the same way as for CUs that do not use ACT, apart from a fixed offset that compensates for that the color transform is non-normalized. This means that when pps_cb_qp_offset and slice_cb_qp_offset are used to adjust the reconstructed quality of the Blue data, the same offset will also be applied to the Chroma Green data for CUs that uses ACT.

# Proposal

We are proposing to add three new syntax elements to PPS and three new syntax elements to the slice header and to let the QP derivation process depend on the value of cu_residual_act_flag.

The proposed new syntax elements in the PPS would be pps_transformed_y_qp_offset, pps_transformed_cb_qp_offset and pps_transformed_cr_qp_offset.

The proposed new syntax elements in the slice header would be slice_transformed_y_qp_offset, slice_transformed_cb_qp_offset and slice_transformed_cr_qp_offset.

It is proposed that these parameters are applied (instead of pps_cb_qp_offset, slice_cb_qp_offset, pps_cr_qp_offset and slice_cr_qp_offset) in the derivation process of the QP values to be used in CUs with cu_residual_act_flag equal to 1.

It is further proposed to move the addition of the compensation for the non-normalized colour transform into the QP derivation process instead of applying it afterwards. By making this move it is guaranteed that the QP value applied for a colour transformed block does not end up with an illegal value (i.e. below zero).

## Proposed changes to the SCC draft

[In an extension to the PPS:]

| pps_scc_extension( ) { | Descriptor |
| --- | --- |
| pps_transformed_y_qp_offset | se(v) |
| pps_transformed_cb_qp_offset | se(v) |
| pps_transformed_cr_qp_offset | se(v) |
| pps_slice_transformed_qp_offsets_present_flag | u(1) |
| } |  |

[In the slice segment header:]

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( pps_slice_transformed_qp_offsets_present_flag ) { |  |
| slice_transformed_y_qp_offset | se(v) |
| slice_transformed_cb_qp_offset | se(v) |
| slice_transformed_cr_qp_offset | se(v) |
| } |  |
| … |  |

[In the derivation process for quantization parameters (Section 8.6.1):]

When ChromaArrayType is not equal to 0, the following applies.

–	If cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 0, the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )

–	Otherwise (cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 1), the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_transformed_cb_qp_offset + slice_transformed_cb_qp_offset − 5 + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_transformed_cr_qp_offset + slice_transformed_cr_qp_offset − 3 + CuQpOffsetCr )

[In the scaling and transformation process (Section 8.6.2)]

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Clip3(0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag [ xTbY ][ yTbY ] ? pps_transformed_y_qp_offset + slice_transformed_y_qp_offset −5 : 0)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag [ xTbY ][ yTbY ] ? −-5 : 0)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr	 + (cu_residual_act_flag [ xTbY ][ yTbY ] ? −-3 : 0)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).