| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0095-v2 |
| --- | --- |

| Title: | HLS: Dependent RAP indication SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Jonatan Samuelsson Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 768428380 martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This is a follow-up contribution of JCTVC-R0059. The contribution proposes a new dependent RAP indication SEI message for the HEVC base specification and its extensions. The SEI message is used to indicate the presence of a dependent random access point (DRAP) picture in the bitstream. The following constraints are proposed when a DRAP picture is present:

- The DRAP picture may not include any picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr except its associated IRAP picture.

- The DRAP picture shall be a TRAIL_R picture with temporal id equal to 0 and layer id equal to 0

- Any picture that follow the DRAP picture in output order and decoding order shall not include, in its RPS, any picture that precedes the DRAP picture in output order or decoding order with the exception of the IRAP picture associated with the DRAP picture

When performing random access at a DRAP picture, the associated IRAP picture must first be decoded and the value of pic_output_flag should be inferred to be equal to 0 for all pictures that precede the DRAP picture in output order.

No parameters are signalled in the SEI message.

It is asserted that DRAP pictures improve the compression efficiency for random access coded video, especially for video services that often have very static content including screen sharing and surveillance video.

# Introduction

IRAP pictures can be used in HEVC to enable random access operations and to refresh the video in case of errors. The functionality of IRAP pictures comes at a cost as intra pictures are typically significantly more expensive to encode in terms of bits compared to P- or B-pictures. For video with stationary content the relative cost of coding IRAP pictures is very expensive.

In broadcasting services, IRAP pictures are typically sent at frequent intervals (e.g. once per second) to enable reasonable channel switching times. Having even more frequent IRAP pictures would be desirable for decreasing the channel switching time even further, but that is not feasible since the IRAP pictures then would consume a large portion of the available bitrate and reduce the overall quality.

# Proposal

In this proposal the problems described in the previous section are addressed by introducing a dependent random access point picture referred to as a DRAP picture. The presence and properties of a DRAP picture is indicated by a new dependent RAP indication SEI message.

A DRAP picture is a TRAIL_R picture with temporal id 0 and layer id 0 with the restriction that it may only reference the associated IRAP picture. In order to decode a DRAP picture in a random access operation its associated IRAP picture must first be decoded. A random access configuration with IRAP pictures and a random access configuration with both IRAP and DRAP pictures are illustrated in figure 1 below.

Figure 1		Top: random access configuration with IRAP pictures. Bottom: random access configuration with both IRAP and DRAP pictures. The dark gray images are IRAP pictures, the medium gray pictures are DRAP pictures and the white pictures are either P- or B-pictures.

The HEVC specification already contains the related recovery point SEI message that may be used for indicating the possibility for random access. However, recovery point SEI messages cannot be used to realize the functionality of DRAP pictures. If a recovery point SEI message is sent together with the associated IRAP picture then all pictures that follow it in decoding order must be decoded up until the DRAP picture, which is not desirable. And the recovery point SEI message cannot be sent together with the DRAP picture since it is not possible to indicate dependency on anything that precedes the recovery point SEI message in decoding order.

The dependent RAP indication SEI message ensures that the bitstream complies with the DRAP functionality of the associated DRAP picture. It could be argued that it is possible to parse the RPS to get the functionality of DRAP pictures. However, as depicted in figure 2, if DRAP pictures are not explicitly indicated it would not be possible to know in advance whether a picture following a DRAP picture in decoding order is referencing a non-IRAP picture preceding the DRAP picture in decoding order or not. This could result in erroneous pictures when performing a random access operation at a picture thought of being a DRAP picture. Moreover, parsing the RPS is a complex task and it would be desirable at a system layer to get the DRAP indication directly.

Figure 2	 It is not possible to derive the DRAP functionality by only parsing the RPS. A picture following the DRAP picture in decoding order may still reference a non-IRAP picture preceding the DRAP picture.

Video services that often have very static content include screen sharing and surveillance video. Screen sharing could for instance be used as a live communication tool between individuals or could be set up to monitor other computers such as servers. For these services it is often of interest to store the video material. The stored video material should then preferably be easy to search using random access operations. At the same time it is of interest to keep the video bitrate at a minimum, both to limit the bandwidth usage and to save storage space. By using DRAP pictures with a shorter periodicity and IRAP pictures with a longer periodicity (for instance DRAP pictures every second and IRAP pictures every 60 seconds) the bitrate can be reduced at the same time as the stored video can be easily searched.

Another use case for DRAP pictures is fast channel switching in broadcast services. Assume that an end user is watching a first channel. Then by having multiple tuners in the receiver that buffers IRAP pictures from other channels, a channel switch can be performed at the first DRAP picture that occurs after the user has selected to switch channel. It is asserted that DRAP pictures can be sent much more frequently than IRAP pictures without affecting the overall quality as much.

An example of channel switching using DRAP pictures is shown in figure 3. A user is watching channel A. Additional tuners receive and buffer the latest IRAP picture for channel B, C and D, e.g. the channels directly before and after channel A. At picture position 45 the user is switching to channel B. The channel B tuner waits 4 pictures for the next DRAP picture before the decoder can start to decode channel B with help from the buffered IRAP picture. At picture position 67 the user is switching to channel C. The decoder waits 6 pictures for the next DRAP picture before it can start to decode channel C. The latest IRAP picture is used to decode the DRAP picture. If the streams would not contain any DRAP pictures, the decoder would need to wait for the next IRAP picture before starting decoding after a channel switch. In the example below, the decoder would need to wait 20 pictures when switching between channels A and B and 30 pictures when switching between channels B and C. Also, the IRAP picture is larger than the DRAP picture, which also may contribute to longer delay.

Figure 3		Example of channel switching using DRAP pictures. Dark gray pictures are IRAP pictures, medium gray pictures are DRAP pictures and white pictures are other temporal predictive pictures.

## Proposed changes to HEVC specification

It is proposed to insert the following text marked in yellow into the HEVC specification [1] without any restrictions on which profiles and/or extensions it applies to.

D.2.30	Dependent RAP indication SEI message syntax

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

D.3.30	Dependent RAP indication SEI message semantics

The dependent RAP indication SEI message assists a decoder in determining what parts of a bitstream need to be decoded in order to achieve correct decoding of the picture associated with the dependent RAP indication SEI message and the following pictures.

The picture associated with the dependent RAP indication SEI message is referred to as a DRAP picture. The DRAP picture shall be a TRAIL_R picture with TemporalId equal to 0 and nuh_layer_id equal to 0. The DRAP picture may not include any picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr except its associated IRAP picture.

When performing random access at the DRAP picture the value of pic_output_flag should be inferred to be equal to 0 for all pictures that precede the DRAP picture in output order. Decoded pictures preceding the DRAP picture in output order may contain references to pictures that are unavailable in the decoded picture buffer.

Any picture that follow the DRAP picture in output order and decoding order shall not include, in its RPS, any picture that precedes the DRAP picture in output order or decoding order with the exception of the IRAP picture associated with the DRAP picture.

# Results

In the previous contribution, JCTVC-R0059, bitrate saving results by using DRAP pictures with infrequent IRAP pictures were reported. For those simulations only the first picture of each sequence in the test sets was encoded as an IRAP picture, while the other random access point pictures were encoded as DRAP pictures. Table 1 shows a summary of the previously reported results.

| Test set | Anchor | Test | Avg BD Y | Seq min BD Y | Seq max BD Y |
| --- | --- | --- | --- | --- | --- |
| HEVCv1 RA | IRAP period 32 | DRAP period 32 | -9.5% | -55.9% | 1.8% |
| SCC YUV444 RA | IRAP period 32 | DRAP period 32 | -17.2% | -57.7% | 1.6% |

Table 1	Results comparing DRAP period 32 to IRAP period 32 reported in JCTVC-R0059.

For this follow-up contribution new simulations have been carried out to get an indication of how much can be gained by using DRAP pictures as a complement to IRAP pictures in a fast channel switching scenario. To start with, simulations were carried out to see how much higher bitrate is needed when having IRAP pictures every 8th picture compared to IRAP pictures every 64th picture. Then simulations were carried out with IRAP pictures every 64th picture and DRAP pictures every 8th picture (except at the positions of the IRAP pictures). Table 2 shows the BD Y results for these simulations compared to IRAP pictures every 64th picture. The simulations have been carried out using SCM2.0 [2] with random access configurations over the HEVCv1 and SCC YUV444 test sets.

| Anchor | Test | Avg BD Y HEVCv1 test set | Avg BD Y SCC test set |
| --- | --- | --- | --- |
| IRAP period 64 | IRAP period 8 | 67.7% | 131.3% |
| IRAP period 64 | IRAP period 64, DRAP period 8 | 17.1% | 28.1% |

Table 2	BD Y results for IRAP period 8 vs IRAP period 64 with DRAP period 8 for the HEVCv1 RA and SCC YUV444 RA test sets. Anchor is IRAP period 64 without DRAP.

# References

[1] Draft high efficiency video coding (HEVC) version 2, combined format range extensions (RExt), scalability (SHVC), and multi-view (MV-HEVC) extensions, JCTVC-R1013_v6

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0rc1/

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).