| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0096r1 |
| --- | --- |

| Title: | AhG13: Palette and deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Wei Pu, Chao Pang, Rajan Joshi, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-2429 joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The deblocking filter is applied to palette coded blocks. However, generally, filtering and smoothing processes are avoided for screen content coding. It is proposed (as in JCTVC-R0213) to not deblock CUs coded with palette in the same way that PCM and lossless CUs are not deblocked. Allegedly, this reduces complexity, avoids unwanted filtering and achieves BD-rate reductions (up to 0.2%). Additionally, a flag at SPS level could be included to enable/disable deblocking of palette coded CUs.

# Introduction

During the HEVC, HEVC RExt, and HEVC SCC development, filtering and smoothing have been generally avoided for the coding of screen content, as in the following examples:

- Transform skip avoids the DCT averaging and shows very good gains in SCC

- Flag intra_smoothing_disabled_flag was introduced in HEVC RExt to disable the filtering of intra prediction samples

- Interpolation filters are not used in intra block copying

- Several proposals in HEVC RExt and SCC showed performance gains for SCC when filtering processes are disabled

Another source of smoothing is the deblocking filter, which is still applied in combination with tools that are strictly targeting the improvement of screen content coding.

# Palette and deblocking

Palette is suitable to code regions with small number of colors: flat areas, edges and simple textures. This type of regions should not generally be filtered. However, the default behaviour of palette is to treat palette coded areas the same as any other intra mode, thus the strong deblocking filter is applied. The undesired consequence is that edges are smoothed out.

As shown in JCTVC-R0213, the top part of Figure 1 is the first frame of sc_console coded at QP 22. The CUs using palette are highlighted. The bottom part of the Figure 1 highlights the palette pixels affected by the deblocking filter (in white). In the enlarged fragments shown in Figure 2, it is clear that deblocking filter fails to detect real edges, which are being filtered. For example, the horizontal grid lines and part of the text are being deblocked.  Similar examples are provided in figures 3 to 6 for the sc_desktop_rgb and sc_flyingGraphics_rgb the sequences.

Figure 1. Top) First frame of sc_console at QP 22 with the palette coded areas highlighted in green/yellow. Bottom) Same frame showing pixels modified by the deblocking filter in palette.

Figure 2. Enlarged fragments showing the real edges (in red circles) distorted by the deblocking filter.

Figure 3. Top) First frame of sc_desktop_rgb at QP 22 with the palette coded areas highlighted in green/yellow boxes.

Bottom) Same frame showing pixels modified by the deblocking filter in palette.

Figure 4. Enlarged fragments showing the real edges (in red circles) distorted by the deblocking filter. Top) Reconstructed. Bottom) Parts modified by deblocking.

Figure 5. Top) First frame of sc_flyingGraphics_rgb at QP 22 with the palette coded areas highlighted in green/yellow boxes. Bottom) Same frame showing pixels modified by the deblocking filter in palette.

Figure 6. Enlarged fragments showing the real edges (in red circles) distorted by the deblocking filter.

# Proposal

HEVC disables the deblocking filter for PCM CUs and CUs with cu_transquant_bypass_flag equal 1. It is proposed to use this mechanism in order to also disable the deblocking filter for palette coded CUs.

Additionally, a flag could be included at the SPS to enable/disable the deblocking filter for palette, as done for PCM, so that encoders have the flexibility to use deblocking filter for palette.

# Performance

The simulation results for the proposed method are provided on the top of SCM2.0 using the SCC common test conditions.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080 | 0.0% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080 | -0.1% | -0.3% | -0.2% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.6% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080 | -0.1% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.8% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# WD Text

## Disabling deblocking for palette mode

##### 8.7.2.5.7 Filtering process for a luma sample

…

When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 ][ yP0 ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 ][ yQ0 ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

–	palette_mode_flag  of the coding unit that includes the coding block containing the sample q0 is equal to 1.

##### 8.7.2.5.8 Filtering process for a chroma sample

…

When one or more of the following conditions are true, the filtered sample value, p0′ is substituted by the corresponding input sample value p0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 * SubWidthC ][ yP0 * SubHeightC ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When one or more of the following conditions are true, the filtered sample value, q0′ is substituted by the corresponding input sample value q0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 * SubWidthC ][ yQ0 * SubHeightC ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

## SPS flag

palette_loop_filter_disabled_flag specifies whether the deblocking filter process is disabled on reconstructed samples in a coding unit with palette_mode_flag equal to 1 as follows:

–	If palette_loop_filter_disabled_flag is equal to 1, the deblocking filter process on the reconstructed samples in a coding unit with palette_mode_flag equal to 1 is disabled.

–	Otherwise (palette_loop_filter_disabled_flag value is equal to 0), the deblocking filter process on the reconstructed samples in a coding unit with palette_mode_flag equal to 1 is not disabled.

When palette_loop_filter_disabled_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).